- (normative)
Bioburden control formal system description - DRD

  - DRD identification

    - Requirement identification and source document

This DRD is called from ECSS-Q-ST-70-58, requirement 5.1.1.1a and 5.2.5.1a.

    - Purpose and objective

The bioburden control formal system is established to assess and control factors that can affect the microbiological quality of the process.

  - Expected response

    - Scope and content

          - The bioburden control formal system description shall include:

            - Limits established to ensure control, including alert and action level as established in conformance with requirement 5.1.2.1a.

            - Description of the monitoring scheme and schedule established.

            - Description of the validation procedures established in conformance with clause 5.1.1.2 to verify that the formal system is working properly.

            - Corrective actions established in conformance with requirement 5.1.2.2b.2 to be taken when a monitoring result indicates that a particular factor of the process is not under control.

            - Risk assessment for bioburden control.

- For example: Process FMECA.

            - Description of the training procedures and access constraints established in conformance with clause 5.2.6.

            - Documentation to be established and maintained.

            - Process implementation documentation.

- For example: Logbooks.

            - Periodic reviews/audits to be performed.

            - Maintenance status of  equipment.

            - Cleanroom cleaning programme.

            - A training programme.

            - Description of the sampling plan, including at least:

              - The cleanliness level of the cleanroom to be applied and the appropriate degree of bioburden control for the activity being conducted.

              - The sampling location(s) identified in conformance with clause 5.2.5.3, including the location of the primary “risk zone”; i.e.; the work area immediately adjacent to any contamination-critical hardware and the work activities/personnel associated with this location.

              - The minimum number of samples to be taken in order to ensure representative results.

- This number is dependant on the sample size/ volume. For small sample sizes, representativeness of the results cannot be guaranteed if this number is not significantly increased.

              - Frequency of initial monitoring sampling, established in conformance with requirement 5.2.5.2b.1.

              - Frequency of normal routine sampling, established in conformance with requirement 5.2.5.2b.2.

              - Frequency of sampling for specific cases, established in conformance with requirement 5.2.5.2b.3, including at least the following cases:

- Suspected contamination events.

- Action levels are exceeded.

- Alert levels are exceeded (consecutively).

- A change in the biodiversity.

- After prolonged shut-down of activities.

- After any significant maintenance work has been undertaken on the cleanroom.

- 1	For example: Maintenance on ventilation system, changes to furnishings or fittings, introduction of significant quantities of new hardware or equipment.

- After changes to the process that affect the cleanroom environment.

- After recording of unusual results.

- After non-routine changes to the cleaning or disinfection procedures.

- After unplanned incidents that could contribute to contamination.

- After any incidence of non-conformance to cleanroom operational specifications.

- 2	For example: Breaches of personnel dress/conduct code; out of specification airborne particulate levels, out of specification temperature or humidity levels, exceeding maximum no. of personnel, or introduction of non-sterile items or hardware.

          - Sampling method(s), established in conformance with clause 5.2.5.5.

          - Analysis method(s), established in conformance with clause 5.2.5.6.

          - Factors pertinent to a particular situation that can affect results.

- Ambient temperature, lead-time to processing of samples.

          - The impact of operations, personnel and equipment, which contribute to contamination.

    - Special remarks

None.