- (normative)
Test report (TRPT) - DRD

  - DRD identification

    - Requirement identification and source document

This DRD is called up from ECSS-E-ST-10-02, requirement 5.3.2.1b.

    - Purpose and objective

The test report describes test execution, test and engineering assessment of results and conclusions in the light of the test requirements (including pass-fail criteria).

The test report contains the scope of the test, the test description, the test article and sep-up configuration, and the test results including the asrun test procedures, the considerations and conclusions with particular emphasis on the closeout of the relevant verification requirements including deviations.

  - Expected response

    - Scope and content

          - Introduction

          - The TRPT shall contain a description of the purpose, objective, content and the reason prompting its preparation.

          - Open issues, assumptions and constraints relevant to this document shall be stated and described.

          - Applicable and reference documents

          - The TRPT shall list the applicable and reference documents in support to the generation of the document.

          - Definitions and abbreviations

          - The TRPT shall list the applicable dictionary or glossary and the meaning of specific terms or abbreviations utilized in the document

          - Test results

          - The TRPT shall contain the test results with supporting data (including the test execution dates, the as run procedure, and the test facility results).

          - The TRPT shall contain the analysis of test data and the relevant assessment.

          - The TRPT shall provide a synthesis of the test results.

          - Anomalies

          - The TRPT shall include the list of deviations to the test procedure, the nonconformance including failures and the problems.

          - Conclusions

          - The TRPT shall summarize:

            - the test results, including:

              - the list of the requirements to be verified (in correlation with the VCD),

              - traceability to used documentation,

              - conformance or deviation including references and signature and date),

            - the comparison with the requirements and

            - the verification close-out judgment.

          - Open issues shall be clearly stated and described.

          - Separate test analyses shall be cross-referenced.

    - Special remarks

None.