- (normative)
Hydrogen peroxide bioburden reduction proposal - DRD

  - DRD identification

    - Requirement identification and source document

This DRD is called from ECSS-Q-ST-70-56, requirement 5.1c.

    - Purpose and objective

The purpose of the proposal is to describe a bioburden reduction process for a product. It is written by a supplier of a bioburden reduction service and is the response to a bioburden reduction specification.

  - Expected response

    - Scope and content

          - The proposal for hydrogen peroxide bioburden reduction shall include:

            - Bioburden reduction procedure planned to be used.

            - List and description of equipment planned to be used for applying and controlling the bioburden reduction process.

            - Product specific cycle development.

            - Pre-conditioning for the product.

            - Particular and molecular contamination control before, during and after the bioburden reduction process is applied.

            - Bioburden recontamination control for the product before, during and after the bioburden reduction process is applied.

            - The hydrogen peroxide bioburden reduction cycle, including conditioning, bioburden reduction Ct-value as specified by the procedure used in conformance with requirements from the clause 5.3.1, venting, and set points for the control of the cycle.

- To fulfil this requirement it can be necessary to perform some tests with a geometrically and material representative model, equipped with hydrogen peroxide sensors and BIs, under the same conditions as planned for the flight hardware bioburden reduction.

            - Locations on the product for which it is most difficult to achieve the conditions specified in the procedures in conformance with requirements from the clause 5.3.2.

            - The loading pattern of the equipment.

            - Loading pattern specific locations to measure process parameters.

- 1	Typical process parameters to control the cycle are hydrogen peroxide vapour concentration, temperature, humidity, and pressure.

- 2	Any copper or cellulose in the load can cause difficulties in maintaining the concentration.

            - Values for process parameters and their tolerances to control the bioburden reduction cycle.

- Proper monitoring and documentation is necessary to allow release of the product, see clause 5.2.4.

            - The equipment used for bioburden reduction.

            - The purity of the air or other gases used in the process.

            - Environmental conditions and control of the equipment.

- This includes any level of particulate or molecular contamination control, filtrations systems, use of forced air flow with direction and velocity, use of pumps, pressure level.

            - Deliverables.

            - Quality standards.

    - Special remarks

None.