- (normative)
Process identification document (PID) - DRD

  - DRD identification

    - Requirement identification and source document

This DRD is called from ECSS-E-ST-20-08, requirements 6.2a, 7.2a, 8.4a and 9.3.2a.

    - Purpose and objective

The purpose of the PID is to have a complete set of documentation defining the traceability of materials, processes and test results of the relevant components or sub-assemblies.

  - Expected response

    - Scope and content

          - The PID shall comprise copies of the definition documents of the SCA, and the manufacturing documents and testing procedures, and include the following:

            - in the case of SCAs, a parts list;

            - a materials list;

            - a list of all manufacturing drawings;

            - the production flow chart;

            - the specifications for the process used;

            - procedures for the inspection performed;

            - the overall test programme (including in-process tests and acceptance tests);

            - a table of contents with reference number and issue;

            - the test matrix for acceptance tests, including requirements and failure criteria;

            - the traceability details of the component.

- The traceability details are as agreed between customer and supplier.

    - Special remarks

          - When a document is company confidential or contains proprietary information, the complete document need not be included.

          - In the case specified in requirement 11.2.2.4a, a reference to the document shall be included.