- (normative)
Coordinate Systems Document (CSD) - DRD

  - DRD identification

    - Requirement identification and source document

This DRD is called from ECSS-E-ST-10-09, requirement 5.2.2a.

    - Purpose and objective

Proper documentation and maintenance of coordinate systems and their inter-relationships, throughout the product tree and the project/mission life, shall be initiated with sufficient detail. Standard requirements for the document and related database would help to regularise conventions even earlier.

  - Expected response

    - Scope and content

          - Introduction

          - The CSD shall describe the purpose, objective and the reason prompting its preparation (e.g. programme, project and phase).

          - Applicable and reference documents

          - The CSD shall list the applicable and reference documents supporting the generation of the document.

          - Convention and Notation

          - The CSD shall specify the international conventions used within the project for coordinate systems and transformations.

          - The CSD shall define the naming, notation and acronym rules for coordinate systems, as well as for transformations.

          - The CSD shall define notation conventions for the transfer of coordinates and transformation data.

          - Units

          - The CSD shall specify the units pertaining to the coordinate systems and parameterisations used within the project.

          - Time standards

          - The CSD shall specify the time standards used within the project and shall specify the relationship between them.

          - Coordinate system, overview

          - The CSD shall present the overview of the various coordinate systems.

          - The overview shall contain a brief description of at least the following coordinate systems, if applicable:

            - Inertial Coordinate Systems

            - Orbital Coordinate Systems

            - Launcher Coordinate Systems

            - Satellite-fixed Coordinate System (generic platform and payload)

            - Body-fixed Rotation (planet) Coordinate Systems

            - Topocentric Coordinate Systems

            - Test facility Coordinate Systems

            - Simulator Coordinate Systems

            - Processing / Product Coordinate Systems

          - The description shall refer to the theory and conventions applied in between the various coordinate systems (e.g. precession, nutation, polar motion).

          - Parameterisations

          - The CSD shall describe all parameterisations used within a coordinate system (e.g. azimuth, elevation), including the applicable type of coordinate systems where this parameterisation is allowed.

          - The CSD shall describe all parameterisations used within a transformation (e.g. quaternions, Euler angles), including the applicable type of coordinate systems where this parameterisation is allowed.

          - Diagrams

          - The CSD shall describe in a graphical representation the top-level chain of transformations for the project.

          - The diagram of <8a> shall include any additional constraints between coordinate systems, as well as measurements.

          - The CSD shall describe in a graphical representation the lower level chains of transformations.

          - Each individual coordinate system, identified in <9> below, shall be  graphically represented.

          - These diagrams shall contain any additional constraints between coordinate systems, as well as measurements.

          - Coordinate systems, details

          - The CSD shall give the detailed description of all Coordinate Systems used in the project.

          - The transformation between coordinate systems shall be defined in the one that is the parent coordinate system.

          - Parameterisations within a coordinate system shall be identified and specified.

- E.g. right ascension and declination.

          - The parameterisation of a transformation shall be identified and specified.

- E.g. quaternions.

          - Any source document for a coordinate system shall be identified in the CSD.

    - Special remarks

The CSD can be part of the SEP (as defined in ECSS-E-ST-10).