ECSS Secretariat

ESA-ESTEC

Requirements & Standards Section

Noordwijk, The Netherlands

Space product assurance

Manufacturing and control of electrical harness

Foreword

ECSS is a cooperative effort of the European Space Agency, national space agencies and European industry associations for the purpose of developing and maintaining common standards. Requirements in this Standard are defined in terms of what shall be accomplished, rather than in terms of how to organize and perform the necessary work. This allows existing organizational structures and methods to be applied where they are effective, and for the structures and methods to evolve as necessary without rewriting the standards.

This Standard has been prepared by the ECSS-Q-ST-20-30C Working Group, reviewed by the ECSS Executive Secretariat and approved by the ECSS Technical Authority.

Disclaimer

ECSS does not provide any warranty whatsoever, whether expressed, implied, or statutory, including, but not limited to, any warranty of merchantability or fitness for a particular purpose or any warranty that the contents of the item are error-free. In no respect shall ECSS incur any liability for any damages, including, but not limited to, direct, indirect, special, or consequential damages arising out of, resulting from, or in any way connected to the use of this Standard, whether or not based upon warranty, business agreement, tort, or otherwise; whether or not injury was sustained by persons or property or otherwise; and whether or not loss was sustained from, or arose out of, the results of, the item, or any services that may be provided by ECSS.

Published by: 	ESA Requirements and Standards Section

ESTEC, P.O. Box 299,

2200 AG Noordwijk

The Netherlands

Copyright: 	2025© by the European Space Agency for the members of ECSS

Change log

| ECSS-Q-ST-20-30C 20 March 2025 | First issue |
| --- | --- |

Table of contents

Change log	3

Introduction	9

1 Scope	10

2 Normative references	11

3 Terms, definitions and abbreviated terms	12

3.1	Terms from other standards	12

3.2	Terms specific to the present standard	12

3.3	Abbreviated terms	12

3.4	Conventions	13

3.5	Nomenclature	13

4 Principles	14

4.1	General	14

4.2	Use of this standard	14

4.3	Order of precedence for requirements	14

5 General requirements	16

6 Applicability of IPC/WHMA-A-620E(-S)	17

6.0	IPC/WHMA-A-620E-S, § 0: Addendum for Space and Military Applications Requirements	17

6.0.1	IPC/WHMA-A-620E-S, § 0.1: Scope	17

6.1	IPC/WHMA-A-620E, § 1.0: General	18

6.1.1	IPC/WHMA-A-620E-S, § 1.1: Scope	18

6.1.2	IPC/WHMA-A-620E-S, § 1.2: Purpose	18

6.1.3	IPC/WHMA-A-620E-S, § 1.3: Classification	18

6.1.4	IPC/WHMA-A-620E, § 1.4: Measurement Units and Applications	18

6.1.5	IPC/WHMA-A-620E, § 1.5: Definition of requirements	18

6.1.6	IPC/WHMA-A-620E-S, § 1.6: Process Control	19

6.1.7	IPC/WHMA-A-620E, § 1.7: Order of Precedence	19

6.1.8	IPC/WHMA-A-620E, § 1.8: Terms and Definitions	19

6.1.9	IPC/WHMA-A-620E, § 1.9: Requirements Flowdown	19

6.1.10	IPC/WHMA-A-620E-S, § 1.10: Personnel Proficiency	20

6.1.11	IPC/WHMA-A-620E, § 1.11: Acceptance Requirements	20

6.1.12	IPC/WHMA-A-620E, § 1.12: Inspection Methodology	20

6.1.13	IPC/WHMA-A-620E, § 1.13: Facilities	20

6.1.14	IPC/WHMA-A-620E, § 1.14: Electrostatic Discharge (ESD) Protection	20

6.1.15	IPC/WHMA-A-620E, § 1.15: Tools and Equipment	21

6.1.16	IPC/WHMA-A-620E(-S), § 1.16: Materials and Processes	21

6.1.17	IPC/WHMA-A-620E, § 1.17: Electrical Clearance	21

6.1.18	IPC/WHMA-A-620E, § 1.18: Contamination	21

6.1.19	IPC/WHMA-A-620E, § 1.19: Rework/Repair	21

6.2	IPC/WHMA-A-620E, § 2.0: Applicable documents	22

6.3	IPC/WHMA-A-620E, § 3.0: Wires	22

6.3.1	IPC/WHMA-A-620E(-S), § 3.1: Stripping	22

6.3.2	IPC/WHMA-A-620E(-S), § 3.2: Strand Damage and End Cuts	22

6.3.3	IPC/WHMA-A-620E, § 3.3: Conductor Deformation/Birdcaging	22

6.3.4	IPC/WHMA-A-620E, § 3.4: Twisting of Wires	22

6.3.5	IPC/WHMA-A-620E, § 3.5: Insulation Damage - Stripping	22

6.4	IPC/WHMA-A-620E(-S), § 4.0: Soldered Terminations	22

6.5	IPC/WHMA-A-620E(-S), § 5.0: Crimp Terminations (Contacts and Lugs)	23

6.6	IPC/WHMA-A-620E(-S), § 6.0: Insulation displacement connection (IDC)	23

6.7	IPC/WHMA-A-620E(-S), § 7.0: Ultrasonic Welding	23

6.8	IPC/WHMA-A-620E, § 8.0: Splices	23

6.8.1	IPC/WHMA-A-620E(-S), § 8.1: Soldered Splices	23

6.8.2	IPC/WHMA-A-620E(-S), § 8.2: Crimped Splices	23

6.8.3	IPC/WHMA-A-620E(-S), § 8.3: Ultrasonic Weld Splices	23

6.8.4	IPC/WHMA-A-620E-S, § 8.4: Sleeving over Splices	23

6.9	IPC/WHMA-A-620E, § 9.0: Connectorization	24

6.9.1	IPC/WHMA-A-620E, § 9.1: Hardware Mounting	24

6.9.2	IPC/WHMA-A-620E, § 9.2: Strain relief	24

6.9.3	IPC/WHMA-A-620E, § 9.3: Sleeving and Boots	25

6.9.4	IPC/WHMA-A-620E, § 9.4: Connector damage	25

6.9.5	IPC/WHMA-A-620E-S, § 9.5: Installation of Contacts and Sealing Plugs into Connectors	26

6.10	IPC/WHMA-A-620E-S, § 10: Over-Molding/Potting	26

6.10.1	IPC/WHMA-A-620E, § 10.1: Over-Molding	26

6.10.2	IPC/WHMA-A-620E, § 10.2: Potting (Thermoset Molding)	26

6.10.3	IPC/WHMA-A-620E, § 10.3: Over-Molding of Flexible Flat Ribbon	26

6.11	IPC/WHMA-A-620E, § 11.0: Measuring Cable Assemblies and Wires	27

6.12	IPC/WHMA-A-620E-S, § 12.0: Marking/Labelling	27

6.12.1	IPC/WHMA-A-620E, § 12.1: Content	27

6.12.2	IPC/WHMA-A-620E, § 12.2: Legibility	27

6.12.3	IPC/WHMA-A-620E, § 12.3: Permanency	27

6.12.4	IPC/WHMA-A-620E-S, § 12.4: Location and Orientation	27

6.12.5	IPC/WHMA-A-620E, § 12.5: Functionality	27

6.12.6	IPC/WHMA-A-620E, § 12.6: Marker Sleeve	27

6.12.7	IPC/WHMA-A-620E, § 12.7: Flag Markers	28

6.12.8	IPC/WHMA-A-620E, § 12.8: Tie Wrap Markers	28

6.13	IPC/WHMA-A-620E(-S), § 13.0: Coaxial and Biaxial Cable Assemblies	28

6.14	IPC/WHMA-A-620E, § 14.0: Securing	29

6.14.1	IPC/WHMA-A-620E-S, § 14.1: Tie Wrap/ Lacing application	29

6.14.2	IPC/WHMA-A-620E, § 14.2: Breakouts	30

6.14.3	IPC/WHMA-A-620E, § 14.3: Routing	30

6.14.4	IPC/WHMA-A-620E, § 14.2: Broom Stitching	31

6.15	IPC/WHMA-A-620E, § 15.0: Harness/Cable Electrical Shielding	31

6.15.1	IPC/WHMA-A-620E-S, § 15.1: Braided	31

6.15.2	IPC/WHMA-A-620E-S, § 15.2: Shield Termination	31

6.15.3	IPC/WHMA-A-620E, § 15.3: Shield Termination - Connector	33

6.15.4	IPC/WHMA-A-620E, § 15.4.1: Shield Termination - Splicing Prewoven	33

6.15.5	IPC/WHMA-A-620E, § 15.5: Tapes - Barrier and Conductive, Adhesive or Non-Adhesive	33

6.15.6	IPC/WHMA-A-620E, § 15.6: Conduit (Shielding)	34

6.15.7	IPC/WHMA-A-620E(-S), § 15.6: Shrink Tubing – Conductive Lined	34

6.16	IPC/WHMA-A-620E, § 16.0: Cable/Wire Harness Protective Coverings	34

6.16.1	IPC/WHMA-A-620E, § 16.1: Braid	34

6.16.2	IPC/WHMA-A-620E, § 16.2: Sleeving/Shrink Tubing	34

6.16.3	IPC/WHMA-A-620E(-S), § 16.3: Spiral Plastic Wrap (Spiral Wrap Sleeving)	34

6.16.4	IPC/WHMA-A-620E, § 16.4: Wire Loom Tubing – Split and Unsplit	34

6.16.5	IPC/WHMA-A-620E, § 16.5: Tapes, Adhesive and Non-Adhesive	34

6.17	IPC/WHMA-A-620E-S, § 17.0: Finished assembly Installation	35

6.17.1	IPC/WHMA-A-620E-S, § 17.1: General	35

6.17.2	IPC/WHMA-A-620E-S, § 17.2: Hardware installation	35

6.17.3	IPC/WHMA-A-620E, § 17.3: Wire/Harness installation	35

6.18	IPC/WHMA-A-620E(-S), § 18.0: Solderless Wrap	35

6.19	IPC/WHMA-A-620E, § 19.0: Testing	35

6.19.1	IPC/WHMA-A-620E, § 19.1: Nondestructive Tests	35

6.19.2	IPC/WHMA-A-620E, § 19.2: Testing After Rework or Repair	36

6.19.3	IPC/WHMA-A-620E, § 19.3: Intended Table Usage	36

6.19.4	IPC/WHMA-A-620E-S, § 19.4: Electrical Test	36

6.19.5	IPC/WHMA-A-620E, § 19.5: Electrical Test Methods	37

6.19.6	IPC/WHMA-A-620E, § 19.6: Mechanical Test	38

6.19.7	IPC/WHMA-A-620E, § 19.7: Mechanical Test Methods	38

6.20	IPC/WHMA-A-620E, § 20.0: High Voltage Applications	39

7 Complementary requirements	40

7.1	Information	40

7.2	General Requirements	40

7.3	Soldering	40

7.4	Crimping	40

7.5	Insulation sleeving on contact termination	40

7.6	Labelling and marking:	43

7.7	Branching of metallic braid-shielded bundles:	44

7.8	EMC shielding with conductive metal foil	45

Annex A (informative) Applicability tables	47

A.1	IPC/WHMA-A-620E(-S) applicability table	47

Annex B (informative) Example of a “(De-)mating Logbook”	53

Bibliography	54

Figures

Figure 6-1: Sharp angle definition	25

Figure 6-2: Excessive (defect) overheating of marking sleeves	28

Figure 6-3: Tie-wrap – acceptable cut end protrusion	29

Figure 6-4: Tie-wrap – receded tail	29

Figure 7-1: Double sleeving on D-Sub contacts	41

Figure 7-2: Insulation sleeving over pin contact	41

Figure 7-3: Inspection of exposed conductor into connector insulator	42

Figure 7-4: Gap between sleeve and insulating insert	42

Figure 7-5: Example of foreign particles entrapped under shrinking sleeve	43

Figure 7-6: Example of sleeve showing signs of excessive deformation	43

Figure 7-7: Connector label applied onto harness bundle	43

Figure 7-8: Marking on connector backshell or with a flag	44

Figure 7-9: Insulation protection under the shield at the breakout	44

Figure 7-10: Secured braid along the breakout	45

Figure 7-11: Example of defective braided shield	45

Figure 7-12: Helicoidal wrapping of conductive shield	45

Figure 7-13: Example of acceptable foil shielding	46

Figure 7-14: Example of not acceptable, loose foil shielding	46

Figure 7-15: Example of not acceptable foil shielding	46

Tables

Table A-1 : Applicability of IPC/WHMA-A-620E(-S)	47

Introduction

Electrical harnesses are used to interconnect electronic equipment and as such they have a critical function in any spacecraft.

A harness typically consists of one or more Harness Manufacturing Units (HMU). Each of which is made by arranging wires or cables and electrical termination components in such a way that can be manufactured, handled and integrated as one assembly. Wires and cables may be twisted and tied together to form bundles, and may be protected by common jackets, sleeves or braids.

Electrical harnesses are subjected to the constrains from the manufacturing, assembly, integration and testing/verification phases on ground in addition to those from the launch and space environment.

Electrical harness reliability depends on the robustness of the manufacturing processes, tests and inspections, for which this standard specifies requirements and acceptance criteria.

This standard to large extent applies the well-known IPC standard IPC/WHMA-A-620E ‘Requirements and Acceptance for Cable and Wire Harness Assemblies’ in conjunction with its addendum IPC/WHMA-A-620E-S ‘Space and Military Applications Electronic Hardware Addendum to IPC/WHMA-A-620E’.

Within this standard, clauses and requirements from these IPC standards are made either applicable as is, applicable with modifications, applicable with amendments or not applicable.

In addition, new ECSS requirements are formulated to reflect the needs and best practices in the European Space Industry.

# Scope

This standard specifies requirements for the manufacturing and control electrical harnesses.

This standard does not apply to Coaxial Cables Assemblies and Optical links. Requirements for Coaxial Cables Assemblies can be found in ECSS-Q-ST-70-18 'Preparation, assembly and mounting of RF coaxial cables’.

This standard does not cover requirements for soldering. Requirements for soldering can be found in ECSS-Q-ST-70-61 'Space product assurance – High reliability assembly for surface mount and through hole connections'.

This standard does not specify requirements for crimping. Requirements for crimping can be found in ECSS-Q-ST-70-26 'Space product assurance – Crimping of high-reliability electrical connections'.

This standard needs to be tailored to the specific characteristic and constrains of the space project on which it is applied in conformance with ECSS-S-ST-00-02.

For specialized harness technologies (such as RF, high data rate and optical data bus interfaces), for which dedicated ECSS standards and ESCC specifications exist, those documents supersede requirements specified in this ECSS standard in case of clashes.

1	For radio frequency and microwave cable assemblies see generic specification ESCC 3408 and subordinate detailed specifications.

2	For high data cable assemblies see generic specification ESCC 3409 and subordinate detailed specifications.

3	For optical cable assemblies see generic specification ESCC 3420 and subordinate detailed specifications.

# Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this ECSS Standard. For dated references, subsequent amendments to, or revision of any of these publications do not apply. However, parties to agreements based on this ECSS Standard are encouraged to investigate the possibility of applying the more recent editions of the normative documents indicated below. For undated references, the latest edition of the publication referred to applies.

| ECSS-S-ST-00-01 | ECSS system – Glossary of terms |
| --- | --- |
| ECSS-M-ST-10 | Space project management - Project planning and implementation |
| ECSS-E-ST-20 | Space engineering – Electric and electronic |
| ECSS-Q-ST-70 | Space product assurance – Material, mechanical parts and processes |
| ECSS-Q-ST-70-26 | Space product assurance – Crimping of high-reliability electrical connections |
| ECSS-Q-ST-70-61 | Space product assurance – High reliability assembly for surface mount and through hole connections |
| ESCC 24800 | Resistance to solvents of marking, materials and finishes |
| IPC/WHMA-A-620E | Requirements and acceptance for cable and wire assemblies |
| IPC/WHMA-A-620E-S | Space and military applications electronic hardware addendum to IPC/WHMA-A-620E |

# Terms, definitions and abbreviated terms

## Terms from other standards

For the purpose of this Standard, the terms and definitions from ECSS-S-ST-00-01 apply, in particular for the following terms:

repair

rework

For the purpose of this Standard, the terms and definitions from IPC/WHMA-A-620E and IPC/WHMA-A-620E-S (Addendum for Space and Military Applications) apply, in particular for the following terms:

defect

engineering documentation

manufacturer

## Terms specific to the present standard

harness manufacturing unit

smallest, single harness item that can be manufactured, assembled, tested and integrated as a separate entity

A harness manufacturing unit can be a complex wiring network, a simple 1:1 cable assembly (e.g. two connectors interconnected by one or multiple wires/cables) or a single termination connector assembly (e.g. safe-and-arm plugs). Harness manufacturing units are typically specified by the harness design authority.

## Abbreviated terms

For the purpose of this Standard, the abbreviated terms and symbols from ECSS-S-ST-00-01 and the following apply:

| Abbreviation | Meaning |
| --- | --- |
| HMU | harness manufacturing unit |

## Conventions

| Convention | Meaning |
| --- | --- |
| IPC/WHMA-A-620E-(S) | Refers to the sum of the IPC/WHMA-A-620E plus the complementary IPC/WHMA-A-620E-S (Addendum for Space and Military Applications requirements) |
| Applicable as is | Indicates that the respective section of the IPC standard is fully applicable without any modifications or amendments by this ECSS standard. |
| Applicable with modification | Indicates that the respective section of the IPC standard is partially modified within this ECSS standard. These modifications supersede the respective statements and requirements in the IPC standard. |
| Applicable with amendment | Indicates that the respective section of the IPC standard remains fully applicable and is complemented by this ECSS standard. |
| Not applicable | Indicates that the respective section of the IPC standard is fully not applicable |

## Nomenclature

The following nomenclature applies throughout this document:

- The word “shall” is used in this Standard to express requirements. All the requirements are expressed with the word “shall”.

The word “should” is used in this Standard to express recommendations. All the recommendations are expressed with the word “should”.

It is expected that, during tailoring, recommendations in this document are either converted into requirements or tailored out.

The words “may” and “need not” are used in this Standard to express positive and negative permissions, respectively. All the positive permissions are expressed with the word “may”. All the negative permissions are expressed with the words “need not”.

The word “can” is used in this Standard to express capabilities or possibilities, and therefore, if not accompanied by one of the previous words, it implies descriptive text.

In ECSS “may” and “can” have completely different meanings: “may” is normative (permission), and “can” is descriptive.

The present and past tenses are used in this Standard to express statements of fact, and therefore they imply descriptive text.

# Principles

## General

The design and definition of a harness is always specific to the user needs. The detailed requirements applicable to a given harness is expected to be included in the engineering documentation agreed upon between the customer and the harness manufacturer.

The existence of the requirements specified in this standard as well as compliance to these requirements does not automatically render the related techniques suitable and qualified for the respective, specific mission application.

Compliance to the requirements in this standard does not automatically establish qualification of the final harness product.

For especially demanding applications, such as cryogenic or highly contamination sensitive, careful tailoring of this standard should be performed to ensure the required performance related to the specific application profile.

## Use of this standard

This standard cannot be used as a standalone document, but needs to be used in conjunction with IPC/WHMA-A-620E and IPC/WHMA-A-620E-S (Addendum for Space and Military Applications).

Clause 6 of this standard is structured in accordance with the sections of the applicable IPC standard. This is done to ease future modifications in line with the given structure of the IPC standard.

## Order of precedence for requirements

Requirements in the engineering document always supersede requirements in this standard. Clashes between engineering documentation provided by the customer and this standard should be brought to the attention of the customer for clarification via statement of compliance or verification control process.

When no requirements are specified in the engineering document for a manufacturing operation, requirements from this standard should be considered with the following order of precedence:

- Requirements specified in clause 7 of this standard.

- Requirements specified in IPC/WHMA-A-620E-S considering applicability in clause 6 of this standard.

- Requirements specified in IPC/WHMA-A-620E for product Class 3 with modifications and amendments given in clause 6 of this standard.

Product Class 1 and 2 as defined in IPC/WHMA-A-620E are considered insufficient for this standard.

For any cross-reference given within the IPC/WHMA-A-620E-(S), the tailoring specified in clause 6 of this standard applies.

For instance, IPC/WHMA-A-620E-S section 17.1 is declared [applicable as is], with some requirements making reference to IPC sections 4 and 5. However, sections 4 and 5 are declared [not applicable] as per clause 6 of this ECSS Standard. As a result, these requirements from section 17.1 are not applicable.

# General requirements

The use of any technique or technology involved in harness manufacturing shall be agreed upon between customer and manufacturer.

Examples for techniques (non-exhaustive) are crimping, soldering, splicing, welding and potting.

The manufacturer shall establish a qualified process in accordance with ECSS-Q-ST-70 for each technique used for the harness manufacturing.

All processes are expected to be listed and approved via the DPL issued by the manufacturer in accordance with ECSS-Q-ST-70, Annex C.

IPC/WHMA-A-620E-S (Addendum for Space and Military Applications Requirements) and IPC/WHMA-A-620E shall apply with the conventions for ‘Applicable as is’, ‘Applicable with modification’, ‘Applicable with amendment’ and ‘Not applicable’ defined in clause 3.4.

# Applicability of IPC/WHMA-A-620E(-S)

This clause specifies the applicability of the IPC/WHMA-A-620E-S (Addendum for Space and Military Applications Requirements) and IPC/WHMA-A-620E.

The applicability statements, as defined in clause 3.4, for a given section of IPC/WHMA-A-620E(-S) is applicable for all its sub-sections unless for a given sub-section another applicability statement is made.

## IPC/WHMA-A-620E-S, § 0: Addendum for Space and Military Applications Requirements

### IPC/WHMA-A-620E-S, § 0.1: Scope

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 0.1.1: Purpose

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 0.1.2: Precedence

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 0.1.3: Existing or Previously Approved Design

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 0.1.4: Use of this Addendum

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 0.1.5: Lead (Pb)-Free Tin

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 0.1.6: Red Plague (Cuprous Oxide Corrosion)

[Applicable with modification]

For wires and cables, which are manufactured, qualified, procured and handled in accordance with ESCC specifications, a Red Plague control plan may be omitted.

#### IPC/WHMA-A-620E-S, § 0.1.7: White Plague (Fluorine Attack)

[Applicable with modification]

For wires and cables, which are manufactured, qualified, procured and handled in accordance with ESCC specifications, a White Plague control plan may be omitted.

## IPC/WHMA-A-620E, § 1.0: General

### IPC/WHMA-A-620E-S, § 1.1: Scope

[Applicable as is]

### IPC/WHMA-A-620E-S, § 1.2: Purpose

[Applicable as is]

### IPC/WHMA-A-620E-S, § 1.3: Classification

[Applicable as is]

### IPC/WHMA-A-620E, § 1.4: Measurement Units and Applications

[Applicable as is]

### IPC/WHMA-A-620E, § 1.5: Definition of requirements

[Applicable as is]

#### IPC/WHMA-A-620E, § 1.5.1: Inspection Conditions

[Applicable as is]

##### IPC/WHMA-A-620E, § 1.5.1.1: Acceptable

[Applicable as is]

##### IPC/WHMA-A-620E, § 1.5.1.2: Defect

[Applicable as is]

###### IPC/WHMA-A-620E-S, § 1.5.1.2.1: Disposition

[Applicable as is]

##### IPC/WHMA-A-620E, § 1.5.1.3: Process indicator

[Applicable as is]

##### IPC/WHMA-A-620E, § 1.5.1.4: Condition Not Specified

[Applicable as is]

##### IPC/WHMA-A-620E, § 1.5.1.5: Uncommon or Specialized Designs

[Applicable as is]

#### IPC/WHMA-A-620E, § 1.5.2: Material and Process Nonconformance

[Applicable as is]

### IPC/WHMA-A-620E-S, § 1.6: Process Control

[Applicable as is]

#### IPC/WHMA-A-620E, § 1.6.1: Statistical Process Control

[Applicable as is]

### IPC/WHMA-A-620E, § 1.7: Order of Precedence

[Applicable as is]

### IPC/WHMA-A-620E, § 1.8: Terms and Definitions

[Applicable as is]

### IPC/WHMA-A-620E, § 1.9: Requirements Flowdown

[Not applicable]

Requirement flow down shall be performed in accordance with ECSS-M-ST-10.

### IPC/WHMA-A-620E-S, § 1.10: Personnel Proficiency

[Applicable with modification]

Modifications to the IPC/WHMA-A-620E(-S) specified in the frame of this ECSS standard cannot be covered by the IPC/WHMA-A-620-Space Addendum Training and Certification Program.

Modifications and amendments made to the IPC baseline by this standard shall be covered by a supplier developed program that is provided to the customer for review and approval.

For a manufacturer developed program it is still recommended, but not mandatory, that the training of personnel related to the IPC/WHMA-A-620-Space Addendum Training and Certification Program is traceable to a Master IPC trainer.

### IPC/WHMA-A-620E, § 1.11: Acceptance Requirements

[Applicable as is]

### IPC/WHMA-A-620E, § 1.12: Inspection Methodology

#### IPC/WHMA-A-620E, § 1.12.1: Process Verification Inspection

[Applicable as is]

#### IPC/WHMA-A-620E, § 1.12.2: Visual Inspection

[Applicable as is]

##### IPC/WHMA-A-620E-S, § 1.12.2.1: Lighting

[Applicable as is]

##### IPC/WHMA-A-620E-S, § 1.12.2.2: Magnification Aids

[Applicable as is]

### IPC/WHMA-A-620E, § 1.13: Facilities

[Applicable as is]

### IPC/WHMA-A-620E, § 1.14: Electrostatic Discharge (ESD) Protection

[Applicable as is]

### IPC/WHMA-A-620E, § 1.15: Tools and Equipment

[Applicable as is]

### IPC/WHMA-A-620E(-S), § 1.16: Materials and Processes

[Not applicable]

Materials and Processes shall be handled in accordance with ECSS-Q-ST-70.

### IPC/WHMA-A-620E, § 1.17: Electrical Clearance

[Applicable as is]

### IPC/WHMA-A-620E, § 1.18: Contamination

[Applicable as is]

### IPC/WHMA-A-620E, § 1.19: Rework/Repair

[Applicable as is]

#### IPC/WHMA-A-620E, § 1.19.1: Rework

[Not applicable]

The definition of ‘rework’ as per ECSS-S-ST-00-01 applies.

Rework shall be documented.

Rework shall be performed in accordance with the applicable requirements in this standard.

#### IPC/WHMA-A-620E, § 1.19.2: Repair

[Not applicable]

The definition of ‘repair’ as per ECSS-S-ST-00-01 applies.

Repair shall be documented.

Repair shall be performed in accordance with documented procedures agreed between the manufacturer and the customer.

Any deviation of the repair procedure to the requirements in this standard and any deviation of the final product as consequence of the repair procedure to the requirements in this standard shall be highlighted by the manufacturer and agreed with the customer prior to performing.

For deviations a “Request for Deviation” or “Waiver” can be used.

#### IPC/WHMA-A-620E, § 1.19.3: Post Rework/Repair Cleaning

[Applicable as is]

## IPC/WHMA-A-620E, § 2.0: Applicable documents

[Not applicable]

Applicable documents listed in the IPC/WHMA-A-620E are considered for reference only unless specifically referenced by a requirement made applicable by this standard.

## IPC/WHMA-A-620E, § 3.0: Wires

[Applicable as is]

### IPC/WHMA-A-620E(-S), § 3.1: Stripping

- [Not applicable]

### IPC/WHMA-A-620E(-S), § 3.2: Strand Damage and End Cuts

- [Not applicable]

### IPC/WHMA-A-620E, § 3.3: Conductor Deformation/Birdcaging

- [Not applicable]

### IPC/WHMA-A-620E, § 3.4: Twisting of Wires

- [Applicable as is]

### IPC/WHMA-A-620E, § 3.5: Insulation Damage - Stripping

- [Not applicable]

## IPC/WHMA-A-620E(-S), § 4.0: Soldered Terminations

[Not applicable]

## IPC/WHMA-A-620E(-S), § 5.0: Crimp Terminations (Contacts and Lugs)

[Not applicable]

## IPC/WHMA-A-620E(-S), § 6.0: Insulation displacement connection (IDC)

[Not applicable]

## IPC/WHMA-A-620E(-S), § 7.0: Ultrasonic Welding

[Not applicable]

## IPC/WHMA-A-620E, § 8.0: Splices

[Applicable as is]

### IPC/WHMA-A-620E(-S), § 8.1: Soldered Splices

[Not applicable]

### IPC/WHMA-A-620E(-S), § 8.2: Crimped Splices

[Not applicable]

### IPC/WHMA-A-620E(-S), § 8.3: Ultrasonic Weld Splices

[Not applicable]

### IPC/WHMA-A-620E-S, § 8.4: Sleeving over Splices

[Applicable with amendment]

The minimum sleeving overlap shall be 3 mm.

## IPC/WHMA-A-620E, § 9.0: Connectorization

[Applicable as is]

### IPC/WHMA-A-620E, § 9.1: Hardware Mounting

#### IPC/WHMA-A-620E-S, § 9.1.1: Hardware Mounting – Jackpost - Height

[Applicable as is]

#### IPC/WHMA-A-620E, § 9.1.2: Hardware Mounting – Jackscrews - Protrusion

[Applicable as is]

#### IPC/WHMA-A-620E, § 9.1.2: Hardware Mounting – Retaining Clips

[Applicable as is]

#### IPC/WHMA-A-620E, § 9.1.2: Hardware Mounting – Connector Alignment

[Applicable as is]

### IPC/WHMA-A-620E, § 9.2: Strain relief

#### IPC/WHMA-A-620E-S, § 9.2.1: Strain relief - Clamp fit

[Applicable with amendment]

The use of build-up material shall be agreed between manufacturer and customer prior to use.

Wires and cables under a cable clamp shall be protected against mechanical damages induced by the cable clamp by means of a dedicated layer of protective material.

Typical protective, build-up materials utilised for that purpose are sleeves or tapes (e.g. Polyimide/Kapton or Aluminium)

#### IPC/WHMA-A-620E, § 9.2.2: Strain relief - Wire dress

[Applicable as is]

##### IPC/WHMA-A-620E, § 9.2.2.1: Strain relief - Wire dress - Straight approach

[Applicable with amendment]

The following complement is added to clarify the definition of a sharp angle in the frame of the defect definition. A sharp angle is an angle of more than 45° between the longitudinal axis of the contact and the wires (see Figure 6-1).

Figure 6-1: Sharp angle definition

##### IPC/WHMA-A-620E, § 9.2.2.1: Strain relief - Wire dress - Side approach

[Applicable as is]

### IPC/WHMA-A-620E, § 9.3: Sleeving and Boots

[Applicable as is]

### IPC/WHMA-A-620E, § 9.4: Connector damage

#### IPC/WHMA-A-620E, § 9.4.1: Connector Damage - Criteria

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 9.4.2: Connector Damage - Limits - Hard Face - Mating Surface

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 9.4.3: Connector Damage - Limits - Soft Face - Mating Surface or Rear Seal Area

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 9.4.4: Connector Damage - Contacts

[Applicable as is]

##### IPC/WHMA-A-620E-S, § 9.4.4.1: Connector Damage - Contacts - Exposed Sockets, Protected Pins, e.g., MIL-DTL-83513 Micro D

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 9.4.5: Connector Damage – Seals/Gaskets

[Applicable as is]

### IPC/WHMA-A-620E-S, § 9.5: Installation of Contacts and Sealing Plugs into Connectors

[Applicable as is]

#### IPC/WHMA-A-620E, § 9.5.1: Installation of Contacts and Sealing Plugs into Connectors – Installation of Contacts

[Applicable as is]

#### IPC/WHMA-A-620E, § 9.5.2: Installation of Contacts and Sealing Plugs into Connectors – Installation of Sealing Plugs

[Applicable as is]

## IPC/WHMA-A-620E-S, § 10: Over-Molding/Potting

[Applicable as is]

### IPC/WHMA-A-620E, § 10.1: Over-Molding

[Applicable as is]

### IPC/WHMA-A-620E, § 10.2: Potting (Thermoset Molding)

[Applicable as is]

### IPC/WHMA-A-620E, § 10.3: Over-Molding of Flexible Flat Ribbon

[Applicable as is]

## IPC/WHMA-A-620E, § 11.0: Measuring Cable Assemblies and Wires

[Applicable as is]

## IPC/WHMA-A-620E-S, § 12.0: Marking/Labelling

[Applicable as is]

### IPC/WHMA-A-620E, § 12.1: Content

[Applicable as is]

### IPC/WHMA-A-620E, § 12.2: Legibility

[Applicable as is]

### IPC/WHMA-A-620E, § 12.3: Permanency

[Applicable with amendment]

Marking and colour-coding shall be resistant to solvents as specified in ESCC 24800.

### IPC/WHMA-A-620E-S, § 12.4: Location and Orientation

[Applicable as is]

### IPC/WHMA-A-620E, § 12.5: Functionality

[Applicable as is]

### IPC/WHMA-A-620E, § 12.6: Marker Sleeve

#### IPC/WHMA-A-620E, § 12.6.1: Marker Sleeve – Wrap Around

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 12.6.2: Marker Sleeve – Tubular

[Applicable with amendment]

Marking sleeves shall be free from burn marks or charring due to excessive overheating (see Figure 6-2Figure 6-1).

Figure 6-2: Excessive (defect) overheating of marking sleeves

Slight discoloration due to minor overheating shall not be a cause of rejection but be treated as process indicator in accordance with the IPC/WHMA-A-620 (see Figure 6-3).

Figure 6-3: Minor (acceptable) overheating of marking sleeves

### IPC/WHMA-A-620E, § 12.7: Flag Markers

#### IPC/WHMA-A-620E, § 12.7.1: Flag Markers - Adhesive

[Applicable as is]

### IPC/WHMA-A-620E, § 12.8: Tie Wrap Markers

[Applicable as is]

## IPC/WHMA-A-620E(-S), § 13.0: Coaxial and Biaxial Cable Assemblies

[Not applicable]

Requirements for preparation, assembly and mounting of RF coaxial cables are specified in ECSS-Q-ST-70-18.

## IPC/WHMA-A-620E, § 14.0: Securing

[Applicable as is]

### IPC/WHMA-A-620E-S, § 14.1: Tie Wrap/ Lacing application

[Applicable with amendment]

Cut end protrusion shall be 0 mm to 0,5 mm and square to the face of the tie wrap.

A receded tie-wrap tail with regards to the collar head (see Figure 6-4) is considered as a negative protrusion as thus not acceptable.

Figure 6-3: Tie-wrap – acceptable cut end protrusion

Figure 6-4: Tie-wrap – receded tail

Where a tie wrap is used to fix a harness bundle to a structural element, a dedicated layer of protection shall be applied onto the bundle under the tie wrap.

The term structural element here means any element that prevents the tie-wrap from moving with the harness under mechanical loads.

#### IPC/WHMA-A-620E, § 14.1.1: Tie Wrap/ Lacing Application - Tightness

[Applicable with modification]

The insulation shall not be damaged by the restraining device

Compression of the insulation shall not degrade the performance of the cables and is controlled by a qualified process in accordance with requirement 5b.

Compression of insulation can lead to reduction in insulation thickness and thus affect performance parameters such as insulation resistance and insulation breakdown voltage.

For insulation materials sensitive to creeping (cold-flow), measures shall be taken to protect the respective material from compressive stress induced by the restraining device.

Example for materials sensitive to creeping (cold-flow) are PTFE and FEP.

Measures can include the addition of local cable protection under the restraining device

#### IPC/WHMA-A-620E, § 14.1.2: Tie Wrap/ Lacing Application - Damage

[Applicable as is]

#### IPC/WHMA-A-620E, § 14.1.3: Tie Wrap/ Lacing Application - Spacing

[Applicable as is]

### IPC/WHMA-A-620E, § 14.2: Breakouts

#### IPC/WHMA-A-620E-S, § 14.2.1: Breakouts – Individual Wires

[Applicable as is]

#### IPC/WHMA-A-620E, § 14.2.2: Breakouts - Spacing

[Applicable as is]

### IPC/WHMA-A-620E, § 14.3: Routing

#### IPC/WHMA-A-620E, § 14.3.1: Routing – Wire Crossover

[Applicable as is]

#### IPC/WHMA-A-620E, § 14.3.2: Routing – Bend Radius

[Applicable as is]

#### IPC/WHMA-A-620E, § 14.3.3: Routing – Coaxial Cable

[Applicable as is]

#### IPC/WHMA-A-620E, § 14.3.4: Routing – Unused Wire Termination

[Applicable as is]

#### IPC/WHMA-A-620E, § 14.3.5: Routing – Ties over Splices and Ferrules

[Applicable as is]

### IPC/WHMA-A-620E, § 14.2: Broom Stitching

[Applicable as is]

## IPC/WHMA-A-620E, § 15.0: Harness/Cable Electrical Shielding

### IPC/WHMA-A-620E-S, § 15.1: Braided

[Applicable with modification]

Table 15-1 of IPC/WHMA-A-620E-S is not applicable. Any broken strand shall be considered as a defect.

### IPC/WHMA-A-620E-S, § 15.2: Shield Termination

[Applicable as is]

#### IPC/WHMA-A-620E, § 15.2.1: Shield Termination - Shield Jumper Wire

##### IPC/WHMA-A-620E(-S), § 15.2.1.1: Shield Termination - Shield Jumper Wire - Attached Lead

[Not applicable]

###### IPC/WHMA-A-620E(-S), § 15.2.1.1.1: Shield Termination - Shield Jumper Wire - Attached Lead - Solder

[Not applicable]

###### IPC/WHMA-A-620E, § 15.2.1.1.2: Shield Termination - Shield Jumper Wire - Attached Lead - Crimp

[Not applicable]

##### IPC/WHMA-A-620E(-S), § 15.2.1.2: Shield Termination - Shield Jumper Wire - Shield braid

[Not applicable]

###### IPC/WHMA-A-620E, § 15.2.1.2.1: Shield Termination - Shield Jumper Wire - Shield braid - Woven

[Applicable with modification]

Any broken strand shall be considered as a defect

###### IPC/WHMA-A-620E, § 15.2.1.2.2: Shield Termination - Shield Jumper Wire - Shield braid – Combed and twisted

[Applicable as is]

##### IPC/WHMA-A-620E(-S), § 15.2.1.3: Shield Termination - Shield Jumper Wire – Daisy Chain

[Not applicable]

##### IPC/WHMA-A-620E, § 15.2.1.4: Shield Termination - Shield Jumper Wire – Common Ground point

[Not applicable]

#### IPC/WHMA-A-620E, § 15.2.2: Shield Termination – Unterminated Shield

[Applicable as is]

##### IPC/WHMA-A-620E, § 15.2.2.1: Shield Termination - Unterminated Shield - Shield Not Folded Back

[Applicable with modification]

Any criteria (acceptable, process indicator or defect) limiting the stripped shield length specified in this IPC section is considered not applicable.

An additional insulation layer shall be added between the exposed shielding and the wire insulation and shall extend beyond the end of the exposed shield by at least 2 mm to prevent the sharp braid ends from damaging the wire insulation.

##### IPC/WHMA-A-620E, § 15.2.2.1: Shield Termination - Unterminated Shield - Shield Folded Back

[Applicable as is]

### IPC/WHMA-A-620E, § 15.3: Shield Termination - Connector

#### IPC/WHMA-A-620E-S, § 15.3.1: Shield Termination - Connector - Shrink

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 15.3.2: Shield Termination - Connector - Crimp

[Applicable with modification]

The crimp ring shall not extend beyond the crimp area.

The number of shield strands not contained under the crimp ring shall be less than 6% of the total number of strands within the shield.

Trimmed strand ends shall be covered by a protective insulation layer.

#### IPC/WHMA-A-620E-S, § 15.3.1: Shield Termination - Connector - Shield Jumper Wire attachment

[Applicable as is]

#### IPC/WHMA-A-620E, § 15.3.1: Shield Termination - Connector - Soldered

[Not applicable]

### IPC/WHMA-A-620E, § 15.4.1: Shield Termination - Splicing Prewoven

[Applicable as is]

#### IPC/WHMA-A-620E(-S), § 15.4.1: Shield Termination - Splicing Prewoven - Soldered

[Not applicable]

#### IPC/WHMA-A-620E, §15.4.2: Shield termination - Splicing Prewoven - Tie/Tape On

[Applicable as is]

### IPC/WHMA-A-620E, § 15.5: Tapes - Barrier and Conductive, Adhesive or Non-Adhesive

[Applicable with amendment]

IPC/WHMA-A-620E section 15.5 is not applicable for EMI shielding or shielding against ionizing radiation.

The application of shielding for EMI and ionizing radiation is described in clause 7.8 of this standard.

### IPC/WHMA-A-620E, § 15.6: Conduit (Shielding)

[Applicable as is]

### IPC/WHMA-A-620E(-S), § 15.6: Shrink Tubing – Conductive Lined

[Not applicable]

## IPC/WHMA-A-620E, § 16.0: Cable/Wire Harness Protective Coverings

[Applicable as is]

### IPC/WHMA-A-620E, § 16.1: Braid

#### IPC/WHMA-A-620E, § 16.1.1: Braid – Direct Applied

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 16.1.2: Braid - Prewoven

[Applicable as is]

### IPC/WHMA-A-620E, § 16.2: Sleeving/Shrink Tubing

[Applicable as is]

### IPC/WHMA-A-620E(-S), § 16.3: Spiral Plastic Wrap (Spiral Wrap Sleeving)

[Not applicable]

### IPC/WHMA-A-620E, § 16.4: Wire Loom Tubing – Split and Unsplit

[Applicable as is]

### IPC/WHMA-A-620E, § 16.5: Tapes, Adhesive and Non-Adhesive

[Applicable as is]

## IPC/WHMA-A-620E-S, § 17.0: Finished assembly Installation

[Applicable as is]

### IPC/WHMA-A-620E-S, § 17.1: General

[Applicable as is]

### IPC/WHMA-A-620E-S, § 17.2: Hardware installation

[Applicable as is]

#### IPC/WHMA-A-620E, § 17.2.1: Hardware installation – Threaded Fasteners

[Not applicable]

#### IPC/WHMA-A-620E(-S), § 17.2.2: Hardware installation - Wires

[Not applicable]

#### IPC/WHMA-A-620E, § 17.2.3: Hardware installation – Safety Wiring

[Not applicable]

### IPC/WHMA-A-620E, § 17.3: Wire/Harness installation

[Not applicable]

## IPC/WHMA-A-620E(-S), § 18.0: Solderless Wrap

[Not applicable]

Solderless Wrap connection requirements are specified in ECSS-Q-ST-70-30.

## IPC/WHMA-A-620E, § 19.0: Testing

[Applicable as is]

### IPC/WHMA-A-620E, § 19.1: Nondestructive Tests

[Applicable as is]

### IPC/WHMA-A-620E, § 19.2: Testing After Rework or Repair

[Applicable as is]

### IPC/WHMA-A-620E, § 19.3: Intended Table Usage

[Applicable as is]

### IPC/WHMA-A-620E-S, § 19.4: Electrical Test

[Applicable with Amendment]

The manufacturer shall maintain a mating and de-mating logbook for each connector, including at a minimum

Project designation

Harness / HMU identifier

CI number of the Harness / HMU

Serial number (SN) of the Harness / HMU

Connector identifier

Connectors are typically identified using the scheme UNIT_A000, where:

‘UNIT’ denotes the name of the unit (e.g. PCDU)

‘A’ denotes the connector gender with P for Plug and J for Jack

‘000’ denotes a sequential number unique to the unit, starting with 001

Dedicated record of each mating and de-mating operation

Date and signature for each mating and de-mating operation by the respective operator or inspector

Maximum number of allowed mating cycles

1	A mating cycle is defined as one (1) mating and one (1) de-mating operation

2	The maximum number of allowed cycles is typically specified in the engineering documentation, manufacturer datasheet or applicable standards

(De-)Mating justification

1	Typically includes the reason for the mating operation (e.g. electrical testing, electrical AIT) and indicates the use of savers or mating to flight equipment

2	Annex B can be used as template for the mating logbook.

The test procedure shall ensure that the harness is fully discharged at the end of each test and prior to disconnecting the harness from the test equipment.

#### IPC/WHMA-A-620E-S, § 19.4.1: Electrical Test - Selection

[Applicable with modification]

The ‘Shorts’ test is not required by default and shall only be performed when specified by the customer.

### IPC/WHMA-A-620E, § 19.5: Electrical Test Methods

#### IPC/WHMA-A-620E, § 19.5.1: Electrical Test Methods - Continuity

[Applicable with amendment]

Continuity testing shall be performed on any intentionally conductive paths, including shielding.

#### IPC/WHMA-A-620E-S, § 19.5.1: Electrical Test Methods - Shorts

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 19.5.1: Electrical Test Methods - Dielectric Withstanding Voltage (DWV)

[Applicable as is]

#### IPC/WHMA-A-620E-S, § 19.5.1: Electrical Test Methods - Insulation Resistance (IR)

[Applicable as is]

#### IPC/WHMA-A-620E, § 19.5.1: Electrical Test Methods - Voltage Standing Wave Ratio (VSWR)

[Not applicable]

#### IPC/WHMA-A-620E, § 19.5.1: Electrical Test Methods - Insertion Loss

[Not applicable]

#### IPC/WHMA-A-620E, § 19.5.1: Electrical Test Methods - Reflection Coefficient

[Not applicable]

#### IPC/WHMA-A-620E, § 19.5.1: Electrical Test Methods - User Defined

[Applicable as is]

### IPC/WHMA-A-620E, § 19.6: Mechanical Test

#### IPC/WHMA-A-620E(-S), § 19.6.1: Mechanical Test - Selection

[Not applicable]

Contact Retention Verification shall be performed in accordance with IPC/WHMA-A-620E-S, section 19.7.5.

### IPC/WHMA-A-620E, § 19.7: Mechanical Test Methods

#### IPC/WHMA-A-620E(-S), § 19.7.1: Mechanical Test Methods – Crimp Height (Dimensional Analysis)

[Not applicable]

#### IPC/WHMA-A-620E(-S), § 19.7.2: Mechanical Test Methods – Pull Force (Tensile)

[Not applicable]

#### IPC/WHMA-A-620E, § 19.7.3: Mechanical Test Methods – Crimp Force Monitoring

[Not applicable]

#### IPC/WHMA-A-620E, § 19.7.4: Mechanical Test Methods – Crimp Tool Qualification

[Not applicable]

#### IPC/WHMA-A-620E-S, § 19.7.5: Mechanical Test Methods – Contact Retention Verification

[Applicable as is]

##### IPC/WHMA-A-620E-S, § 19.7.5.1: Mechanical Test Methods – Contact Retention Verification – Push Testing

[Applicable as is]

##### IPC/WHMA-A-620E-S, § 19.7.5.2: Mechanical Test Methods – Contact Retention Verification – Pull Testing

[Applicable as is]

#### IPC/WHMA-A-620E(-S), § 19.7.6: Mechanical Test Methods – RF Connector Shield Pull Force (Tensile)

[Not applicable]

#### IPC/WHMA-A-620E, § 19.7.7: Mechanical Test Methods – RF Connector Shield Ferule Torsion

[Not applicable]

#### IPC/WHMA-A-620E, § 19.7.7: Mechanical Test Methods – User Defined

[Applicable as is]

## IPC/WHMA-A-620E, § 20.0: High Voltage Applications

[Not Applicable]

- Soldering requirements for high voltage applications are specified in ECSS-Q-ST-70-61.

# Complementary requirements

## Information

This clause specifies topics and respective requirements to complement and/or substitute parts of the IPC standards.

## General Requirements

Any bare conductor at the back of a connector shall be insulated.

The back of a connector is where the contact, crimped to a wire, enters the connector insulator as opposed to the front of the connector which mated the respective interface counterpart.

Common means to implement insulation are:

- - Application of insulation sleeves

- - Potting

- - Use of grommets

## Soldering

All processes related to soldering operations shall be performed in accordance with ECSS-Q-ST-70-61.

## Crimping

All processes related to crimping operations shall be performed in accordance with ECSS-Q-ST-70-26.

## Insulation sleeving on contact termination

Insulation sleeving shall overlap the contact barrel and the wire insulation by a minimum of two times the wire diameter or 3 mm, whichever is greater.

Insulation sleeving shall not prevent the movement of the contact inside connector insulator.

Insulation sleeve shall not slide or vibrate off.

Additional shrinking sleeves can be added to build up the diameter of the wire so the outer shrinking sleeve is tight on the assembly from one end to the other as illustrated in Figure 7-1.

Figure 7-1: Double sleeving on D-Sub contacts

Insulation sleeve shall not cover the inspection hole as illustrated in Figure 7-2.

Figure 7-2: Insulation sleeving over pin contact

In case the geometry of the connector allows, the insulation sleeve shall enter into the insulator cavity of the connector, not leaving any exposed conductor detectable by visual inspection at a 45° angle to the cavity axis as illustrated in Figure 7-3.

Verification can be done by visual inspection only.

Figure 7-3: Inspection of exposed conductor into connector insulator

In case the geometry of the connector does not allow the insulation sleeve to enter into the insulator cavity of the connector, the gap between the insulator and the sleeve shall be 0,5 mm or less from the point where the connector terminal enters the connector insert.

Examples of acceptable and unacceptable gaps are depicted in Figure 7-4.

In case the connector is subject to reliable insulation, clause 4.2.1.2 of ECSS-E-ST-20 shall be applied instead of requirement 7.5e and 7.5f.

Acceptable		                              Defect

Figure 7-4: Gap between sleeve and insulating insert

There shall be no foreign particles entrapped underneath the shrinking sleeve.

Examples of a foreign particle trapped underneath a shrinking sleeve is shown in Figure 7-5.

Figure 7-5: Example of foreign particles entrapped under shrinking sleeve

Shrinking sleeve shall not show any signs of excessive deformation or stress.

For example whitening as shown in Figure 7-6

Figure 7-6: Example of sleeve showing signs of excessive deformation

## Labelling and marking:

For connector identification labels applied longitudinally to the harness bundle the read direction of the marking on the label shall be towards connector (see Figure 7-7).

Figure 7-7: Connector label applied onto harness bundle

For connector identification labels applied perpendicular to the harness bundle the marking on the label shall be readable upright looking from harness side towards connector (see Figure 7-8).

Figure 7-8: Marking on connector backshell or with a flag

## Branching of metallic braid-shielded bundles:

An additional insulation layer shall be positioned under shielding and shall extend beyond shielding termination to prevent sharp edges from cut braid ends to damage the underlying wire and cable insulation (see Figure 7-9).

Overlapping braids shall be secured on all branches over the full length of the overlap (see Figure 7-10).

The required optical coverage of the braided shield shall be maintained across the entire branching area (see Figure 7-11).

The ends of the braid shall be secured.

The braids of the different branches shall overlap for at least 30 mm or two times the diameter of the largest bundle, whichever is longer, at the breakout.

The open ends of the braid shall be cut in a uniform manner.

The open ends of the braid shall be free of fraying, unraveling and pulled loops (see Figure 7-11).

The braid shall be free of bulges and dents.

The braid shall be free of tears and cuts (see Figure 7-11).

Figure 7-9: Insulation protection under the shield at the breakout

Figure 7-10: Secured braid along the breakout

Figure 7-11: Example of defective braided shield

## EMC shielding with conductive metal foil

Shielding of harness bundles with conductive, metal foil to achieve Electromagnetic Compatibility shall be performed by helicoidal wrapping around the bundle as depicted in Figure 7-12.

Figure 7-12: Helicoidal wrapping of conductive shield

The wrapping shall be performed with an overlap of at least 50 %.

Handling and in particular bending of harness bundles can alter the overlap of the wrapping locally when using non-adhesive foils.

The harness manufacturer shall ensure the electrical continuity of the shield along the entire harness.

For example by wrapping using a single, continuous piece of foil or utilizing a drain wire.

Wrapping shall be tight to the bundle as shown in Figure 7-13 and not slide along the bundle as indicated in Figure 7-14.

Figure 7-13: Example of acceptable foil shielding

Figure 7-14: Example of not acceptable, loose foil shielding

The ends of the foil pieces shall be secured.

The foil shall be free of tears, cracks and holes as shown in Figure 7-15.

Figure 7-15: Example of not acceptable foil shielding

The foil shall be free of wrinkles that jeopardize shield continuity and proper shield function or are prone to tearing.

Minor wrinkles are considered unavoidable and acceptable.

Foil wrappings of the different branches shall overlap at the breakout for at least 30 mm or two times the diameter of the largest bundle, whichever is longer.

(informative)
Applicability tables

IPC/WHMA-A-620E(-S) applicability table

Table A-1 provides the applicability of all sections for both IPC/WHMA-A-620E and the Addendum for Space and Military Applications IPC/WHMA-A-620E-S in line with clause 6 of this standard for quick and easy reference.

: Applicability of IPC/WHMA-A-620E(-S)

| Section | Section Title | Applicability | Standard |
| --- | --- | --- | --- |
| 0 | Addendum for Space and Military Applications Requirements | - | 620E-S |
| 0.1 | Scope | as is | 620E-S |
| 0.1.1 | Purpose | as is | 620E-S |
| 0.1.2 | Precedence | as is | 620E-S |
| 0.1.3 | Existing or Previously Approved Design | as is | 620E-S |
| 0.1.4 | Use of this Addendum | as is | 620E-S |
| 0.1.5 | Lead (Pb)-Free Tin | as is | 620E-S |
| 0.1.6 | Red Plague (Cuprous Oxide Corrosion) | with modification | 620E-S |
| 0.1.7 | White Plague (Fluorine Attack) | with modification | 620E-S |
| 1.0 | General | - | 620E |
| 1.1 | Scope | as is | 620E-S |
| 1.2 | Purpose | as is | 620E-S |
| 1.3 | Classification | as is | 620E-S |
| 1.4 | Measurement Units and Applications | as is | 620E |
| 1.5 | Definition of Requirements | as is | 620E |
| 1.5.1 | Inspection Conditions | as is | 620E |
| 1.5.1.1 | Acceptable | as is | 620E |
| 1.5.1.2 | Defect | as is | 620E |
| 1.5.1.2.1 | Disposition | as is | 620E-S |
| 1.5.1.3 | Process indicator | as is | 620E |
| 1.5.1.4 | Condition Not Specified | as is | 620E |
| 1.5.1.5 | Uncommon or Specialized Designs | as is | 620E |
| 1.5.2 | Material and Process Nonconformance | as is | 620E |
| 1.6 | Process Control | as is | 620E-S |
| 1.6.1 | Statistical Process Control | as is | 620E |
| 1.7 | Order of Precedence | as is | 620E |
| 1.8 | Terms and Definitions | as is | 620E |
| 1.9 | Requirements Flowdown | not applicable | 620E |
| 1.10 | Personnel Proficiency | with modification | 620E-S |
| 1.11 | Acceptance Requirements | as is | 620E |
| 1.12 | Inspection Methodology | - | 620E |
| 1.12.1 | Process Verification Inspection | as is | 620E |
| 1.12.2 | Visual Inspection | as is | 620E |
| 1.12.2.1 | Lighting | as is | 620E-S |
| 1.12.2.2 | Magnification Aids | as is | 620E-S |
| 1.13 | Facilities | as is | 620E |
| 1.14 | Electrostatic Discharge (ESD) Protection | as is | 620E |
| 1.15 | Tools and Equipment | as is | 620E |
| 1.16 | Materials and Processes | not applicable | Both |
| 1.17 | Electrical Clearance | as is | 620E |
| 1.18 | Contamination | as is | 620E |
| 1.19 | Rework/Repair | as is | 620E |
| 1.19.1 | Rework | not applicable | 620E |
| 1.19.2 | Repair | not applicable | 620E |
| 1.19.3 | Post Rework/Repair Cleaning | as is | 620E |
| 2.0 | Applicable Documents | not applicable | 620E |
| 3.0 | Wires | as is | 620E |
| 3.1 | Stripping | not applicable | Both |
| 3.2 | Strand Damage and End Cuts | not applicable | Both |
| 3.3 | Conductor Deformation/Birdcaging | not applicable | 620E |
| 3.4 | Twisting of Wires | as is | 620E |
| 3.5 | Insulation Damage – Stripping | not applicable | 620E |
| 4.0 | Soldered Terminations | not applicable | Both |
| 5.0 | Crimp Terminations (Contacts and Lugs) | not applicable | Both |
| 6.0 | Insulation Displacement Connection (IDC) | not applicable | Both |
| 7.0 | Ultrasonic Welding | not applicable | Both |
| 8.0 | Splices | as is | 620E |
| 8.1 | Soldered Splices | not applicable | Both |
| 8.2 | Crimped Splices | not applicable | Both |
| 8.3 | Ultrasonic Weld Splices | not applicable | Both |
| 8.4 | Sleeving over Splices | with amendment | 620E-S |
| 9.0 | Connectorization | as is | 620E |
| 9.1 | Hardware Mounting | - | 620E |
| 9.1.1 | Hardware Mounting – Jackpost - Height | as is | 620E-S |
| 9.1.2 | Hardware Mounting – Jackscrews - Protrusion | as is | 620E |
| 9.1.3 | Hardware Mounting – Retaining Clips | as is | 620E |
| 9.1.4 | Hardware Mounting – Connector Alignment | as is | 620E |
| 9.2 | Strain Relief | - | 620E |
| 9.2.1 | Clamp Fit | with amendment | 620E-S |
| 9.2.2 | Wire Dress | as is | 620E |
| 9.2.2.1 | Straight Approach | with amendment | 620E |
| 9.2.2.2 | Side Approach | as is | 620E |
| 9.3 | Sleeving and Boots | as is | 620E |
| 9.4 | Connector Damage | - | 620E |
| 9.4.1 | Criteria | as is | 620E |
| 9.4.2 | Mating Surface | as is | 620E-S |
| 9.4.3 | Mating Surface or Rear Seal Area | as is | 620E-S |
| 9.4.4 | Contacts | as is | 620E-S |
| 9.4.4.1 (a) | MIL-DTL-83513 Micro D | as is | 620E-S |
| 9.4.5 (a) | Seals/Gaskets | as is | 620E-S |
| 9.5 | Installation of Contacts and Sealing Plugs into Connectors | as is | 620E-S |
| 9.5.1 | Installation of Contacts | as is | 620E |
| 9.5.2 | Installation of Sealing Plugs | as is | 620E |
| 10.0 | Over-Molding/Potting | as is | 620E-S |
| 10.1 | Over-Molding | as is | 620E |
| 10.2 | Potting (Thermoset Molding) | as is | 620E |
| 10.3 | Over-Molding of Flexible Flat Ribbon | as is | 620E |
| 11.0 | Measuring Cable Assemblies and Wires | as is | 620E |
| 12.0 | Marking/Labelling | as is | 620E-S |
| 12.1 | Content | as is | 620E |
| 12.2 | Legibility | as is | 620E |
| 12.3 | Permanency | with amendment | 620E |
| 12.4 | Location and Orientation | as is | 620E-S |
| 12.5 | Functionality | as is | 620E |
| 12.6 | Marker Sleeve | - | 620E |
| 12.6.1 | Wrap Around | as is | 620E |
| 12.6.2 | Tubular | with amendment | 620E-S |
| 12.7 | Flag Markers | - | 620E |
| 12.7.1 | Adhesive | as is | 620E |
| 12.8 | Tie Wrap Markers | as is | 620E |
| 13.0 | Coaxial and Biaxial Cable Assemblies | not applicable | Both |
| 14.0 | Securing | as is | 620E |
| 14.1 | Tie Wrap/Lacing Application | with amendment | 620E-S |
| 14.1.1 | Tightness | with modification | 620E |
| 14.1.2 | Damage | as is | 620E |
| 14.1.3 | Spacing | as is | 620E |
| 14.2 | Breakouts | - | 620E |
| 14.2.1 | Individual Wires | as is | 620E-S |
| 14.2.2 | Spacing | as is | 620E |
| 14.3 | Routing | - | 620E |
| 14.3.1 | Wire Crossover | as is | 620E |
| 14.3.2 | Bend Radius | as is | 620E |
| 14.3.3 | Coaxial Cable | as is | 620E |
| 14.3.4 | Unused Wire Termination | as is | 620E |
| 14.3.5 | Ties over Splices and Ferrules | as is | 620E |
| 14.4 | Broom Stitching | as is | 620E |
| 15.0 | Harness/Cable Electrical Shielding | - | 620E |
| 15.1 | Braided | with modification | 620E-S |
| 15.2 | Shield Termination | as is | 620E-S |
| 15.2.1 | Shield Jumper Wire | - | 620E |
| 15.2.1.1 | Attached Lead | not applicable | Both |
| 15.2.1.1.1 | Solder | not applicable | Both |
| 15.2.1.1.2 | Crimp | not applicable | 620E |
| 15.2.1.2 | Shield braid | not applicable | Both |
| 15.2.1.2.1 | Woven | with modification | 620E |
| 15.2.1.2.2 | Combed and twisted | as is | 620E |
| 15.2.1.3 | Daisy Chain | not applicable | Both |
| 15.2.1.4 | Common Ground Point | not applicable | 620E |
| 15.2.2 | Unterminated Shield | as is | 620E |
| 15.2.2.1 | Shield Not Folded Back | with modification | 620E |
| 15.2.2.2 | Shield Folded Back | as is | 620E |
| 15.3 | Connector | - | 620E |
| 15.3.1 | Shrink | as is | 620E-S |
| 15.3.2 | Crimp | with modification | 620E-S |
| 15.3.3 | Shield Jumper Wire Attachment | as is | 620E-S |
| 15.3.4 | Soldered | not applicable | 620E |
| 15.4 | Splicing Prewoven | as is | 620E |
| 15.4.1 | Soldered | not applicable | Both |
| 15.4.2 | Tie/Tape On | as is | 620E |
| 15.5 | Tapes – Barrier and Conductive, Adhesive or Non-Adhesive | with amendment | 620E |
| 15.6 | Conduit (Shielding) | as is | 620E |
| 15.7 | Shrink Tubing – Conductive Lined | not applicable | Both |
| 16.0 | Cable/Wire Harness Protective Coverings | as is | 620E |
| 16.1 | Braid | - | 620E |
| 16.1.1 | Braid – Direct Applied | as is | 620E |
| 16.1.2 | Braid - Prewoven | as is | 620E-S |
| 16.2 | Sleeving/Shrink Tubing | as is | 620E |
| 16.3 | Spiral Plastic Wrap (Spiral Wrap Sleeving) | not applicable | Both |
| 16.4 | Wire Loom Tubing – Split and Unsplit | as is | 620E |
| 16.5 | Tapes, Adhesive and Non-Adhesive | as is | 620E |
| 17.0 | Finished assembly Installation | as is | 620E-S |
| 17.1 | General | as is | 620E-S |
| 17.2 | Hardware Installation | as is | 620E-S |
| 17.2.1 | Threaded Fasteners | not applicable | 620E |
| 17.2.2 | Wires | not applicable | Both |
| 17.2.3 | Safety Wiring | not applicable | 620E |
| 17.3 | Wire/Harness Installation | not applicable | 620E |
| 18.0 | Solderless Wrap | not applicable | Both |
| 19.0 | Testing | as is | 620E |
| 19.1 | Nondestructive Tests | as is | 620E |
| 19.2 | Testing After Rework or Repair | as is | 620E |
| 19.3 | Intended Table Usage | as is | 620E |
| 19.4 | Electrical Test | with amendment | 620E-S |
| 19.4.1 | Selection | with modification | 620E-S |
| 19.5 | Electrical Test Methods | - | 620E |
| 19.5.1 | Continuity | with amendment | 620E |
| 19.5.2 | Shorts | as is | 620E-S |
| 19.5.3 | Dielectric Withstanding Voltage (DWV) | as is | 620E-S |
| 19.5.4 | Insulation Resistance (IR) | as is | 620E-S |
| 19.5.5 | Voltage Standing Wave Ratio (VSWR) | not applicable | 620E |
| 19.5.6 | Insertion Loss | not applicable | 620E |
| 19.5.7 | Reflection Coefficient | not applicable | 620E |
| 19.5.8 | User Defined | as is | 620E |
| 19.6 | Mechanical Test | - | 620E |
| 19.6.1 | Selection | not applicable | Both |
| 19.7 | Mechanical Test Methods | - | 620E |
| 19.7.1 | Crimp Height (Dimensional Analysis) | not applicable | Both |
| 19.7.2 | Pull Force (Tensile) | not applicable | Both |
| 19.7.3 | Crimp Force Monitoring | not applicable | 620E |
| 19.7.4 | Crimp Tool Qualification | not applicable | 620E |
| 19.7.5 | Contact Retention Verification | as is | 620E-S |
| 19.7.5.1 (a) | Push Testing | as is | 620E-S |
| 19.7.5.2 (a) | Pull Testing | as is | 620E-S |
| 19.7.6 | RF Connector Shield Pull Force (Tensile) | not applicable | Both |
| 19.7.7 | RF Connector Shield Ferrule Torsion | not applicable | 620E |
| 19.7.8 | User Defined | as is | 620E |
| 20.0 | High Voltage Applications | not applicable | 620E |
| (a) Section not included in IPC/WHMA-A-620E, but only added through the Addendum for Space and Military Applications, IPC/WHMA-A-620E-S |  |  |  |

(informative)
Example of a “(De-)mating Logbook”

| *Company Logo* |  |  |  | (De-)mating Logbook |  |  | Program / Project: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Harness / HMU CI-No.: |  |  |
|  |  |  |  |  |  |  | Harness / HMU Serial-No.: |  |  |
|  |  |  |  |  |  |  | * Harness / HMU Identifier * |  |  |
| Max. no of allowed cycles: |  |  |  | * a cycle is defined as one (1) mating and one (1) demating operation |  |  |  |  |  |
| Connector Identifier | Cycle No. | Date of Mating |  | Operator / Inspector (Sign. / Stamp / Date) | Date of De-Mating | Operator/ Inspector (Sign. / Stamp / Date) |  | Justification / Remarks (indicate reason for mating, use of saver, etc.) |  |

Bibliography

| ECSS-S-ST-00 | ECSS system – Description, implementation and general requirements |
| --- | --- |
| ECSS-Q-ST-70-18 | Space product assurance – Preparation, assembly and mounting of RF coaxial cables |
| ECSS-Q-ST-70-30 | Space product assurance – Wire wrapping of high-reliability electrical connections |
| ESCC 3408 | RF Cable Assemblies |
| ESCC 3409 | High Data Rate Cable Assemblies |
| ESCC 3420 | Optical Fibre Cable Assemblies with Single Fibre Ferrules, Generic |