ECSS Secretariat

ESA-ESTEC

Standards Section

Noordwijk, The Netherlands

Space product assurance

Cleanliness and contamination control

Foreword

ECSS is a cooperative effort of the European Space Agency, national space agencies and European industry associations for the purpose of developing and maintaining common standards. Requirements in this Standard are defined in terms of what shall be accomplished, rather than in terms of how to organize and perform the necessary work. This allows existing organizational structures and methods to be applied where they are effective, and for the structures and methods to evolve as necessary without rewriting the standards.

This Standard has been prepared by the ECSS-Q-ST-70-01C Rev.1 Working Group, reviewed by the ECSS Executive Secretariat and approved by the ECSS Technical Authority.

Disclaimer

ECSS does not provide any warranty whatsoever, whether expressed, implied, or statutory, including, but not limited to, any warranty of merchantability or fitness for a particular purpose or any warranty that the contents of the item are error-free. In no respect shall ECSS incur any liability for any damages, including, but not limited to, direct, indirect, special, or consequential damages arising out of, resulting from, or in any way connected to the use of this Standard, whether or not based upon warranty, business agreement, tort, or otherwise; whether or not injury was sustained by persons or property or otherwise; and whether or not loss was sustained from, or arose out of, the results of, the item, or any services that may be provided by ECSS.

Published by: 	ESA Standards Section

ESTEC, P.O. Box 299,

2200 AG Noordwijk

The Netherlands

Copyright: 	2025 © by the European Space Agency for the members of ECSS

Change log

| ECSS-Q-70-01A 11 December 2002 | First issue Transforming ESA PSS-01-701 into an ECSS Standard |  |
| --- | --- | --- |
| ECSS-Q-70-01B | Never issued |  |
| ECSS-Q-ST-70-01C 15 November 2008 | Second issue The main differences between ECSS-Q-70-01A and this Standard are listed hereunder: - Reorganization of the document to conform to the ECSS drafting rules (e.g. split of descriptive and normative text), and - Creation of two DRDs |  |
| ECSS-Q-ST-70-01C Rev.1 15 October 2025 | Second issue, Revision 1 Changes with respect to ECSS-Q-ST-70-01C (15 November 2008 are the following: Documentation milestones clarified: CRS due at SRR; C&CCP due at PDR; new C&CCV (verification report) required and delivered with a Cleanliness Declaration of Conformity at DRB. Structure aligned to ECSS drafting rules; clearer split of normative vs. informative text and clarified “shall/should” usage. Updated terms/abbreviations and added key definitions (e.g. vacuum cycle, visibly clean, witness sample). Documentation flow clarified: CRS at SRR, C&CCP at PDR, new C&CCV (verification report) + Cleanliness DoC at DRB. Materials/outgassing policy tightened (recent data, trusted test houses); criteria clarified incl. cryogenic/RT views. Annexes reworked; new normative DRD for C&CCV; editorial renumbering/reordering. NOTE: Due to the number of changes it was decided to create a new DOORS module that is not linked to the previous version. |

Table of contents

1 Scope	9

2 Normative references	10

3 Terms, definitions and abbreviated terms	11

3.1	Terms from other standards	11

3.2	Terms specific to the present standard	11

3.3	Abbreviated terms	15

3.4	Nomenclature	17

4 Principles	18

5 Requirements	19

5.1	Cleanliness and contamination control programme	19

5.1.1	General	19

5.1.2	Documentation	20

5.2	Phases	24

5.2.1	Design	24

5.2.2	MAIT	27

5.2.3	pre-launch and launch	29

5.2.4	Mission	31

5.3	Environments	31

5.3.1	Cleanrooms	31

5.3.2	Vacuum facilities	44

5.3.3	Other facilities	46

5.4	Activities	47

5.4.1	Cleaning of hardware	47

5.4.2	Cleanliness monitoring of space hardware	49

5.4.3	Packaging, containerization, transportation, storage	56

Annex A (normative) Cleanliness requirement specification (CRS) - DRD	59

A.1	DRD identification	59

A.1.1	Requirement identification and source document	59

A.1.2	Purpose and objective	59

A.2	Expected response	59

A.2.1	Scope and content	59

A.2.2	Special remarks	61

Annex B (normative) Cleanliness and contamination control plan (C&CCP) - DRD	62

B.1	DRD identification	62

B.1.1	Requirement identification and source document	62

B.1.2	Purpose and objective	62

B.2	Expected response	63

B.2.1	Scope and content	63

B.2.2	Special remarks	67

Annex C (normative) Cleanliness and contamination control verification report (C&CCV report) - DRD	68

C.1	DRD identification	68

C.1.1	Requirement identification and source documentation	68

C.1.2	Purpose and objectives	68

C.2	Expected response	68

C.2.1	Scope and content	68

C.2.2	Special remarks	70

Annex D (informative)  Cleanliness and contamination control process overview	71

Annex E (informative) Guidelines for general cleanliness and contamination control	72

E.1	General	72

E.2	Contamination attributes	72

E.2.1	Typical contaminants and their sources	72

E.2.2	Transport mechanisms	78

E.2.3	Main effects of contamination on space systems	80

Annex F (informative)  Cleanliness-oriented design	81

Annex G (informative)  Modelling guidelines	83

G.1	Introduction	83

G.2	Molecular Contamination	83

G.3	Particulate Contamination	84

G.4	DRR-Model Foundation	85

G.5	Input Parameters and Environmental Factors	85

G.6	Validation and Documentation	85

Annex H (informative)  Compatibility of various solvents with listed materials	86

Annex I (informative)  Particulate levels on surfaces	88

I.1	Standard method 1: Particle distribution	88

I.2	Standard method 2: Obscuration factor	88

I.2.1	Overview	88

I.2.2	Correlation for particles on surfaces	88

Annex J (informative)  Effects of humidity on materials and components	90

Annex K (informative)  Cleaning methods	91

K.1	Removal of particulate contamination	91

K.1.1	Overview	91

K.1.2	Vacuum cleaning and wiping	91

K.1.3	Gas jet cleaning	91

K.1.4	Tapes and films trapping	92

K.2	Removal of molecular contamination	92

K.2.1	Overview	92

K.2.2	Mechanical cleaning	92

K.2.3	Solvent and detergent cleaning	92

K.2.4	Films trapping	92

K.2.5	Gas jet cleaning	92

K.2.6	Plasma cleaning	93

K.2.7	Bakeout	93

K.2.8	Ultra­violet­ozone cleaning	93

Annex L (informative) Bakeout Analysis Methodology	94

L.1	Introduction	94

L.2	Methodology	94

Figures

Figure D-1 : Cleanliness and contamination control process overview	71

Tables

Table 5-1: Outgassing criteria for materials in the vicinity of sensitive items around RT	26

Table 5-2: Outgassing criteria for materials in the vicinity of sensitive items at temperature below RT	26

Table 5-3: Outgassing criteria for materials in the vicinity of cryogenic surfaces	26

Table 5-4: ISO Classes of air cleanliness by particle concentration.  (Table reproduced from ISO 14644-1:2015)	34

Table 5-5: Correlation airborne particles and PFO for cleanrooms	37

Table 5-6: Correlation airborne particles and PFO for TVAC facilities	46

Table H-1 : Examples of compatibility of various solvents with listed materials	86

Table I-1 : Correlation between ideal class of IEST-STD-CC1246E and obscuration factor	89

Table J-1 : Effect of humidity on materials and components	90

Introduction

The objective of this Standard is to ensure a successful mission by the definition of acceptable contamination levels for space system elements, their achievement, and maintenance, throughout

performance assessment versus contamination,

facilities and tools definition for contamination control and monitoring,

materials and processes selection, and

planning of activities.

The reader of this standard is reminded that a “NOTE” statement does not modify the contractual obligations stated in the requirements. The “NOTE” statement is descriptive, which means that if removed, contractual obligations remain the same.

# Scope

The purpose of this standard is to define:

The selection of contamination sensitive and contamination critical items, the definition of cleanliness requirements to satisfy the mission performance requirements and control the levels to be met by personnel, items, facilities and operations of space projects.

The management, including organization, reviews and audits, acceptance status and documentation control.

It covers design, development, production, testing, operation of space products, launch and mission.

In this standard are also guidelines given for identification of possible failures and malfunctions due to contamination and guidelines for achieving and maintaining the required cleanliness levels during ground activities, launch and mission.

This Standard applies to all types and combinations of projects, organizations and products, and during all the project phases, except manned missions.

It also applies to those ground systems that have a hardware interface to space systems, such as MGSE integration stands.

This Standard does not address magnetic, electrical or electrostatic cleanliness.

This Standard does not address completely biocontamination aspects. However, references to relevant ECSS standards are provided.

This standard may be tailored for the specific characteristic and constrains of a space project in conformance with ECSS-S-ST-00.

# Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this ECSS Standard. For dated references, subsequent amendments to, or revision of any of these publications do not apply, However, parties to agreements based on this ECSS Standard are encouraged to investigate the possibility of applying the more recent editions of the normative documents indicated below. For undated references, the latest edition of the publication referred to applies.

| ECSS-S-ST-00-01 | ECSS system — Glossary of terms |  |
| --- | --- | --- |
| ECSS-Q-ST-10-09 | Space product assurance – Nonconformance control system |  |
| ECSS-Q-ST-20 | Space product assurance — Quality assurance |
| ECSS-Q-ST-20-07 | Space product assurance — Quality assurance for test centres |
| ECSS-Q-ST-70 | Space product assurance — Materials, mechanical parts and processes |
| ECSS-Q-ST-70-02 | Space product assurance — Thermal vacuum outgassing test for the screening of space materials |
| ECSS-Q-ST-70-05 | Space product assurance – Detection of organic contamination of surfaces by infrared spectroscopy |
| ECSS-Q-ST-70-29 | Space product assurance — Determination of offgassing products from materials and assembled articles to used in manned space vehicle crew compartment |
| ECSS-Q-ST-70-50 | Space product assurance — Particle contamination monitoring for spacecraft systems and cleanrooms |
| ECSS-Q-TM-70-52 | Space product assurance — Kinetic outgassing of materials for space |
| ECSS-Q-ST-70-53 | Space product assurance — Material and hardware compatibility test for sterilization processes |
| ECSS-Q-ST-70-55 | Space product assurance — Microbial examination of flight hardware and cleanrooms |
| ECSS-Q-ST-70-58 | Space product assurance — Bioburden control of cleanrooms |
| ISO 14644 (latest edition) | Cleanrooms and associated controlled environments |
| IEST-STD-CC1246E | Product cleanliness levels and contamination control program |

# Terms, definitions and abbreviated terms

## Terms from other standards

For the purpose of this Standard, the terms and definitions from ECSS-ST-00-01 and ECSS-Q-ST-70 apply.

## Terms specific to the present standard

airborne particle

particle suspended in air

airborne particle cleanliness class

level of cleanliness specified by the maximum allowable number of particles per cubic metre of air

bakeout

activity of increasing the temperature of hardware to accelerate its outgassing rates with the intent of reducing the content of molecular contaminants within the hardware

Bakeout is usually performed in a vacuum environment, but can be done in a controlled atmosphere.

biocontamination

contamination of materials, devices, individuals, surfaces, liquids, gases or air with viable particles

blank test

verification run of an empty thermal vacuum test facility to confirm proper functionality, and to demonstrate its cleanliness status

cleaning

actions to reduce the contamination level

cleanliness (contamination) control

any organized action to control the level of contamination

cleanliness level

quantitative level of contamination

cleanliness verification

activity intended to verify that the actual cleanliness conditions of the space system, the cleanrooms or the vacuum chambers are in conformance with the applicable specifications and other cleanliness requirements

cleanroom

room in which the concentration of airborne particles is controlled, and which is constructed and used in a manner to minimize the introduction, generation and retention of particles inside the room, and in which other relevant parameters, e.g. temperature, humidity and pressure, are controlled as necessary

[ISO 14644-6]

clean zone

dedicated space in which the concentration of airborne particles is controlled, and which is constructed and used in a manner to minimize the introduction, generation and retention of particles inside the room, and in which other relevant parameters, e.g. temperature, humidity and pressure, are controlled as necessary

[ISO 14644-6]

This zone can be open or enclosed and can or cannot be located within a cleanroom.

contaminant

any unwanted molecular or particulate matter (including microbiological matter) on the surface or in the environment of interest, that can affect or degrade the relevant performance or life time

contaminate, to

act of introducing any contaminant

contamination budget

permissible contamination levels defined at different stages of the life of the instrument and satellite

contamination allocation

portion of the contamination budget assigned to individual elements, subsystems, or phases of the mission

contamination critical item

item that can contaminate a sensitive item by redistribution its particulate and molecular contamination

contamination potential

potential amount of contaminant in the source which can produce contamination

contamination sensitive item

item whose contamination may affect its performance or life time

controlled area

environmentally controlled area, operated as a cleanroom, with two pre­filter stages but without the final stage of HEPA (or better) filters used in cleanrooms

fibre

particle with a length to diameter, or width or thickness ratio of 10 or more

FTIR spectrometer

analyser (chemical identification) of organic and inorganic contamination using infrared wavelengths

HEPA particle filter

throwaway, extended­medium, dry type filter in a rigid frame that has a minimum particle­collection efficiency of 99,97 % (that is a maximum particle penetration of 0,03 %) for 0,3 m thermally generated DOP or specified alternative aerosol

induced contaminant environment

environment created by the presence of contaminating items

molecular contamination

airborne or surface contamination (vapour, gas, liquid, or solid) without observable dimensions (i.e. with dimensions at molecular level) including but not limited to water, organic, acid, base

monitoring

to perform routine, quantitative measurements of environmental parameters in and around cleanrooms, clean zones, and other clean areas, including contamination parameters

non­volatile residue (NVR)

quantity of residual soluble, suspended, and particulate matter remaining after the controlled evaporation of a volatile liquid at a specified temperature

obscuration factor (OF)

ratio of the projected area of all particles to the total surface area on which they rest

offgassing

evolution of gaseous products from a liquid or solid material into an atmosphere

outgassed quantity

total quantity of outgassed species expressed as a mass (e.g. gram or percent of the initial specimen) or as pressure volume (e.g. hPa  m3)

outgassing

evolution of gaseous species from a material, usually in vacuum

Outgassing also occurs in a higher­pressure environment.

particle

unit of matter with observable length, width and thickness

particle fallout

accumulated deposit of particulate matter on a surface by fallout

particle size

apparent maximum linear dimension of a particle in the plane of observation as observed with an optical microscope, or the equivalent diameter of a particle detected by automatic instrumentation

The equivalent diameter is the diameter of a reference sphere having known properties and producing the same response in the sensing instrument as the particle being measured.

particulate

of or relating to minute, separate particles

particulate contamination (PAC)

airborne or surface contamination relating to particles

plume

exhaust (molecules or particles) of thrusters and engines

pre-thermal vacuum test (pre-TV test)

dedicated, project-specific thermal vacuum test run comprising the entire test setup, minus the test article

precision cleaning

cleaning process used to achieve cleanliness levels more stringent than visibly clean

[ECSS-E-ST-35-06]

purging

supply of clean gas at a certain flow rate to protect the critical hardware from contamination

quartz crystal microbalance (QCM)

device for measuring small quantities of mass deposited on a quartz crystal using the properties of a crystal oscillator

ram direction

in the direction of velocity vector

ULPA particle filter

throwaway, extended­medium, dry­type filter in a rigid frame that has a minimum particle­collection efficiency of 99,999 % (that is, a maximum particle penetration of 0,001 %) for particles in the size range of 0,1 m to 0,2 m

vacuum cycle

entire sequence of a thermal vacuum test run , encompassing the complete process from the initial pump-down of the facility to the final venting to ambient conditions

Ambient conditions are typically defined by the specifications of the surrounding cleanroom or controlled zone.

venting

conveying unwanted gaseous products through an aperture

visibly clean

absence of surface contamination when examined utilizing a specific light source, light intensity, angle of incidence and viewing distance using normal or magnified vision

1	Inspection methods described in detail in ECSS-Q-ST-70-50, equivalence can also be found in IEST 1246E.

2	UV light can also be used in addition for the inspection.

visual inspection

act of examining an object under defined illumination and viewing conditions with normal or magnified vision

wake direction

direction opposite to the velocity vector

witness sample

sample used to collect contaminants during exposure, usually in an environmentally controlled area, and then analysed or measured

## Abbreviated terms

For the purpose of this Standard, the abbreviated terms from ECSS-S-ST-00-01 and the following apply:

| Abbreviation | Meaning |
| --- | --- |
| ACS | American Chemical Society |
| AIT | assembly, integration and testing |
| AIV | assembly, integration and verification |
| AO | atomic oxygen |
| BOL | beginning of life |
| CC | contamination control |
| C&CC | cleanliness and contamination control |
| C&CCP | cleanliness and contamination control plan |  |
| C&CCV | cleanliness and contamination verification report |
| CRS | cleanliness requirement specification |
| CVCM | collected volatile condensable material |
| DIW | deionised water |
| DML | declared materials list |
| DOP | dioctylphthalate |
| DRB | Delivery Review Board |
| DRD | Documents Requirements Definition |
| ECLS | environmental control and life support |
| EGSE | electrical ground support equipment |
| EMC | electromagnetic compatibility |
| EOL | end of life |
| EVA | extra vehicular activity |
| FTIR | Fourier transform infrared |
| GSE | ground support equipment |
| HEPA | high­efficiency particulate air filter |
| ICC | internal contamination control |
| IPA | isopropyl alcohol |
| IR | infrared |
| LEO | low Earth orbit |
| MAIT | manufacturing assembly integration and testing |
| MGSE | mechanical ground support equipment |
| MLI | multi layer insulation |
| MOC | molecular contamination |
| MRR | manufacturing readiness review |
| NVR | non­volatile residue |
| OF | obscuration factor |
| PAC | particulate contamination |
| PDR | product definition review |
| PFO | particle fallout |
| PMP | parts, materials and processes |
| ppm | parts per million (10-6) |
| pre-TV test | pre-thermal vacuum test |
| QCM | quartz crystal microbalance |
| RH | relative humidity |
| RT | room temperature |
| RML | recovered mass loss |
| SRR | system requirement review |
| TB | thermal balance |
| TML | total mass loss |
| TRR | test readiness review |
| TVAC | thermal vacuum |
| UV | ultra­violet |
| ULPA | ultra­low­particle air filter |
| VBQC | vacuum balance quartz crystal |
| VCM | volatile condensable material |

## Nomenclature

The following nomenclature applies throughout this document:

The word “shall” is used in this Standard to express requirements. All the requirements are expressed with the word “shall”.

The word “should” is used in this Standard to express recommendations. All the recommendations are expressed with the word “should”.

It is expected that, during tailoring, recommendations in this document are either converted into requirements or tailored out.

The words “may” and “need not” are used in this Standard to express positive and negative permissions, respectively. All the positive permissions are expressed with the word “may”. All the negative permissions are expressed with the words “need not”.

The word “can” is used in this Standard to express capabilities or possibilities, and therefore, if not accompanied by one of the previous words, it implies descriptive text.

In ECSS “may” and “can” have completely different meanings: “may” is normative (permission), and “can” is descriptive.

The present and past tenses are used in this Standard to express statements of fact, and therefore they imply descriptive text.

# Principles

The cleanliness and contamination control process is applied all along the project life cycle, from the definition of the C&CCP programme during the early phases (see clause 5.1) until its implementation during phases B, C, D, E and F (see clause 5.2) through the systematic verification of the cleanliness requirements baseline including: predictions through contamination modelling and the establishment of agreed procedures (see clause 5.3 and 5.4) for: environments control (see clause 5.3) packaging, containerization, transportation and storage of the space system.

Figure D-1 of Annex D gives an overview of an example of a cleanliness and contamination process.

# Requirements

## Cleanliness and contamination control programme

### General

ECSS-Q-ST-70-01_1650001

The supplier shall define and implement a cleanliness and contamination control programme for each level of configuration.

1	Surveys can also be made to determine the contamination control requirements, based on mission objectives and scenarios.

2	The objective of this programme is, starting from the mission performance requirements, to establish cleanliness and contamination levels to be achieved at different manufacturing, AIT and mission stages.

3	In general, the organization of regular workshops dedicated to cleanliness and contamination control for a specific programme is a good practice.

ECSS-Q-ST-70-01_1650002

The supplier shall establish measures for the coordination and resolution of cleanliness and contamination control issues among the parties involved in the project.

ECSS-Q-ST-70-01_1650003

The supplier shall appoint a C&CC responsible.

ECSS-Q-ST-70-01_1650004

The C&CC responsible shall define contamination sensitivity of the programme for each level of configuration.

Contamination sensitivity is driven by the design, program phases depending on mission, and involved technologies.

### Documentation

#### Contamination requirements specification (CRS)

ECSS-Q-ST-70-01_1650005

The supplier shall define and document the cleanliness levels applicable to the contamination critical and contamination sensitive items, and any other needs and provisions for contamination control, in a cleanliness requirement specification (CRS), in conformance with the DRD in Annex A.

“Cleanliness levels” are intended as:

The maximum level of contamination for acceptable mission performances until EOL, and

The maximum level of contamination applicable at the delivery of hardware from lower-tier suppliers.

ECSS-Q-ST-70-01_1650006

The CRS shall be defined as early as possible in the programme to ensure proper consideration during the design phase.

Cleanliness is of fundamental importance for the space system’s performance.

ECSS-Q-ST-70-01_1650007

The CRS shall be reviewed and approved by users and engineers from all applicable and impacted disciplines.

1	Users can be, for example, experimenters or scientists.

2	Engineers can be, for example, payload, instrument, optics, mechanisms, thermal, structural engineers.

ECSS-Q-ST-70-01_1650008

The CRS shall be provided by the SRR as part of the review data package.

ECSS-Q-ST-70-01_1650009

The C&CC responsible shall ensure compliance to both the ground and flight functional requirements and constraints of the project for each level of configuration.

Typical good practices e.g. Preparing a C&CC coordination plan, organising and supporting co-engineering activities starting before SRR, establishing and maintaining cleanliness and contamination control boards starting no later than PDR.

#### Cleanliness and contamination control plan (C&CCP)

##### General

ECSS-Q-ST-70-01_1650010

The supplier shall establish a Cleanliness and Contamination Control Plan (C&CCP) in conformance with the DRD outlined in Annex B (C&CCP DRD) and to be provided no later than the PDR.

##### Contamination budget

ECSS-Q-ST-70-01_1650011

As part of the C&CCP (see Annex B), a contamination budget and allocations shall be established.

ECSS-Q-ST-70-01_1650012

The contamination budgets shall allocate the applicable cleanliness levels to the activities along the lifetime, at different MAIT, launch and mission phases.

As an example for reusable vehicles mission phases are on ground, launch, in-flight and atmospheric entry and re-entry to a planet, dwarf planet or a natural satellite.

##### Contamination predictions

ECSS-Q-ST-70-01_1650013

As part of the C&CCP (see Annex B), prediction tables shall be established including MAIT, launch campaign and all mission phases for the following:

Particulate contamination;

Molecular contamination.

ECSS-Q-ST-70-01_1650014

The contamination predictions shall demonstrate the accumulation of molecular and particulate contaminants expected at MAIT, launch campaign and all mission phases.

ECSS-Q-ST-70-01_1650015

An analysis shall be performed to predict particulate and molecular deposition on sensitive items and documented in agreement with the customer, including:

In-flight molecular contamination (e.g. organic contaminants, water ice):

Kinetic parameters for in-flight molecular contamination modelling obtained as per ECSS-Q-TM-70-52.

Particulate and molecular contamination during launch campaign including lift off until separation.

Particulate and molecular contamination during atmospheric entry, re-entry and landing.

            - Particulate and molecular contamination on sensitive items during mission phases such as on orbit servicing, in orbit manufacturing, docking.

1	to item c: Examples of modelling techniques are given in Annex G.

2	to item c: Heritage data can be used for predictions of particulate and molecular deposition on sensitive items.

3	to item 1: Mathematical models and test methodology outlined in the ECSS-Q-TM-70-52 can be modified to increase accuracy by obtaining more parameters to increase the accuracy of the simulation.

4	to item 3: An example for reusable vehicles mission phases are on ground, launch, in-flight and atmospheric entry and re-entry to a planet, dwarf planet or a natural satellite.

ECSS-Q-ST-70-01_1650016

Contamination corrective, mitigation and preventative actions to reduce contamination shall be investigated and implemented including their predicted efficiencies:

If contamination predictions or when available actual measurements, result in a higher than the specified level,

If models used for analysis in the early project phases are preliminary and considered high risk by the customer.

The cleanliness verification activities can be specified in the C&CCP. The C&CCP as per Annex B can be tailored for test facilities with a particular view to their cleanliness and contamination control policy.

#### Cleanliness and contamination control verification report (C&CCV)

##### General

ECSS-Q-ST-70-01_1650017

          - The supplier shall establish a Cleanliness and Contamination Control Verification Report (C&CCV) in conformance with the DRD outlined in Annex C (C&CCV DRD), to be provided no later than the PDR.

##### Contamination verification and model validation

ECSS-Q-ST-70-01_1650018

The supplier shall verify the fulfilment of the applicable cleanliness levels.

1	Cleanliness requirements are the maximum level of contamination for acceptable mission performances until EOL, and/or the maximum level of contamination applicable at the delivery of hardware.

2	The demonstration as requirement 5.1.2.2.3b, with predictions updated using the actual measurements, is part of the verification.

ECSS-Q-ST-70-01_1650019

The cleanliness verification of all activities specified in the C&CCP shall be demonstrated in the C&CCV.

ECSS-Q-ST-70-01_1650020

On-ground cleanliness and contamination predictions shall be regularly updated with actual data from ground activities, including contamination monitoring measurements, the actual duration of activities, and other relevant factors affecting the predictions.

1	Updates of the predictions can be documented in the Cleanliness and Contamination Control Verification Report as per 5.1.2.3.2, or in a separate document referenced by the Contamination Control Verification Report.

2	Requirement 5.3.1.6a applies until the measurements from contamination monitoring are available.

3	The document delivery schedule can be established in agreement with the customer.

4	Some examples of “other relevant factors” are e.g., performed anomalies, deviations, bakeouts, hardware orientation, applied protections.

ECSS-Q-ST-70-01_1650021

Predictions for contamination sensitive and contamination critical items based on modelling analysis shall be verified and corrected against experimental data.

For in-flight molecular contamination via dedicated or as a part of planned testing activities during MAIT, or in-orbit measurements;

For particulate and molecular contamination during launch via dedicated tests, in situ data provided by the launch provider or heritage data.

##### Cleanliness declaration of conformity

ECSS-Q-ST-70-01_1650022

A cleanliness declaration of conformity shall be delivered for space hardware at DRB together with the C&CCV.

## Phases

### Design

#### General design aspects

ECSS-Q-ST-70-01_1650023

The level of sensitivity to contamination shall be one of the drivers in the initial design.

ECSS-Q-ST-70-01_1650024

The design shall be cleanliness oriented.

1	A way to implement a cleanliness-oriented design is given in Annex F.

2	Such design can contribute to achieve the contamination levels defined by the CRS on ground as well during the launch and mission.

3	A way to achieve the target contamination levels can be found in Annex E.

ECSS-Q-ST-70-01_1650025

When the design baseline is incompatible with cleanliness requirements, the design changes shall be identified, and corrective actions be taken in close cooperation with all levels involved.

#### Materials selection

ECSS-Q-ST-70-01_1650026

When the offgassing effect of a material is a selection criterion, the supplier shall apply ECSS-Q-ST-70-29.

For modelling the molecular contamination during on-ground activities, when outgassing data are too conservative, offgassing data are advisable.

ECSS-Q-ST-70-01_1650027

For the particulate contamination monitoring, inspection and quantification the supplier shall apply ECSS-Q-ST-70-50.

ECSS-Q-ST-70-01_1650028

When the microbiological contamination effect is a selection criterion, the supplier shall apply ECSS-Q-ST-70-55.

ECSS-Q-ST-70-01_1650029

When sterilization and material compatibility are selection criteria, the supplier shall apply ECSS-Q-ST-70-53.

ECSS-Q-ST-70-01_1650030

The outgassing screening test of materials shall be performed in accordance with ECSS-Q-ST-70-02 including the following conditions:

outgassing data older than 10 years is not admissible for material selection;

screening outgassing tests is performed by a trusted test house.

1	MODESA outgassing database can be referred to obtain updated list of the trusted test houses and reach the outgassing data.

2	Trusted test houses are those that can provide a Certificate of Conformity in compliance with ECSS-Q-ST-70-02 standards.

ECSS-Q-ST-70-01_1650031

Outgassing requirements shall be based on the quantity of material concerned, and the specific environmental conditions.

Specific environmental conditions can be available volumes and temperatures.

ECSS-Q-ST-70-01_1650032

The outgassing criteria for materials within the direct and indirect view of sensitive items shall conform to following tables based on defined temperatures:

at RT to Table 5-1;

at temperature below RT to Table 5-2.

1	“Indirect view” in a vacuum environment refers not only to the immediate surroundings but also to the consideration of reflections from other volumes and surfaces.

2	For contamination sensitive items more stringent requirements can be applied based on analysis and mission requirements.

ECSS-Q-ST-70-01_1650033

The outgassing criteria for materials in the view of cryogenic surfaces shall conform to Table 5-3.

For materials in the view of cryogenic surfaces, more stringent requirements can be applied based on analysis and mission requirements.

ECSS-Q-ST-70-01_1650034

Volatile metals shall not be used.

1	This is especially the case when the temperatures are above room temperatures.

2	Some metals such as cadmium and zinc have high vapour pressures and deposit metallic films can occur on adjacent surfaces.

ECSS-Q-ST-70-01_1650035

Table 5-1: Outgassing criteria for materials in the vicinity of sensitive items around RT

| Mass of material concerned (g) | CVCM (%) | RML (%) |
| --- | --- | --- |
| >100 | ≤ 0,01 | ≤ 1 |
| 10 - 100 | ≤ 0,05 | ≤ 1 |
| < 10 | ≤ 0,1 | ≤ 1 |

ECSS-Q-ST-70-01_1650036

Table 5-2: Outgassing criteria for materials in the vicinity of sensitive items at temperature below RT

| Mass of material concerned (g) | CVCM (%) | RML (%) |
| --- | --- | --- |
| >100 | ≤ 0,01 | ≤ 0,1 |
| 10 - 100 | ≤ 0,05 | ≤ 1 |
| < 10 | ≤ 0,1 | ≤ 1 |

ECSS-Q-ST-70-01_1650037

Table 5-3: Outgassing criteria for materials in the vicinity of cryogenic surfaces

| Mass of material concerned (g) | CVCM (%) | TML (%) |
| --- | --- | --- |
| >100 | ≤ 0,01 | ≤ 0,1 |
| 10 - 100 | ≤ 0,05 | ≤ 1 |
| < 10 | ≤ 0,1 | ≤ 1 |

### MAIT

#### Manufacturing

ECSS-Q-ST-70-01_1650038

Personnel involved in the manufacturing of sensitive items shall be trained with respect to the cleanliness control policy.

ECSS-Q-ST-70-01_1650039

All elements manufactured in non­controlled areas or under non­clean conditions shall be the object of a cleaning process until the cleanliness requirements are met, before they are packaged for delivery.

ECSS-Q-ST-70-01_1650040

Cleaning and packaging operations for all elements shall be processed according to procedures approved by the customer for the specific application/product.

ECSS-Q-ST-70-01_1650041

Elements that can be cleaned after manufacturing shall be cleaned till the cleanliness requirements are met.

ECSS-Q-ST-70-01_1650042

For elements that cannot be cleaned after manufacturing, then manufacturing and assembling areas shall meet the cleanliness level requirements specification.

ECSS-Q-ST-70-01_1650043

The cleanliness conformity of the facilities shall be verified:

for manufacturing at MRR;

for integration at IRR;

for testing at TRR.

ECSS-Q-ST-70-01_1650044

An audit of the manufacturing facilities shall be performed according to ECSS-Q-ST-10 clause 5.2.3 criteria.

#### Assembly and Integration

ECSS-Q-ST-70-01_1650045

Involved personnel shall be trained with respect to the cleanliness policy.

ECSS-Q-ST-70-01_1650046

Contamination critical and contamination sensitive elements shall only be exposed during a predefined MAIT activity.

1	Exposure of sensitive and critical elements cannot be avoided e.g., during optical calibration or alignment.

2	Depending on the type of contamination, critical elements can be exposed to specific environments.

ECSS-Q-ST-70-01_1650047

When an exposure of contamination critical and contamination sensitive elements to an environmentally controlled area cannot be avoided, the exposure time and conditions shall be recorded.

ECSS-Q-ST-70-01_1650048

A set of assembly tools and equipment for assembly and integration shall be used and maintained in clean conditions.

ECSS-Q-ST-70-01_1650049

Procedures for assembly and integration shall be established for contamination critical item assembly.

ECSS-Q-ST-70-01_1650050

For the selection of the cleanroom, the allocated contamination budget and the duration of the integration shall be known.

The correlation between the airborne contamination and the particle fallout for normal cleanrooms is basically known (see clause 5.3.1), and so a rough estimate can be made of the type of cleanroom required. A practical contamination level for the cleanroom can be measured with representative activities and a representative number of operators. The expected contamination levels depend on the type of protection applied to critical hardware (e.g. covers, shields and purging).

ECSS-Q-ST-70-01_1650051

The conformity of the facilities shall be verified during MRR or TRR.

ECSS-Q-ST-70-01_1650052

An audit of the integration facilities shall be performed according to ECSS-Q-ST-10 clause 5.2.3 criteria.

#### Testing

ECSS-Q-ST-70-01_1650053

Involved personnel shall be trained with respect to the cleanliness policy.

ECSS-Q-ST-70-01_1650054

For test centres, ECSS-Q-ST-20-07 shall apply.

ECSS-Q-ST-70-01_1650055

The conformity of the facilities shall be verified during TRR.

### pre-launch and launch

#### General

ECSS-Q-ST-70-01_1650056

End to end launch campaign shall be designed and planned in accordance with the CRS, controlled by the C&CCP and verified in the C&CCV.

Launch campaign is intended to cover the activities from arrival to the launch site until the S/C separation including contributions from the separation.

ECSS-Q-ST-70-01_1650057

A dedicated C&CCP shall be provided for the launch campaign by the launch provider.

ECSS-Q-ST-70-01_1650058

Personnel involved in pre-launch activities shall be trained with respect to the cleanliness policy of

the launch provider;

spacecraft.

ECSS-Q-ST-70-01_1650059

The space system shall be shipped to the launch base under clean conditions as defined in the CRS, controlled by the C&CCP and verified in the C&CCV.

ECSS-Q-ST-70-01_1650060

The potential contamination during launch preparation shall be controlled.

This can be done through the C&CCP or through specific launch base procedures approved by the project.

ECSS-Q-ST-70-01_1650061

Contamination during launch shall be controlled through specific preventive actions and design provisions.

Preventive actions can consist of cleaning and purging of the fairing. Specific design provisions can consist of shields controlling the depressurization.

ECSS-Q-ST-70-01_1650062

Contamination control activities for launch shall include:

if available, providing heritage data from previous launches;

cleanliness verification of fairing prior to encapsulation at the latest possible moment;

verification of all specific design provisions including but not limited to purging, shields, depressurization.

#### Specific design provisions

ECSS-Q-ST-70-01_1650063

Launcher parts shall be clean to comply with CRS, and in line with the C&CCV of the space system.

Launcher parts can be fairings and mechanical systems for double or multiple launches.

ECSS-Q-ST-70-01_1650064

The materials of the launch hardware in the direct and indirect view of the space system shall comply with CRS, and in line with the C&CCV of the space system itself.

ECSS-Q-ST-70-01_1650065

The environment in which the spacecraft is put inside the fairing shall be compatible with the spacecraft characteristics.

1	Specific spacecraft design provisions can be protection mechanisms used to limit the launch contaminants, especially the “unknown” figure of particle transfer during launch.

2	A second design aspect is the location of the contamination sensitive items with respect to the position of thrusters and of pyrotechnics or other contamination sources.

3	The reflection by atmospheric molecules (i.e. atmospheric scattering) or by outgassing molecules (i.e. self­scattering) can take place and some form of modelling is of interest.

### Mission

ECSS-Q-ST-70-01_1650066

Spacecraft contamination control during mission shall be done through preventive actions, specific design provisions and operations.

1	Preventive actions include materials selection (see clause 5.5.3 in ECSS-Q-ST-70-02), bakeout (see clause 5.4.2.4) and purging (see clause 5.4.2.5).

2	Specific design provisions include the implementation of heaters for decontamination of sensitive surfaces, of shutters and baffles.

3	Operations include shielding during dumping, thrusters firing or venting, decontamination of sensitive surfaces through exposure to the Sun.

ECSS-Q-ST-70-01_1650067

Fluids that can emerge to the exterior by leakage or intentional use of valves shall be considered in the design and operational requirements of system and equipment hardware.

ECSS-Q-ST-70-01_1650068

A specific analysis shall be performed to ensure an optimum level of detection, location and isolation techniques.

These fluids are originating from thermal, environmental or life support systems or subsystems or released due to crew activities (nutrients, wastes), during maintenance and repair and from experiments or payloads as well as the propellant systems.

## Environments

### Cleanrooms

#### Design of cleanroom: shell, entrances and anterooms

ECSS-Q-ST-70-01_1650069

Cleanroom shell, floors, walls and ceiling shall be low shedding and the finish readily cleanable.

ECSS-Q-ST-70-01_1650070

The covering floor shall consist of one piece or, if this is not feasible, have a minimum number of joints.

ECSS-Q-ST-70-01_1650071

The floor shall be resistant to withstand wear by personnel and operations within the room.

ECSS-Q-ST-70-01_1650072

The room shall be designed such that only one door or entrance can be opened at one time, except in case of emergency.

ECSS-Q-ST-70-01_1650073

Entrances shall provide an air lock to allow a maintained pressurisation of the area.

ECSS-Q-ST-70-01_1650074

Anterooms shall be equipped for the changing of clothes and the storage of clothing, personal belongings and cleaning equipment.

ECSS-Q-ST-70-01_1650075

Cleanrooms shall comply with ISO 14644-4:2022 requirements.

#### Air supply

ECSS-Q-ST-70-01_1650076

Air supply and filtration equipment shall have the capacity to filter all new and recirculated air entering the room to guarantee the defined ISO class.

ECSS-Q-ST-70-01_1650077

Air conditioning equipment for prefiltering (particular and molecular), cooling, heating, humidification and dehumidification of the cleanroom air supply shall be supplied to guarantee the environmental conditions.

See clauses 5.3.1.8, 5.3.1.9, 5.3.1.10 and 5.3.1.11.

ECSS-Q-ST-70-01_1650078

In laminar flow cleanrooms, the air flow velocity through the cross section of the room shall be maintained at 27 m/min with a uniformity within  20 % throughout the undisturbed room.

ECSS-Q-ST-70-01_1650079

Airflow patterns shall be uniform with minimum turbulence.

#### Filters

ECSS-Q-ST-70-01_1650080

In laminar flow cleanrooms, (HEPA) filters shall cover either one entire wall or the entire ceiling, except when diffusion ceiling or wall systems are used or when built­in benches are included in the incoming air end of the room.

ECSS-Q-ST-70-01_1650081

Monitoring shall be done and any work with highly sensitive equipment not be performed before the defined ISO class for the hardware has been reached. as specified in the C&CCP for the following situations:

after the installation of new filters,

after “at rest“ period,

after stand by period.

Due to the transitory pressure gradients, contamination previously trapped by HEPA filters, together with a reduction in the operating life of the filters themselves can be released.

ECSS-Q-ST-70-01_1650082

The air flow inside cleanrooms and independent HEPA filtering systems shall be maintained during “at rest” periods, except for the maintenance operations.

1	For example, during filters replacement.

2	Independent HEPA filtering systems can be like those used for the laminar flow tents and benches.

3	This is to avoid the risk of redistribution of particles at restart of the flow.

4	Exception can be made for independent HEPA filtering systems that can work with a reduced air flow rate during stand­by periods.

ECSS-Q-ST-70-01_1650083

In cases where a uniform and controlled molecular environment is required, the filtering system shall be equipped with additional molecular absorption filters positioned before the HEPA filters.

Filtering systems can be equipped with, for example, charcoal and zeolites.

ECSS-Q-ST-70-01_1650084

When molecular absorption filters are used, the initial charge shall be assessed on installation and analysed regularly.

It can be useful to evaluate the charge in contaminants of the filtering system which can release its charge in contaminants trapped. in order to be able to monitor the evolution and when a failure occurs.

#### Particle levels and cleanroom classification

ECSS-Q-ST-70-01_1650085

Any airborne controlled environment shall be classified according to the Table 5-4 reproduced from ISO 14644-1:2015.

1	Suitability of the cleanroom class can only be assessed once the specified obscuration factors for critical surfaces are determined.

2	Particles of 5 μm are more critical than smaller particles, as fallout is mainly influenced by particles of this size or larger.

3	The 5 μm particle size is often used as a benchmark because particles larger than 5 μm are particularly detrimental to optical surfaces, while particles in the 10 μm to 40 μm range are generally more harmful to bearings and gears. However, more stringent criteria can be applied in certain situations to ensure even higher levels of cleanliness and protection for sensitive components.

4	The cleanroom classification can be determined based on at least two particle sizes (e.g., between 0,1 µm and 5 µm) relevant to the activities and sensitive items present in the cleanroom.

ECSS-Q-ST-70-01_1650086

Table 5-4: ISO Classes of air cleanliness by particle concentration. 
(Table reproduced from ISO 14644-1:2015)

| ISO Class number (N) | Maximum allowable concentrations (particles/m3) for particles equal to and greater than the considered sizes, shown below a |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0,1 µm | 0,2 µm | 0,3 µm | 0,5 µm | 1 µm | 5 µm |
| 1 | 10b | d | d | d | d | e |
| 2 | 100 | 24b | 10b | d | d | e |
| 3 | 1 000 | 237 | 102 | 35b | d | e |
| 4 | 10 000 | 2 370 | 1 020 | 352 | 83b | e |
| 5 | 100 000 | 23 700 | 10 200 | 3 520 | 832 | d, e, f |
| 6 | 1 000 000 | 237 000 | 102 000 | 35 200 | 8 320 | 293 |
| 7 | c | c | c | 352 000 | 83 200 | 2 930 |
| 8 | c | c | c | 3 520 000 | 832 000 | 29 300 |
| 9g | c | c | c | 35 200 000 | 8 320 000 | 293 000 |
| a All concentrations in the table are cumulative, e.g. for ISO Class 5, the 10 200 particles shown at 0,3 µm include all particles equal to and greater than this size. b These concentrations will lead to large air sample volumes for classification. Sequential sampling procedure may be applied; see ISO 14644:1 Annex D c Concentration limits are not applicable in this region of the table due to very high particle concentration. d Sampling and statistical limitations for particles in low concentrations make classification inappropriate. e Sample collection limitations for both particles in low concentrations and sizes greater than 1 μm make classification at this particle size inappropriate, due to potential particle losses in the sampling system. f In order to specify this particle size in association with ISO Class 5, the macroparticle descriptor M may be adapted and used in conjunction with at least one other particle size. (See ISO 14644:1 C.7.) g This class is only applicable for the in-operation state. |  |  |  |  |  |  |

#### Monitoring of cleanroom air

ECSS-Q-ST-70-01_1650087

The cleanroom air shall be monitored with dust counters.

ECSS-Q-ST-70-01_1650088

Accuracy and repeatability of instrumentation shall be demonstrated.

ECSS-Q-ST-70-01_1650089

Particle counts shall be acquired continuously for the monitoring of the cleanroom itself.

ECSS-Q-ST-70-01_1650090

A minimum of two particles counters shall be installed, one close to the air inlet and one or more according to the surface extent of the cleanroom by using the following law:

N = log10 (S)

where N is the number of particle counters and S is the surface of the cleanroom in m².

The purpose of the location close to the air inlet is to check the quality of the in-coming air.

ECSS-Q-ST-70-01_1650091

For any contamination sensitive hardware and contamination critical operation, sampling frequency and locations shall be defined in the C&CCP.

ECSS-Q-ST-70-01_1650092

Monitoring techniques and routines shall be established to meet the requirements of a specific category of cleanroom or clean workstation.

ECSS-Q-ST-70-01_1650093

Sampling air volume for the cleanroom classification shall be established on the basis of the ISO 14644-1:2015, Annex B.

ECSS-Q-ST-70-01_1650094

Compliance with particles concentration limits shall be done with a frequency as specified per ISO 14644-2:2015, Clause 4.2.1.

ECSS-Q-ST-70-01_1650095

Determining the extent to which particles are deposited on surfaces shall be achieved.

This can be done through the exposure of test surfaces or samples to an environmentally controlled area and counting the settled particles by appropriate methods.

ECSS-Q-ST-70-01_1650096

Air monitoring of class ISO 8 or better shall be achieved by means of light scattering equipment.

ECSS-Q-ST-70-01_1650097

Tests shall be performed to determine if leaks exceed the specified limits, according to the filter characteristics:

in the filter media themselves,

in the bond between filter media and the interior of the filter frame,

between filter frame gasket and filter bank supporting frames,

between supporting frames and walls or ceilings.

ECSS-Q-ST-70-01_1650098

The cleanrooms shall have a monitoring function of the contamination levels and the environmental parameters.

ECSS-Q-ST-70-01_1650099

The cleanroom shall have an alarm function activated when warning levels are exceeded.

1	Those warning levels are usually defined well below the out of specification limits in order to prevent their exceedance.

2	Environmental parameters are temperature, relative humidity and differential pressure.

ECSS-Q-ST-70-01_1650100

Planned corrective actions shall be initiated to re­establish the nominal conditions in the shortest possible time and to prevent recurrence.

#### Surface particulate levels

ECSS-Q-ST-70-01_1650101

Predictions shall assume particulate contamination deposition in cleanroom environments based on the correlation between airborne particles and PFO as established at rest values in Table 5-5, until actual PFO measurements are available.

1	PFO calculation from Table 5-5 is not a conservative calculation and for this reason confirmation by PFO measurements as per this requirement is needed.

2	The PFO calculation in Table 5-5 is based on the particle size distribution as defined for the cleanroom classes in the ISO 14644-1:2015 and does not consider bigger particles or fibres.

3	At-rest values need to be adjusted with the predicted in-operation durations of the mission.

ECSS-Q-ST-70-01_1650102

The budget, during the different project phases, shall be consolidated with in-situ measurements.

ECSS-Q-ST-70-01_1650103

PFO shall be monitored by witness plate exposure in an environmentally controlled area and measured according to ECSS-Q-ST-70-50.

ECSS-Q-ST-70-01_1650104

PFO levels shall be measured during a cleanroom acceptance, or after rework.

ECSS-Q-ST-70-01_1650105

Exposition duration shall be sufficient to ensure that the measured values are exceeding the method uncertainties.

ECSS-Q-ST-70-01_1650106

Measured PFO rates of a cleanroom at-rest shall be compliant with Table 5-5.

ECSS-Q-ST-70-01_1650107

Location and number of witness plates shall be selected in order to measure particles in significant points of the environment.

ECSS-Q-ST-70-01_1650108

When MAIT activities are ongoing, local particulate contamination monitoring shall be implemented as per clause5.4.2.2 and adapted to hardware configuration and sensitivity.

1	Particulate deposit are strongly correlated to the level and type of activities in close vicinity, and therefore localized effects are expected

2	The ISO class of the cleanroom is guaranteed through airborne particle measurements (see clause 5.3.1.5).

ECSS-Q-ST-70-01_1650109

Table 5-5: Correlation airborne particles and PFO for cleanrooms

| ISO class | PFO (mm2/m2/24 h) |
| --- | --- |
| 5 | 2,0 |
| 6 | 10 |
| 7 | 52 |
| 8 | 275 |
| NOTE The data contained in this table are based on several measurements performed in different cleanrooms. They are represented by this approximate law: PFO = 0,069 10(0,72M-2.16) where M is the ISO class (e.g. ISO class 5) |  |

#### Surface molecular levels

ECSS-Q-ST-70-01_1650110

Predictions shall assume a molecular contamination deposition of 0,5 × 10‐7 g/cm² in cleanroom environments over a continuous one-week period, until actual MOC measurements are available.

Predictions can also be based on reliable historical molecular contamination deposition records and proven trends for a given environment.

ECSS-Q-ST-70-01_1650111

Molecular contamination in controlled environments shall not exceed 2  10-7 g/cm2 during a continuous period of one month, independently of the cleanroom classes.

1	In case of contamination sensitive equipment, a lower level can be required, and more stringent requirements can be set in agreement with the customer based on the contamination budget including exposure time.

2	Cleanliness verification frequency can be adjusted depending on the customer needs.

ECSS-Q-ST-70-01_1650112

Molecular contamination measurements shall be monitored, measured, and reported according to ECSS-Q-ST-70-05, using at least two sets of witness plates, each set comprising two plates, and minimum in two different locations.

1	The number of sets of witnesses can be increased depending on the size of the cleanroom.

2	A technical justification supported by actual measurements can be used to reduce the number of exposed witness samples within the specific cleanroom configuration in agreement with the customer.

ECSS-Q-ST-70-01_1650113

One of the two witness of each set, namely step witness plate, shall be analysed at least once per month.

Measurement frequency can be adjusted depending on the customer needs.

ECSS-Q-ST-70-01_1650114

The second witness plate of each set, namely cumulative witness plate, shall be analyzed:

not longer than 6 months after the previous cumulative witness measurement, and

prior to cleanroom maintenance, cleaning and rework or other major activities performed in the cleanroom.

to item 1: Measurement frequency can be adjusted depending on the customer needs.

ECSS-Q-ST-70-01_1650115

MOC measurement locations shall be selected to measure molecular contamination in significant points of the environment with at least one representative of the cleanroom at rest.

#### Temperature control

ECSS-Q-ST-70-01_1650116

Cleanroom temperature shall be maintained and continuously monitored at nominally 22 C  3 C .

More stringent temperature variations conditions can be imposed in case of critical operations.

ECSS-Q-ST-70-01_1650117

The temperature distribution inside a cleanroom shall be controlled at representative locations for the hardware items.

1	In order to ensure that a nominal temperature is achieved throughout the room. Automatic devices can be used for temperature monitoring.

2	If items being worked on are extremely sensitive to temperature changes, automatic devices with a warning system that comes into operation when a temperature change occurs can be used.

#### Pressure control

ECSS-Q-ST-70-01_1650118

Segregation of the controlled environment shall be specified according to ISO 14644-4:2022 Section B.2.2 Segregation.

ECSS-Q-ST-70-01_1650119

In case a physical barrier segregation is specified the pressure differential between adjacent areas shall at least be 5 Pa, where

a positive pressure differential is maintained between the cleanroom and the personnel lock, and

the pressure decreases successively between the cleanroom, personnel lock and the surroundings.

ECSS-Q-ST-70-01_1650120

The effectiveness of the utilised segregation concept shall be demonstrated as per utilised methodology defined in the ISO 14644-3:2019.

1	The effectiveness of the physical barrier concept can be demonstrated by undertaking a containment leak test and experimental airflow visualization as described in ISO 14644-3:2019.

2	Operational limitations that affect segregation can occur during certain activities, such as logistics in and out of the airlock. All impacted cleanroom users need to be informed in advance to implement appropriate contamination protection measures.

3	The effectiveness can be reassessed with an agreement to the customer.

ECSS-Q-ST-70-01_1650121

Pressure in all areas shall be monitored continuously.

In order to take timely corrective actions in case of a pressure drop.

#### Humidity control

ECSS-Q-ST-70-01_1650122

The relative humidity shall be maintained at (55  10) % for general applications and be monitored continuously.

Relative humidity environments can be adjusted due to specific hardware constraints (see Annex J).

#### Maintenance and cleaning

ECSS-Q-ST-70-01_1650123

All maintenance and cleaning activities shall be reported in a logbook.

ECSS-Q-ST-70-01_1650124

A maintenance and cleaning procedure or document shall be available, along with a planning.

ECSS-Q-ST-70-01_1650125

Maintenance shall comprise regular inspections of the cleanroom, its control facilities and its operating equipment, including calibration of all inspection and monitoring devices as specified in ECSS-Q-ST-20.

ECSS-Q-ST-70-01_1650126

Inspections of the cleanroom shall be performed at specified frequency, depending on the ISO class.

Those inspections assess the quality of the clean facility and describe any contamination production or events that are detrimental to the cleanroom cleanliness (e.g. repairs, system modifications, replacements, filter resistance measurements, leak checks or air speed measurements).

ECSS-Q-ST-70-01_1650127

The frequency of inspections and cleaning processes for a cleanroom shall be optimized.

The inspections and cleaning can themselves be the source of contamination.

ECSS-Q-ST-70-01_1650128

Data shall be recorded in a logbook.

ECSS-Q-ST-70-01_1650129

Regular cleaning shall be performed depending on the ISO class.

ECSS-Q-ST-70-01_1650130

Procedures shall cover the cleaning of the cleanroom including personnel air lock; equipment air lock, walls, floors, furniture, crane lifting devices and GSE.

ECSS-Q-ST-70-01_1650131

The cleanliness after the cleaning operations shall be verified by inspection by means of UV or high intensity white light.

ECSS-Q-ST-70-01_1650132

Any personnel involved in cleaning operations shall be trained and informed about the criticality of a cleaning operation within a cleanroom.

ECSS-Q-ST-70-01_1650133

Cleaning tools, solvents and gases that are used for cleaning purposes shall be chosen not to have a detrimental effect on the hardware within the cleanroom.

Cleanroom air of better than class ISO 8 is transported in a close loop. Since only a limited percentage of fresh air is fed to the loop, excessive use of solvents offgassing into the air, even if not flammable or toxic, can cause health problems.

ECSS-Q-ST-70-01_1650134

When the level of contamination exceeds the cleanliness requirements specification, corrective actions shall be taken.

The decision as to whether or not to clean depends on the integration flow of the unit within the cleanroom.

#### Access control requirements

ECSS-Q-ST-70-01_1650135

An access control system shall be available independently for cleanrooms, storage area and equipment airlock.

ECSS-Q-ST-70-01_1650136

The access to the areas shall be controlled by a permanently operating access control or door lock system.

ECSS-Q-ST-70-01_1650137

Only authorized personnel shall have access to the cleanroom.

Access control areas have a security lock at the entrance.

ECSS-Q-ST-70-01_1650138

Visitors and personnel without a work order shall not be allowed to enter the cleanroom.

ECSS-Q-ST-70-01_1650139

Visitors working in the cleanroom, shall:

wear specified cleanroom clothing,

be identifiable,

be instructed about the behaviour in a cleanroom.

ECSS-Q-ST-70-01_1650140

Racks or cabinets for street clothing shall be separated from those used for cleanroom clothing.

ECSS-Q-ST-70-01_1650141

Barriers or similar means separating clean and not clean zones inside the airlock shall be placed.

Similar means can be tacky mats.

ECSS-Q-ST-70-01_1650142

Lint­free clothing shall be available and worn by all personnel within cleanroom area.

ECSS-Q-ST-70-01_1650143

Head covers or other garments shall be used as required to trap loose particles of hair, including but not only beard, moustache or skin flakes.

ECSS-Q-ST-70-01_1650144

Gloves, approved finger cots, tweezers or clean handling methods and equipment shall be used while working with or handling sensitive parts.

This is to avoid contamination of those parts by loose skin or natural skin oils.

ECSS-Q-ST-70-01_1650145

All equipment shall be cleaned by dusting, vacuum suction, washing, or other means suited to the equipment involved before being brought into the area.

ECSS-Q-ST-70-01_1650146

Exhaust systems for grinding, welding or soldering, machining or related operations shall be installed.

ECSS-Q-ST-70-01_1650147

Actions related to equipment items with cooling fans shall be identified and mitigated in order to avoid contamination of critical hardware.

Equipment items with cooling fans are potential contamination sources.

ECSS-Q-ST-70-01_1650148

Personnel shall be instructed about the behaviour in a cleanroom.

ECSS-Q-ST-70-01_1650149

Personnel movements to and from the cleanroom shall be kept to a minimum.

ECSS-Q-ST-70-01_1650150

Smoking, eating and drinking shall not be permitted in the cleanroom, including the entering areas and air locks.

ECSS-Q-ST-70-01_1650151

Local cleanroom instructions shall specify the amount of protective clothing to be worn to reduce the contaminant transfer.

ECSS-Q-ST-70-01_1650152

If air showers are used, only suitably clothed personnel shall be allowed to enter.

ECSS-Q-ST-70-01_1650153

Paper, pencils or erasers shall be kept outside the clean facilities and only special non­shedding papers and ball­points be used.

ECSS-Q-ST-70-01_1650154

Cosmetics and medicaments that can produce contamination shall not be used by any personnel.

In particular, eye make­up, rouge, face powder, hair spray and fingernail polish.

ECSS-Q-ST-70-01_1650155

Fingernail polish shall not be permitted in the area.

ECSS-Q-ST-70-01_1650156

Before entering a cleanroom, hand lotions, creams or soap containing lanolin to tighten skin particles shall be used.

ECSS-Q-ST-70-01_1650157

Contact of hands with solvents shall be avoided.

Many solvents remove natural oils and cause excessive skin peeling or flaking.

### Vacuum facilities

ECSS-Q-ST-70-01_1650158

Procedures shall be available for:

The cleaning of the test facilities.

The pump­down and recovery sequences with respect to contamination redistribution.

The regeneration of sorption pumps.

The cleaning of cold traps.

Verifying the cleanliness of the cold traps.

Contamination monitoring strategy:

QCM locations in the facility and operating procedures,

MOC witness locations in the facility and operating procedures,

PFO witness locations in the facility and operating procedures.

1	to item 1: Handling of solvents and running a bakeout.

2	to item 2: A good solution, for chamber repressurization, is to add an HEPA filter to the repressurization piping and to collect the air for repressurization in a clean area (preferably ISO class 5).

3	to item 3: Sorption pumps can be e.g. cryopumps, zeolites, or charcoal.

ECSS-Q-ST-70-01_1650159

During testing in a vacuum facility, the test item shall not exceed the contamination limits specified by the test facility including the following:

Threshold PFO values defined by the test facility in agreement with the customer.

Threshold MOC values and measurement techniques defined by the test facility in agreement with the customer.

ECSS-Q-ST-70-01_1650160

A minimum vacuum level of 1  10-5 hPa shall be reached for TVAC testing.

ECSS-Q-ST-70-01_1650161

An approved declared material list (DML) of the hardware under test, including the test adapter and all connections shall be provided.

Including, for example mechanical and electrical connexions.

ECSS-Q-ST-70-01_1650162

The cleanliness verification of the blank TVAC chamber shall be demonstrated by either:

a pre-TV to prove the cleanliness of the facility with a representative temperature and a duration of a single cycle of the TVAC test not less than 24 hours, after reaching the required in vacuum temperature and pressure for the item under test, or

as an alternative to performing a dedicated pre-TV, and subject to customer approval, using documented test results obtained immediately prior to the TVAC test, provided these tests meet or exceed the TVAC test in terms of temperature, duration, sequence, and contamination sensitivity.

ECSS-Q-ST-70-01_1650163

During the pre-TV test, pump down and repressurization sequences shall be the same to the actual test.

ECSS-Q-ST-70-01_1650164

During the pre-TV test, test equipment, support equipment and cabling shall be included in the facility.

ECSS-Q-ST-70-01_1650165

Thermal vacuum test facilities shall demonstrate meeting requirement for a total molecular contamination below 1,0 x 10-7 g/cm² per vacuum cycle during pre-TV test.

1	More stringent requirements can be defined on the specific project needs.

2	Less stringent requirement can be defined for non-contamination sensitive missions e.g. some telecommunication missions in agreement with the customer.

ECSS-Q-ST-70-01_1650166

The temperature and location of the witness sample shall be representative for the most contamination sensitive hardware in the vacuum chamber during the pre-TV test and actual TVAC testing.

ECSS-Q-ST-70-01_1650167

Surface particle contamination deposition levels per TVAC cycles shall be

compatible with customer TVAC cycle budget allocations;

verified and demonstrated to be compatible against the PFO values specified in Table 5-6.

More stringent requirement can be specified due to mission/hardware constraints.

ECSS-Q-ST-70-01_1650168

The locations and temperatures of QCMs during TVAC cycles shall be assessed if TVAC is used as the verification activity for the in-flight molecular contamination model.

ECSS-Q-ST-70-01_1650169

Table 5-6: Correlation airborne particles and PFO for TVAC facilities

| ISO class | PFO (mm2/m2/per cycle) |
| --- | --- |
| 5 | 2,0 |
| 6 | 10 |
| 7 | 52 |
| 8 | 275 |
| NOTE The data contained in this table are based on several measurements performed in different cleanrooms. They are represented by this approximate law: PFO = 0,069 10(0,72M-2.16) where M is the ISO class (e.g. ISO class 5) |  |

### Other facilities

ECSS-Q-ST-70-01_1650170

The CRS and the C&CCP shall address the cleanliness and contamination control policy for any other facility such as acoustic chamber, vibration facilities, anechoic chamber, and EMC chamber.

## Activities

### Cleaning of hardware

#### General aspects

ECSS-Q-ST-70-01_1650171

Cleaning shall be performed in order to ensure that the required cleanliness levels, expected in the contamination budget, and the final product cleanliness level are achieved.

In order to meet the BOL requirements, a final cleaning of external surfaces can take place just before the entry of the space system into the fairing, or even just before closing the fairing.

ECSS-Q-ST-70-01_1650172

The choice of the cleaning method shall be determined by the following criteria:

The type of contaminants to be removed.

The physical or chemical nature of the item to be cleaned.

The actual on ground phase.

1	Some examples are provided in Annex K for removal of both particulate and molecular contamination.

2	The cleaning of some parts is particularly important during the course of manufacture or before processing (e.g. prior to bonding, painting, vacuum, coating, welding and soldering).

3	Any detrimental effect of cleaning is evaluated as well as the order of the defined cleaning methods.

4	For those items that are too delicate to withstand cleaning, preventive contamination control is of the utmost importance.

5	See also ECSS-Q-ST-70-54 “Ultracleaning of flight hardware”.

ECSS-Q-ST-70-01_1650173

The cleaning procedures shall be mentioned in the process specification.

ECSS-Q-ST-70-01_1650174

The cleaning procedures shall be validated by tests on representative samples, or by experience from previous and similar projects, in which they were validated.

#### Cleaning tools

##### Cleaning aids

ECSS-Q-ST-70-01_1650175

Cleaning aids shall not increase the contaminant levels of the items to be cleaned.

ECSS-Q-ST-70-01_1650176

Aids, such as wipe tissues, papers, cloths, brushes and foams shall be non­flaking, lint free and dust free.

ECSS-Q-ST-70-01_1650177

Damage to surfaces as scratches shall be minimal.

ECSS-Q-ST-70-01_1650178

NVR of cleaning wipe materials shall be less than 0,01g/m2 for wiping extremely clean surfaces when extracted with IPA.

Different examples of NVR for common tissues are given in Annex A.

ECSS-Q-ST-70-01_1650179

When wipe materials are selected for cleaning, measurements shall be taken to determine their contaminant content.

ECSS-Q-ST-70-01_1650180

All wipe materials should be precleaned to achieve the specified level of cleanliness.

Extraction by solvents is the way for precleaning the wipes materials.

##### Cleaning fluids

ECSS-Q-ST-70-01_1650181

The cleaning solvent shall be selected based on its compatibility with the material or item to be cleaned and its efficiency in removing contaminants.

A compatibility table between materials and solvents is given in Annex H.

ECSS-Q-ST-70-01_1650182

Toxicity and flammability of solvents shall be evaluated to demonstrate fit for use for the intended environment (see MIL-HDBK-406).

ECSS-Q-ST-70-01_1650183

For precision cleaning, solvents of high purity shall be used.

ECSS-Q-ST-70-01_1650184

Verified cleaning processes shall be applied to clean sensitive surfaces.

ECSS-Q-ST-70-01_1650185

The cleaning gas shall be:

verified clean or certified,

free of oil,

filtered to remove water contamination, molecular or particulate contamination in accordance with the mission needs.

ECSS-Q-ST-70-01_1650186

The use of chemicals shall be in line with applicable local laws.

1	EU REACH, UK REACH, Swiss Chemicals Ordinance (ChemO) regulation and others ban or restrict the use of certain substances.

2	Basics of chemical regulatory framework and associated obsolescence risk management is described in detail in ECSS-Q-HB-70-23A, Annex D.

### Cleanliness monitoring of space hardware

#### General

ECSS-Q-ST-70-01_1650187

Particulate and molecular contamination shall be monitored.

during all the on-ground phases;

for specific missions during launch and in space.

If monitoring is not feasible, advanced modelling (e.g. outgassing via CVCMs, PAC redistribution under fairing, decontamination efficiency) can be used as an alternative.

#### Contamination monitoring in ambient environments

##### Particulate contamination monitoring

ECSS-Q-ST-70-01_1650188

Particulate contamination shall be monitored through visual inspection and be quantified through optical monitoring of surfaces as per ECSS-Q-ST-70-50.

ECSS-Q-ST-70-01_1650189

Surfaces shall be examined using a suitable methodology and inspection sensitivity as outlined in ECSS-Q-ST-70-50:

visibly clean standard to be performed under oblique white light of more than 540 lx and from a distance of 150 cm to 300 cm using normal vision;

visibly clean sensitive to be performed under oblique white light of more than 540 lx and from a distance of 60 cm to 120 cm using normal vision;

visibly clean highly sensitive to be performed under oblique white light of more than 1080 lx and from a distance of 15 cm to 45 cm using normal or magnified vision.

1	Different kinds of lights can be used: portable diving light or “white light” is often used for standard inspection. In addition, ultra­violet lamp or “black light” (365 nm) can be used for inspection of organic residues and dust particles as it increases their visibility.

2	Typical methods are the measurement of transmission or reflection loss and nephelometry (i.e. scattering of light). These methods can be used for all types of contaminants, both organic and inorganic. Photographic determination of dust particles on surfaces is also possible, as is automatic counting.

3	Extraction methods can be performed by:

tape lift, using sticky tapes (according to ECSS-Q-ST-70-50);

blowing and suction of air;

washing of the surface of interest and counting the particles in the washing fluid either directly using a commercial instrument, or on a filter after filtration of the liquid.

4	Visible inspection cannot be used to quantify surface cleanliness levels, it is only a qualitative method as per ECSS-Q-ST-70-50.

ECSS-Q-ST-70-01_1650190

When using ultra­violet or “black light” (365 nm) lamps for inspection of organic residues, the induced thermal and health effects shall be assessed.

##### Molecular contamination monitoring

ECSS-Q-ST-70-01_1650191

Molecular contamination shall be monitored as follows:

visual inspection methodology used only for qualitative assessment;

quantitative methods using direct surface measurements, and witness materials, or indirect methods as described in ECSS-Q-ST-70-05.

1	to item 1: Surfaces can be examined by the same visual inspection methods as for particulate contamination; A contamination can be revealed by contrast.

2	to item 2: Direct measurements can be made in situ using quartz crystal microbalances (QCM).

3	to item 2: Further analyses can be performed to characterize molecular contamination (e.g. gas chromatography, mass spectrometry, SEM).

#### Contamination monitoring in vacuum facility

ECSS-Q-ST-70-01_1650192

Monitoring of molecular and particulate contaminants in vacuum facilities shall be achieved using:

for all contamination sensitive and contamination critical items the QCM together with the witness method, including auxiliary items, and

for all non-contamination sensitive and non-contamination critical items the QCM or the witness method or a combination of both.

ECSS-Q-ST-70-01_1650193

When using the witness method the temperature and location of witnesses shall be representative of the item.

1	Witnesses (for both molecular and particulate contamination) can be placed on or near suspect places for a specified time and then subjected to one of the standard analyses.

2	A QCM can be used to detect contamination levels down to 1  10-9 g/cm2, and to measure condensation rates. Such QCMs can operate down to liquid nitrogen temperatures.

3	A mass spectrometer is not sufficient to monitor the condensable contaminants but in combination with a QCM, it can help describing the different condensed species during a controlled re­evaporation from the QCM.

4	A cryopanel can be used to collect all molecular contaminants for further analyses.

5	Additional representative witness materials can be used to evaluate contaminant-surface interactions.

#### Bakeout

ECSS-Q-ST-70-01_1650194

The supplier shall perform vacuum bakeouts to reduce the non-water contaminants outgassed in vacuum and the surface contamination collected during ground life.

1	Bakeouts started at the lowest possible product level allow to reach highest bakeout temperatures (i.e. avoiding the temperature constraints at higher assembly level) and hence a more efficient bakeout programme.

2	Typical materials on which bakeout is applied are:

Harness,

MLI,

Carbon and glass fibre components,

Glued, coated or potted materials,

Contamination critical items/ equipment / sub-system,

Contamination sensitive items/ equipment / sub-system.

ECSS-Q-ST-70-01_1650195

The bakeout conditions (temperature, time, pressure) shall not have a detrimental effect on the functionality of the item / equipment / sub-system under bakeout.

ECSS-Q-ST-70-01_1650196

The bakeout chamber shall incorporate (by design) at least one “cryo-panel” (Liquid Nitrogen cooled plate - “scavenger plate” or “cold trap”) to collect and trap outgassed components evolved during the bake-out.

ECSS-Q-ST-70-01_1650197

When determining the bakeout temperature, the following aspects shall be considered:

The maximum survivable temperature of the limiting material within the item, equipment, or subsystem is determined, ensuring no degradation occurs.

The maximum temperature experienced during the mission, whether in operational or non-operational mode, is identified.

The qualification temperature of the item, equipment, or subsystem does not exceed, accounting for the temperature measurement uncertainties of the facility.

Items with cavities and venting holes require an effective bakeout strategy due to potential inefficiencies during the process. Outgassed products can take a long time to fully vent and can potentially cross-contaminate sensitive surfaces within the cavity.

ECSS-Q-ST-70-01_1650198

The effectiveness of the bakeout shall be monitored by using a QCM considering:

the QCM is properly installed and operated;

the observed frequency change during the hardware test is large enough to avoid artefacts in the measurement while avoiding saturation of the sensor;

The bakeout is performed when all parts have reached temperature within 0,5 °C of the steady state temperature;

temperature fluctuations and temperature r.m.s. variation cannot be more than ±1,0 °C in an hour and no more than ±2,0 °C in a day;

temperature drift cannot be more than a 1,0 °C change in the average temperature of consecutive 24-hour periods;

QCM temperature is constant;

sample temperature and facility pressure data is provided along with the QCM data.

1	to item 2: Depending on which type of the QCM is used, the dynamic range of a QCM can be anywhere from a few kHz to a few 100 kHz.

2 	to item 2: The actual observed frequency rate depends on the view factor (VF) of the QCM with the hardware. Ideally, the VF is such that the full range of the QCM is used. If the VF is too small, the observed frequency change can contain artefacts from the temperature dependence of the sensor crystals and/or electronics. These artefacts are typically of the order of a few Hz, so a sensor signal of 10s of a few Hz is necessary to avoid that the artefacts become dominant in the analysis. On the other hand, if the VF is too large, the risk is that the sensor saturates within hours making analysis impossible.

3	to item 2: A saturated QCM in-situ can be cleaned by thermal evaporation of the contaminants on the sensor crystal, by temporarily setting the sensor temperature to the maximum temperature before returning to the original (or new) set point.

4	to item 6: In addition to the QCM, other complementary methods e.g. Residual Gas Analysis (RGA) can be used to monitor the chemical species in vacuum.

5	to item 6: QCM sensor temperatures are typically maintained at temperatures in the range from -50 °C to 0 °C. Since the deposition rate on the QCM depends on the QCM temperature, with low sensor temperatures showing a higher deposition rate than high sensor temperatures, QCM sensor temperature can be used to modify the QCM signal during the bake-out.

ECSS-Q-ST-70-01_1650199

If the steady state temperature as specified in 5.4.2.4e.3 changes during the bakeout the following shall be performed:

a re-evaporation of the QCM, and

restarting of the timer on the 48 hours of data requirement for the analysis.

ECSS-Q-ST-70-01_1650200

Independently of the chosen method, success criteria shall be established and approved before starting the bakeout.

ECSS-Q-ST-70-01_1650201

Independently of any stopping and verification criteria, the minimum bakeout duration shall be 72 hours.

ECSS-Q-ST-70-01_1650202

A bakeout "stopping criterion" of a 1 % per hour deviation from linearity shall be used based on QCM monitoring, with a minimum bakeout duration of 72 hours.

1	This "stopping" criterion is a method to assess whether continuing the bakeout is worthwhile. For example, this criterion can be based on the change in the mass rate, specifically the second derivative of the QCM frequency.

2	The deviation from linearity of the parameter can be optimized based on the contamination sensitivity of the item.

3	If a facility that has a demonstrated QCM view factor is used for the bakeout stopping criterion can be based on deposition rate e.g. grams/hour.

4	See Annex L for detailed information about Bakeout Analysis Methodology.

ECSS-Q-ST-70-01_1650203

The background of the baking facility shall be determined before starting the bakeout.

ECSS-Q-ST-70-01_1650204

Baking time shall start when the material/item under baking has reached the predefined bakeout temperature.

#### Purging

ECSS-Q-ST-70-01_1650205

The purging shall be performed inside a cavity to maintain a constant exchange of the gas present in the cavity.

1	This exchange depends on the entry flow rate of the gas and the total surface leaks.

2	The aim of the purging is not only to protect the critical hardware such as optics from contamination by injecting a non­ionized high­purity dry gas inside a cavity but also a way for decontamination (e.g. removal of water for dimensional stability of composite).

3	The purging can be implemented at instrument or spacecraft level during functional and performance tests at ambient conditions, during repressurization after TB/TV and TV tests, during all the phases without activities and during storage, transport and pre-launch phases up to the final close of the fairing. (In case of an aborted launch, purging can be not re­installed).

ECSS-Q-ST-70-01_1650206

The analytical method used to verify the purge system cleanliness shall be agreed with the customer depending on one or more of the following items:

chemical species of interest;

water content;

number of locations to be verified;

number of gas outlets;

limit(s) of detection of the measurement(s).

ECSS-Q-ST-70-01_1650207

The cleanliness of each pipe shall be verified when multiple purge system pipes are used throughout the system lifetime.

ECSS-Q-ST-70-01_1650208

The purity of the gas at each outlet of the purge system shall be verified using an analytical technique agreed with the customer.

1	For example, purging the gas through a sorption tube followed by thermal desorption GC-MS is an established technique for verifying VOC.

2	In practice, multiple purge system pipes are used throughout the lifetime of the project. Thus, it is a good practice to verify the cleanliness of each pipe individually.

3	In this context “each outlet” is not limited to the final interfaces between the purge system and the spacecraft. Rather, outlets preceding the final interface are also described by this term. For example, connection between the gas source and the purge cart is also considered an outlet.

ECSS-Q-ST-70-01_1650209

Filtering systems (both for MOC and PAC) compatible with the relevant cleanliness requirements shall be provided before the gas comes into contact with the hardware.

ECSS-Q-ST-70-01_1650210

The purging strategy of the mission shall be described and delivered as a part of the CRS in conformance with the DRD in Annex A.

### Packaging, containerization, transportation, storage

ECSS-Q-ST-70-01_1650211

Provisions shall be taken for packaging, containerization, transportation and storage.

In order to maintain the cleanliness levels achieved at any point from initial precision cleaning to delivery to the launch site.

ECSS-Q-ST-70-01_1650212

Cleanliness protection shall be provided prior to leaving the controlled areas, or whenever a storage period is planned.

ECSS-Q-ST-70-01_1650213

Transport and storage containers shall be made of non-particle shedding materials.

ECSS-Q-ST-70-01_1650214

Containers carrying sensitive items shall be pressurized with gaseous nitrogen.

Optical units and payloads are examples of sensitive units

ECSS-Q-ST-70-01_1650215

Containers carrying sensitive items shall also have as rigorous cleaning schedule as the parts themselves.

ECSS-Q-ST-70-01_1650216

It shall be ensured that containers used for transportation of clean parts do not transfer contamination from surface to surface within the cleanroom itself.

Witness plates can be placed inside containers.

ECSS-Q-ST-70-01_1650217

When sensitive items are packaged, containers for long­term storage or transportation, shall include provision for internal flushing with dry high­purity nitrogen and over­pressurization of 100 hPa minimum, except if units are put in sealed bags.

ECSS-Q-ST-70-01_1650218

For long term storage of sensitive items, containers shall be equipped with an inlet valve and an outlet valve clearly identified.

ECSS-Q-ST-70-01_1650219

The design of the container shall facilitate easy cleaning and inspection of its surfaces, avoiding any kind of dirt traps.

ECSS-Q-ST-70-01_1650220

Small clean parts shall be double bagged in airtight envelopes during storage or transportation outside controlled clean areas.

ECSS-Q-ST-70-01_1650221

Bags for contamination sensitive items shall be flushed with dry nitrogen or dry clean air and then sealed.

ECSS-Q-ST-70-01_1650222

Only approved materials that were procured as cleaned films shall be used.

ECSS-Q-ST-70-01_1650223

Static sensitive items shall use metallized films.

ECSS-Q-ST-70-01_1650224

Outer bags shall not enter controlled clean areas.

ECSS-Q-ST-70-01_1650225

When used, desiccants shall be in bags that are clean and do not produce particulate contamination.

ECSS-Q-ST-70-01_1650226

Desiccants and humidity indicators shall be placed in the external envelope.

ECSS-Q-ST-70-01_1650227

Procedures shall be provided for packaging, containerization, transportation and storage.

(normative)
Cleanliness requirement specification (CRS) - DRD

DRD identification

Requirement identification and source document

The CRS is called by the ECSS-Q-ST-70-01, requirement 5.1.2.1a.

Purpose and objective

The purpose of the cleanliness requirement specification (CRS) is to establish cleanliness and contamination levels to be achieved at different MAIT, launch and mission stages.

Based on system or subsystem contamination budget, a CRS is established and agreed by all parties involved.

The CRS defines and identifies the spacecraft items and the environmental areas that are sensitive to contamination; and describes the effects of contaminants on their performance.

Specifying the spacecraft performance requirements to be met is the responsibility of the customer. The spacecraft performances specification is a major input parameter to define the acceptable contamination levels.

The CRS provides the acceptable contamination levels for all on ground and in-flight phases to guarantee that the mentioned spacecraft performances are met.

On ground surface cleanliness levels are also univocally defined.

By using ISO 14644 or IEST-STD-CC1246E.

Expected response

Scope and content

Introduction

ECSS-Q-ST-70-01_1650228

          - The CRS shall give a general overview of the item to which the CRS refers, describing sensitive items and contamination sources, in consideration of:

possible impacts of contaminants on their physical or functional characteristics;

possible effects of contamination on the performance;

their impact as potential sources of contamination.

ECSS-Q-ST-70-01_1650229

The CRS shall specify the pressures (or other molecular fluxes) that can be reached in connection with voltage breakdown, arcing, corona discharges, multipaction, opening time of shutters and ejection time of covers.

Environmental factors

ECSS-Q-ST-70-01_1650230

          - The CRS shall basically specify major on­ground activities to be analysed for their impact on contamination and the relevant on-ground contamination environment.

Usually, the preparation of a flowchart, that can be added as appendix to the CRS, helps in the description (see Annex D).

ECSS-Q-ST-70-01_1650231

The CRS shall specify the flight environmental factors (natural and induced) that affect the contamination phenomena, such as solar radiation, electron, proton and AO fluxes, together with the planned mission profile/duration.

ECSS-Q-ST-70-01_1650232

The CRS shall specify sensitive item temperatures to be used for the analyses during ground and in-flight operations.

The expected temperatures and temperature profiles of these items can be important for condensation and the residence times of the contaminants.

Contaminants

ECSS-Q-ST-70-01_1650233

          - The CRS shall describe all possible contamination sources to be analysed and the maximum acceptable emissions.

For example: Materials outgassing, lubricants escaping from bearings, wear particles from moving parts, terrestrial contaminants such as dust, plume contaminants from thrusters and engines, leaks from fuel systems and from hermetically sealed components, dumps and EVA, co­passengers, fairing and equipment bay items of the launcher.

ECSS-Q-ST-70-01_1650234

The CRS shall specify the chemical nature of the contaminants listed under <3>a. with their vapour pressures or relevant condensation conditions.

ECSS-Q-ST-70-01_1650235

The CRS shall specify the transport mechanisms of the potential contaminants from the sources under <3>a. to the contamination sensitive items or areas to be considered in the contamination analysis.

For example: direct flux, reflected flux, ambient scatter, self­scatter and creeping.

ECSS-Q-ST-70-01_1650236

The CRS shall specify the contamination environment to be applied for design.

For example, molecular column density, maximum molecular deposition on the sensitive items.

ECSS-Q-ST-70-01_1650237

The CRS shall specify the maximum allowable contamination levels for all contamination-sensitive items, where the limits are determined through analysis, based on the acceptable performance losses expected by the mission's EOL.

Laboratory test data could be necessary for adequate analyses.

Contamination budget

ECSS-Q-ST-70-01_1650238

          - The CRS shall specify all cleanliness requirements allocated to the major integration and testing phases.

ECSS-Q-ST-70-01_1650239

The CRS shall specify acceptable contamination levels of MOC and PAC for all on ground and in-flight phases.

Special remarks

None.

(normative)
Cleanliness and contamination control plan (C&CCP) - DRD

DRD identification

Requirement identification and source document

The C&CCP is called by the ECSS-Q-ST-70-01, requirement 5.1.2.2.1a.

Purpose and objective

The purpose of the Cleanliness and contamination control plan (C&CCP) is to establish the data content requirements for the cleanliness and contamination control plan. This DRD does not define format, presentation or delivery requirements for the cleanliness and contamination control plan (C&CCP), which can vary depending on product level (i.e. equipment, subsystem, system), and specific contractual requirements.

A cleanliness and contamination control plan is prepared in order to set out the ways in which the required cleanliness levels are achieved and maintained during the life of the programme, from design to end­of­life.

As it is of fundamental importance for the space system’s performance, the C&CCP is established as early as possible in the programme, in order to properly address the design.

The C&CCP is prepared for all levels of configuration items defined in the project at the following levels:

System

Subsystem

Equipment

The C&CCP is based on the requirements defined by the cleanliness requirements specification (CRS).

The supplier is responsible for this document.

The C&CCP is prepared in collaboration with experimenters and engineers.

Expected response

Scope and content

Introduction

ECSS-Q-ST-70-01_1650240

          - The C&CCP shall contain description of the purpose, objective, content and the reason prompting its preparation.

Applicable and reference documents

ECSS-Q-ST-70-01_1650241

          - The C&CCP shall list the applicable and reference documents to support the generation of the document.

Terms, definitions and abbreviated terms

ECSS-Q-ST-70-01_1650242

          - The C&CCP shall include any additional terms, definitions or abbreviated terms used.

Description of [item name]

ECSS-Q-ST-70-01_1650243

          - The C&CCP shall give a general overview of the item it pertains to.

ECSS-Q-ST-70-01_1650244

The C&CCP shall describe sensitive items and contamination critical items and contamination sources, listing those surfaces/items to be strictly controlled or protected from the cleanliness point of view due to:

The possible impacts of contaminants on their physical or functional characteristics.

Their impact as potential sources of contamination.

Cleanliness requirements

Requirements in CRS

ECSS-Q-ST-70-01_1650245

          - The C&CCP shall contain a summary of cleanliness requirements, relevant for the system or hardware and eventual sub-assemblies, as given in CRS or dedicated analysis.

Example of such requirements are MOC, PAC, and during the different phases on ground and in-flight.

Contamination budgets

ECSS-Q-ST-70-01_1650246

          - The C&CCP shall contain the allocation of contamination levels through the splitting of cleanliness requirements during the major integration and testing phases.

In case the outgassing contribution to the performance loss is large with respect to other contributions (mainly for sensitive instruments with tight requirements), more detailed modelling are performed.

Selection of materials and processes

ECSS-Q-ST-70-01_1650247

          - The C&CCP shall define the requirements that have design impacts like PMP selection criteria (see ECSS-Q-ST-70), venting, purging and thrusters’ locations, in accordance with the mission cleanliness and outgassing requirements and the outcome of the clauses 5.1 and 5.2.

Mitigation and corrective actions

ECSS-Q-ST-70-01_1650248

          - The C&CCP shall describe the preventative and corrective measures for the coordination and resolution of cleanliness and contamination control issues among the parties involved in the project.

ECSS-Q-ST-70-01_1650249

The C&CCP shall describe the preventative and corrective actions in terms of design, shielding, purging, bakeout in case the predictions are outside acceptance limits and in cases where corrective actions are necessary because of deviation from the original cleanliness policy.

In general, the organization of regular workshops dedicated to cleanliness and contamination control for a specific programme is a good practice.

Environments and facilities

ECSS-Q-ST-70-01_1650250

          - The C&CCP shall contain a brief description of MAIT areas, their classification, facility location and tools for contamination control.

ECSS-Q-ST-70-01_1650251

References for internal procedures dedicated to area or facilities verification, control and maintenance shall be included.

ECSS-Q-ST-70-01_1650252

The C&CCP shall contain a list (or brief description) of internal procedures for personnel training and rules to operate under contamination control conditions.

MAIT activities

Contamination prediction

ECSS-Q-ST-70-01_1650253

          - The C&CCP shall detail the splitting of cleanliness prediction during MAIT phases, according to planned duration, environment class, type of operation, and dedicated provisions adopted.

ECSS-Q-ST-70-01_1650254

The C&CCP shall list all phases where contamination can be expected and where the levels can exceed the allocated levels.

Contamination control

ECSS-Q-ST-70-01_1650255

          - The C&CCP shall describe selected methods, procedures and instruments to control contamination levels during MAIT activities on systems or equipment and relevant documentation; in particular:

Contamination monitoring methods and tools.

Inspection procedures and tools.

Verification of tools or hardware.

Dedicated cautions for critical AIV operations.

Cleaning and decontamination methods and tools

ECSS-Q-ST-70-01_1650256

          - The C&CCP shall define the cleaning and decontamination methods, procedures and tools, also making reference to their applicability and eventual process parameters.

List of items and process parameters (e.g. for a bakeout: temperature, pressure and minimum durations, stop criteria are part of the information of this clause.

Packaging, storage and transportation

ECSS-Q-ST-70-01_1650257

          - The C&CCP shall describe the provisions for the transportation of contamination sensitive, contamination critical items, including processes or functions defined in the critical item list (CIL).

Processes or functions that are identified in the Critical Item List susceptible to degradation, failure, or non-compliance and that can negatively affect mission performance result in non-fulfilment of requirements due to cleanliness and contamination control. These items typically require dedicated monitoring, handling, and control measures throughout all project phases, from procurement to launch.

ECSS-Q-ST-70-01_1650258

The C&CCP shall include:

A description of containers and packaging tools to be used during hardware transportation.

The way they are stored.

The way they are handled.

The way they are monitored and cleaned.

The way their cleanliness is verified.

The way their cleanliness is maintained.

Contamination control flow

ECSS-Q-ST-70-01_1650259

          - The C&CCP shall define:

the sampling plan for PAC and MOC,

cleaning operations (when planned), and

inspection points.

ECSS-Q-ST-70-01_1650260

A cleanliness control flow chart shall be established, showing the stages at which specific cleanliness controls are undertaken as defined in the C&CCP, reported in an annex to the C&CCP.

Responsibilities

ECSS-Q-ST-70-01_1650261

          - Responsibility and authority shall be assigned for the implementation of the cleanliness and contamination control tasks.

ECSS-Q-ST-70-01_1650262

The C&CCP shall describe responsibilities for:

Hardware inspections

Cleanrooms and facilities

Contamination monitoring (hardware).

Forms

ECSS-Q-ST-70-01_1650263

          - The C&CCP shall define the forms that are used to document the cleanliness and contamination control activities defined by the C&CCP.

ECSS-Q-ST-70-01_1650264

As minimum, the following forms shall be defined:

PAC and MOC measurement report.

Cleanliness declaration of conformity (see ECSS-Q-ST-20).

Special remarks

None.

(normative)
Cleanliness and contamination control verification report (C&CCV report) - DRD

DRD identification

Requirement identification and source documentation

The C&CCV is called by the ECSS-Q-ST-70-01, requirement 5.1.2.3.1a.

Purpose and objectives

The purpose of this document is to outline the reporting requirements for the cleanliness status of contamination-sensitive instruments. It specifies how the current cleanliness levels are evaluated against the predefined requirements and allocations for all mission phases, as stipulated in the Contamination Control and Cleanliness Plan (C&CCP) as defined in Annex B.

This document provides the outcomes derived from cleanliness measurements and ensures that the relevant requirements are met. It includes references to supporting technical documentation, such as witness measurements, verification techniques, cover efficiency, purge efficiency, and simulations (e.g., outgassing, venting, plume, particulate distribution, and other applicable models), as well as the associated analyses. These documents are available for review in conjunction with this report.

As a controlled, evolving document, it is updated as necessary to reflect any changes. The document also contains a comprehensive list of Requests for Actions (RFAs), including cleaning processes, Requests for Deviations (RFDs), and Requests for Waivers (RFWs), that may impact the cleanliness and contamination control of sensitive instruments and critical surfaces.

Expected response

Scope and content

Introduction

ECSS-Q-ST-70-01_1650265

          - The C&CCV shall contain description of the purpose, objective, content and the reason prompting its preparation.

Applicable and reference documents

ECSS-Q-ST-70-01_1650266

          - The C&CCV shall list the applicable and reference documents to support the generation of the document.

Terms, definitions and abbreviated terms

ECSS-Q-ST-70-01_1650267

          - The C&CCV shall include any additional terms, definitions or abbreviated terms used.

Description of contamination sensitive items

ECSS-Q-ST-70-01_1650268

          - The C&CCV shall provide a table of:

contamination sensitive items and contamination critical items as defined in Annex B;

impacted performance for the contamination sensitive items;

the implemented contamination verification techniques including but not only for the allocated budgets, but also for major MAIT activities, transportation, storage, launch campaign, launch, in-flight and in-orbit;

contamination mitigation measures;

contamination preventive measures;

contamination corrective measures.

ECSS-Q-ST-70-01_1650269

The C&CCV shall provide a list and status of all RFAs, RFDs and RFWs impacting the analysis of all the contamination critical and contamination sensitive items.

Verification Matrix

ECSS-Q-ST-70-01_1650270

          - The C&CCV shall provide:

a verification and a validation matrix and reference to supporting documents for all items in the C.2.1<4>;

a verification matrix for all requirements in Annex A and Annex B for all contamination sensitive items;

a verification matrix for all cleanliness and contamination control impacted items defined in critical item list (CIL);

the values of the verified efficiency of contamination corrective, mitigation  and preventive actions to reduce contamination agreed with the customer;

all mathematical models and simulations verified against experimental data to ensure its accuracy and validity.

to item 3: Processes or functions that are identified in the Critical Item List susceptible to degradation, failure, or non-compliance and that can negatively affect mission performance result in non-fulfilment of requirements due to cleanliness and contamination control. These items typically require dedicated monitoring, handling, and control measures throughout all project phases, from procurement to launch.

Results

ECSS-Q-ST-70-01_1650271

          - The C&CCV shall provide a table comparing the maximum allowable contamination levels for each contamination sensitive item with the contamination levels verified by measurement for on-ground phases and by modelling for the in-orbit phases.

Synthesis

ECSS-Q-ST-70-01_1650272

          - The C&CCV shall certify the compliancy of the contamination levels regarding the contamination requirements for each sensitive items.

ECSS-Q-ST-70-01_1650273

The C&CCV shall provide the acceptability status of the contamination levels documented in the Table requested by requirement <6>a. regarding the impacted system performances.

Special remarks

None.

(informative) 
Cleanliness and contamination control process overview

: Cleanliness and contamination control process overview

(informative)
Guidelines for general cleanliness and contamination control

General

Contamination control cannot be applied effectively without an understanding of the contaminant, the contaminant source and the detrimental effect that the contaminant has.

The known causes of failure and degraded performance of space elements attributed to contamination, including their sources, are given in this Standard.

When they are not known, tests and analyses can be performed (e.g. outgassing rates as a function of time, chemical composition of outgassing products, condensation rates or degradation as result of radiation).

The results of these tests and analyses can be used to calculate expected contamination levels and their subsequent effects if other relevant parameters are known.

Preventive cleanliness control is becoming more important as space systems become more sophisticated and mission durations are extended.

A problem frequently encountered in space technology is the lack of data enabling a good correlation between contaminant levels and mission performance requirements.

This kind of information can be available in the mass production areas of electronics and precision mechanical devices.

Contamination attributes

Typical contaminants and their sources

On ground

Particulate contaminants

Many particulate contaminants, such as dirt, sand, industrial fumes, can, to a large extent, be excluded from cleanrooms by filtering, and the space system can therefore be protected from them right up to the final preparation at the launch site.

Nevertheless, a considerable quantity of particulate contaminants are produced or released during all on ground phases of the space system, especially during testing activities.

Test facilities can be inside cleanrooms, but are basically not clean and can loosen particles and cause their redistribution

For example:

Human sources

Hair cosmetics, dead human skin cells.

Fibres and lint from clothing, dust carried in on hair and clothes.

MAIT

Chips and burrs from machined surfaces, solder and weld spatters.

Particles produced by wear or shedding, corrosion products, flakes from coatings and air filters.

Particles released from anechoic walls during the test.

Redistribution of particles during pumping down and repressurization of vacuum chambers, vibration test, transportation…

Other sources

Bacteria, fungi, viruses and secondary products

Molecular contaminants

A considerable quantity of molecular contaminants are produced during all on ground phases of the space system, especially during testing activities.

Test facilities even if inside cleanrooms can be source of molecular contaminants the tested items.

Molecular contaminants can be found in different chemical phases, such as:

Gases and vapours

Atmospheric gases, desorbed water, leaks in sealed units (e.g. freon, hydrazine, helium, neon and krypton)

Outgassing products from organic materials (e.g. monomers, plasticizers, additives and solvents)

Vapours from packaging materials and test facilities (e.g. vacuum pump oils)

Vapours from substances used in cleanrooms (e.g. plasticizers and cleaning fluids)

Secondary products coming from micro­organisms.

Liquids

Residues from cleaning agents

Residues from adhesive masking tapes

Machine oils

Coolants

Lubricants

Solder fluxes

Cosmetics

Grease from human skin

Secondary products coming from micro­organisms.

Other contaminants

salt

acid

alkaline

corrosion products

oxidation products

finger prints

stains.

On launch

Particulate contamination

Launch contaminants can come from the acoustic noise and mechanical vibrations. The contamination source is then the space system itself and the fairing and structural parts during the first minutes of the launch.

Redistribution of the released particles can occur so that clean surfaces are covered by particles.

Also the co­passengers can be the source of contamination in the case of multiple launches.

During the initial lift­off, the pressure inside the fairing drops from atmospheric pressure to high vacuum within a few minutes, and the turbulence of the air can also redistribute the particles.

The contamination environment during launch can be severe and there is basically no control of the contamination during this period.

Molecular contamination

Next to the particle contamination, the molecular contamination is of importance, especially the outgassing of the materials, the release of contaminants by mechanisms, separation mechanisms, such as pyrotechnics and thermal knives, motors and thrusters.

The mechanism of molecular contamination is based upon outgassing under high vacuum; however, the period for which the space system is under high vacuum with other neighbouring hardware is very short.

Predictions of the molecular transfer during this period can be estimated: this type of estimation can be made for co­passengers when the outgassing requirements are less stringent than those for the space system of interest.

During mission

Overview

Even with a good contamination and cleanliness control policy, contamination during mission can not be completely avoided. Indeed, lessons learned from space systems returned to Earth after quite a long exposure to space e.g. LDEF, EURECA and solar arrays from the Hubble Space Telescope, indicated visible contamination especially near venting holes and at locations where photodeposition and photopolymerization occurred due to solar radiation, or where atomic oxygen has converted the volatile contaminants into non­volatile contaminants.

Natural environments and induced environment are normally taken into consideration.

Natural environment

The natural environments described here affect the contaminants in the environment or on surfaces or affect the deposition of contaminants on surfaces.

Most of the natural environments mentioned in the following clauses are described in detail in ECSS-E-ST-10-04.

- Vacuum and type of gases

The pressure of natural gases around the space system causes reflection of the outgassing molecules originating from the space system. This reflection is called “ambient scatter” and can result in a return of the space­system­produced contaminants to the space system itself.

This type of reflection depends on the level of vacuum and thus upon the orbital altitude. For low Earth orbits the ambient scatter can result in a return contamination flux of a few percent.

The gas composition of the Earth environment is such that only in cryogenic space system applications are contamination problems to be expected.

Radiation (solar and other electromagnetic radiation)

Solar radiation and especially the ultra­violet part can have effects such as polymerization and decomposition of already deposited contaminants. Photon induced deposition.

The generally observed effects are a reduction in reflection and transmission of light for optical experiments and solar arrays. Another observed effect is the increase of solar absorptance of thermal control surfaces, which results in a temperature increase for those surfaces.

Solar radiation can also affect the contamination deposition mechanisms, and although this combined effect of contamination and electromagnetic radiation is theoretically difficult to describe, this phenomenon is well known.

Radiation ionizes the outgassed molecules in space and so can influence the amount of ionized particles.

Also, the ionized molecules are attracted by a negative charged space system and thus contaminate it.

Thermal aspects (thermal cycling)

Solar radiation, rotating space systems and planetary shieldings cause temperature cyclings and these temperature cyclings have effects on the outgassing of materials and on the condensation and evaporation of contaminants on surfaces whose temperatures vary.

Atomic oxygen (AO) (speed effects)

AO is the main constituent of the residual atmosphere in Earth orbit at between 200 km and 700 km altitude. The density is a function of altitude and of other parameters such as solar radiation. In most cases the effect of thermal AO on deposited contaminants can be neglected. However, due to the relative velocity between AO and the space system (approx 8 km/s) the collisional energy in the ram direction is around 5 eV.

The items and surfaces in the ram direction of the space system can be attacked by AO, whereas the items and surfaces in the wake direction are hardly attacked.

The effect of AO can be described as an oxidation and some materials can become resistant to AO, e.g. non­volatile oxides can be formed on some metals.

Organic materials can be oxidized to volatile products such as CO and H2O. The presence of silica contaminants on space system surfaces can be explained by the attack of condensed silicone species by AO and the formation of SiOx.

Charged particles (electrons, ions)

The effect of charged particles on outgassing and on already condensed contaminants is probably small, but no exact data are known at this moment.

Micrometeoroids (debris)

Micrometeoroids have no direct effect on outgassing and on condensed contaminants. Micrometeoroids can pierce some materials and can also result in partly destruction of some materials, which causes release of a large amount of new particles which escape into space or affect neighbouring items. Impacts can also cause evaporation of the micrometeoroid and of the impacted surface.

Redistribution of particles, which were already on space system surfaces by micrometeoroid collisions, have been reported, but the effects are very small.

Speed effects of space systems w.r.t. molecular speeds

The speed of a space system has no direct effect on the outgassing of materials or on the deposition mechanisms of contaminants on surfaces.

However, the return contamination flux via the ambient scatter is highly affected by the local pressure around the space system. This local pressure depends upon the actual space system speed with respect to the speed of the natural species. Because of the speed effects, the ram direction pressure can be orders of magnitude higher than the normal pressure for that orbit and the ambient scatter is then also orders of magnitude higher than the normal ambient scatter.

The same can be expected for the wake directions, i.e. orders of magnitude lower pressures and thus orders of magnitude lower ambient scatter than expected.

Induced environment

The space system environment can be seen as being created by the space system itself or by its operation.

- Gas and fluid leakage from pressurized systems

In space systems one can expect sealed pressurized units, such as batteries and gyros; for these units leak rates have been specified which do not result in unacceptable performance losses of those units. However, the level of their leak rate or their location in the space system can be such that the performances of sensitive items can be affected. Also the leaks from pressurized units such as containers holding propulsion gases or fluids (e.g. hydrazine) can affect contamination sensitive items.

Within long­term space (station) programmes a number of fluids are used, which potentially emerge from containments such as tanks, lines and pressure shells to the exterior by leakage, venting or purging.

All fluids contribute to contamination.

Contaminants from release mechanisms and moving mechanisms

Release mechanisms such as cable cutters and mechanisms based upon sealed units with explosives, release particles from adjacent surfaces due to the mechanical shocks.

Mechanisms that are based upon cutting of cables using thermal knives release both molecular and particulate contaminants.

Contaminants from operating thrusters, engines or other propulsion systems

Solid booster engines produce particles as well as molecular contaminants, liquid gas rockets produce mainly gaseous contaminants, and hydrazine thrusters produce gaseous reaction products and some unburned fuels. Ion thrusters mainly produce not fully neutralized gaseous products such as xenon and a small amount of sputtered metal from the neutralizing grid material.

Release of contaminants that were collected during ground activities

During the ground life of the space system, both molecular and particulate contaminants can be deposited, mainly on the external surfaces. During launch especially particulate contaminants are released and during the mission itself their release is mainly caused by shocks. The release of these particulate contaminants from external surfaces by impingement of micrometeoroids and debris is small compared to the amount of particles released from the surface materials by the same impingements.

For molecular contaminants that collected on surfaces during ground activities, the same outgassing effect can be expected as from material outgassing.

Secondary products

Secondary products are generated by various intermolecular interactions and chemical or physical processes due to payload or experiment operations or interactions of the natural and induced environment constituents and the space

Transport mechanisms

Overview

Most of the effects of contamination occur in space, especially when solar radiation is involved.

Contaminants transport on ground

Overview

Surfaces can become contaminated by particles during all on ground phases (e.g. MAIT, pre-launch, and transportation).

Particle transport

For particles transport, the main mechanisms are fallout and air transport, especially caused by air turbulences (e.g. human activities and pumping down and air inlet in vacuum facilities) and vibrations (e.g. vibration test and acoustic test).

Molecular contaminants transport

For molecular contaminants transport, the main mechanisms are due to diffusion of airborne contaminants and creeping of liquids.

During vacuum tests, the mechanisms are basically the same as in space (see next clause).

Contaminants transport in space

Particle transport

Only the surfaces in direct view of other surfaces can be contaminated by particles originating from the other surface.  Return of released particles to the space system, for example return of charged particles to a charged spacecraft (depending on their mass), can occur but such mechanisms are not often modelled..

Molecular contaminants transport

Creeping

Liquid contaminants and also lubricants can contaminate adjacent items by the liquid creeping over surfaces, and silicone fluids especially are known to have a high creeping effect.

In order to reduce the creeping effect, anti­creep barriers made of special materials are generally applied.

Direct flux

Contaminants in space move in straight lines from the space system into deep space and can sometimes contaminate items located in the direct view.

In order to mitigate this effect, design provision are generally a solution.

Indirect flux

Contaminants from space systems can impinge on a surface and after reflection (specular) or re­evaporation (diffusive) these contaminants can affect other items or areas.

Collision with natural gases

Contaminants coming from the space system can collide with the natural gases around the space system and after collision can return to the space system. This phenomenon is known as ambient atmospheric scattering and depends upon the density (and thus upon the altitude) of the natural gas.

Collision with other outgassed molecules

Molecules released from space systems (e.g. outgassing via venting holes) can collide with other molecules from the same origin or from other origins (e.g. plumes). After the collisions, some molecules can return to the space system. This phenomenon is called self­scattering and the return flux strongly depends upon the intensities of the fluxes from the contaminant sources.

Ionization of gaseous contaminants and the re­attraction by the negative charged space system

Contaminants emitted from the space system can be ionized in space by solar radiation (especially ultra­violet radiation, electrons, protons and ions) and these ions can be re­attracted by a negative charged space system. This phenomenon is well known, but has not yet been quantified.

One of the simple rules is that instruments that have deployable shutters or ejectable covers can be deployed or ejected when the outgassing has dropped to a certain level or after a pre­determined time after the launch.

Main effects of contamination on space systems

The main effects of contamination are:

Failure of precision mechanisms due to particulate matter.

Light scattering by particle and molecular contaminants.

Electrical discharge or arcing in high voltage equipment due to high outgassing and other contamination.

Noise on slip rings and electrical contacts.

Results of certain experiments obscured by excessive molecular contamination (e.g. mass spectrometers and ion counters).

Degradation of optical elements (e.g. lenses, mirrors and windows) due to molecular contamination, especially X­ray and UV equipment and low temperature IR detectors.

Degradation of thermal control surfaces (absorptivity/emissivity ration, /) especially in the case of molecular contamination on optical solar reflectors at low temperatures.

Loss of efficiency in heat pipes.

Effects on conductive and non­conductive surfaces (leak paths in electronics).

Loss of efficiency in solar cell generators.

Corrosion of electrical contacts due to the presence of halogenated solder fluxes.

Space charge and discharge effects related to contaminants.

Thermal radiation from particles.

Disorientation due to erroneous reaction of star trackers to luminous particles.

Multipaction in waveguides.

Bad closing of a valve.

Explosion of a cryotechnic motor (Oil + O2).

HF for a motor.

Disturbance of gas flux and combustion within thrusters.

Disturbance and propagation within RF wave guides.

For the space environment around the space system, the “column density”, local gas pressures and gas composition can be limiting factors for some experiments.

(informative) 
Cleanliness-oriented design

The lowest contamination levels can be achieved by applying the following rules:

- Locate contamination sensitive items far away from the contaminant sources.

Position the sensitive items so that the view factors with respect to contaminant sources (e.g. solar arrays, antennas and thrusters) are as low as possible.

Locate the vent holes of the space system and the instruments away from the sensitive items (= backdoor venting).

Manufacture the hardware in such a way that venting (of, for example, thermal blankets) is directed towards the backdoor.

Design baffles or shields for the sensitive items or even for the contaminant sources.

Design temporary covers (red­tag covers) or hoods to reduce contamination during ground life. (Optically transparent covers can be used for calibration, alignment or functional testing of optical instruments without removing the covers)

Design deployable covers for very sensitive instruments, that are operated only in space.

Design cleaning mechanisms for the removing of contaminants by, for example, heating the sensitive hardware, manoeuvre the space system in such a way that in low orbit the AO can perform a cleaning.

Selection of materials, processes, mechanisms and components with low particulate and molecular, and bio- contamination potential. In this respect low outgassing materials are chosen, and zinc and cadmium (or cadmium plating) are not be used because of the relatively high vapour pressures of these materials.

If the contamination potential of selected materials is still too high, it is good practice to consider bakeout of the hardware before assembly or even during tests.

The design, manufacturing order and assembly are such that bakeout can still be performed (sometimes baking is carried out before further assembly is done because of the temperature limitations of certain hardware or because the products released during the bakeout can have effects on other items).

Where sensitive items are expected, the design of the instruments or space system is such that purging is feasible in the periods of assembly, integration, tests and launch preparations or even up to launch.

Based upon these effects, it is good practice to locate the venting holes and other contaminant sources in the wake side of the space system.

On the other hand, the wake side of a space system can be used for special experiments for which extremely low pressures in relatively low Earth orbit are required.

Cleaning aptitude of materials and mechanical parts, e.g. in case hardware cannot be cleaned after manufacturing temporary protection devices are needed.

Measurement of contamination, e.g. adaptations for contamination sensors.

Verification of the compatibility of the surface treatment with the cleanliness level.

Ground support equipment, packaging, containerization, transportation and storage.

Any other design provisions according to the specificity of the mission (e.g. planetary missions).

(informative) 
Modelling guidelines

Introduction

Molecular and particulate contamination characterizations are essential for the modelling of contamination in the environment surrounding a space system. Regardless of the specific model used, the following key points are essential to ensure the model's reliability and accuracy:

- Model Definition: The type and level of the model is defined according to the project’s aims.

Consistency and Simplicity: Model components  are consistent with the system, and the model  is as simple as possible while still meeting objectives.

Parameterization: Parameters and measurements are  linked to measurable features of the system for accurate predictions.

Mathematical Definition: The model is  fully defined mathematically, including equations, parameters, and units.

Analytical Methods: Well-understood analytical and numerical methods are  used for analysis.

Software Validation: Software and algorithms are  thoroughly tested, and results are  validated before use.

Data Processing: Well-justified methods are  applied to process data and make comparisons.

Model Verification: The model is  verified against experimental data to ensure accuracy.

Clarity in Presentation: Results are  presented clearly, avoiding unnecessary complexity.

Conclusions: Conclusions are  aligned with the model’s scope, and any speculation are clearly indicated.

Molecular Contamination

For molecular contamination, knowledge of outgassing molecule transport in vacuum is essential. The following points highlight the key considerations:

- Outgassing Parameters: Understanding outgassing fluxes over time, surface temperatures, geometric view factors, and residence times (as a function of surface temperature) is critical for modelling contaminants on surfaces.

Outgassing Data: Modelling methodologies are generally based on outgassing data obtained during kinetic tests as per ECSS-Q-TM-70-52,

For worst-case modelling, Micro-VCM data, as per ECSS-Q-ST-70-02, can be used.

The first assumption is that all contaminants released during the 24-hour Micro-VCM test at 125°C are released during the actual mission lifetime.

The second assumption is that contaminants impinging on a surface can stick, using the geometric view factor (VF).

The third assumption is that contaminants, depending on surface temperature, are permanently deposited (as per ECSS-Q-ST-70-01, Tables 5-1 to 5-3).

Plume Contamination: For plume modelling, knowledge of plume shape, effluence composition, temperature, speed, direction, and discharge frequency are necessary.

Contamination sensitive Instruments: For sensitive instruments like optical devices, modelling can estimate the superficial density of contaminants condensed on surfaces over time. Complementary experimental tests are recommended to evaluate the transmittance losses induced by molecular contaminants.

Particulate Contamination

For particulate contamination, it is good practice to address the following key points:

- Particulate Sources: Identify potential sources of particulate contamination, such as exhaust gases, mechanical abrasion, or debris, is crucial for effective modelling.

Modelling of Particulates: The size, shape, distribution and material composition of particulates are  incorporated into the model to understand how they interact with sensitive surfaces.

Deposition Behaviour: The model accounts for particulate behaviour in different environmental conditions (e.g., vacuum, microgravity), as these factors influence deposition rates and patterns.

Contamination Sensitive Areas: For sensitive equipment, such as sensors, optical instruments, radiators use particulate contamination models to estimate deposition rates and their impact on performance.

Mitigation Measures: Include in the modelling process assessments of potential mitigation strategies, such as shielding or air filtration systems, and their effectiveness in reducing particulate contamination.

Modelling of components: The critical surface area of a component is used in accordance with ECSS-E-ST-35-06 clause 6.2.3.

DRR-Model Foundation

The model is based on the actual design or system configuration to ensure that all contamination sensitive and contamination critical areas relevant to the study are accurately represented. Simplifications can be applied, provided they do not compromise the integrity of the results.

Input Parameters and Environmental Factors

The input parameters are derived from reliable, measured data or validated sources. Parameters can reflect real-world measurements or best estimates, and the environmental factors (e.g., temperature, pressure) are based on the latest available data, accounting for operational ranges and potential lifecycle changes.

Validation and Documentation

The model undergoes a final review and validation before execution, ensuring that all assumptions and parameters are agreed upon by stakeholders. All model setups, input data, and results are documented comprehensively, ensuring transparency and compliance with relevant standards.

This annex provides a structured approach to contamination modelling for space systems, ensuring thorough analysis, verification, and documentation throughout the process.

(informative) 
Compatibility of various solvents with listed materials

Table H-1 is provided for information only. It is the user's responsibility to verify both the compatibility, and the effectiveness of the items listed.

: Examples of compatibility of various solvents with listed materials

| Type of material |  | methanol | ethanol | IPA | acetone | MEK | dichloromethane | chloroform |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| polymer | abbreviation |  |  |  |  |  |  |  |
| Acétal (polyoxymethylene) | ACL | A | A | A | A | C |  | A |
| epoxy |  | B | A | A | B | C |  | C |
| Ethylene‐chlorotrifluoroethylene copolymer (HALAR) | E‐CTFE | A | A | A | B | A | C | A |
| Ethylene‐tetrafluoroethylene (TEFZEL) | ETFE | A | A | A | B | A | B | A |
| TEFLON (Fluoroethylene propylene) | FEP | A | A | A | A | A | A | A |
| TEFLON tetrafluoroethylene | TFE | A | A | A | A | A | A | A |
| TEFLON perfluoroalkoxy | PFA | A | A | A | A | A | A | A |
| Polyamide (NYLON) |  | D | D | D | A | A | B | C |
| polycarbonates | PC | B | A | A | D | D | D | D |
| Low density Polyethylene | LDPE | A | B | A | D | D | D | C |
| High density Polyethylene | HDPE | A | A | A | D | D | C | C |
| Polyimide (Kapton) |  | B | B | B | B | B | B | B |
| Polymethyl methacrylate | PMMA | D | D | D | D | D | D | D |
| polyketone | PK (PEEK) | A | A | A | A |  |  | C |
| Polystyrene | PS | B | A | A | D | D | D | D |
| polysulfone | PSF | A | A | B | D | D | D | D |
| Polyethylene terephthalate (MYLAR) | PET | A | A | A | C | A | D | D |
| polypropylene | PP | A | A | A | B | B | C | D |
| polyurethane | PUR | C | C | C | D | D | D | D |
| Polyvinylidene fluoride (KYNAR) | PVDF | A | A | A | D | D | A | A |
| silicone |  | A | C | A | D | D | D | D |
| Vinylidene fluoride – hexafluoropropylene (VITON A) |  | C | A | A | D | D |  | A |
| metal |  |  |  |  |  |  |  |  |
| aluminum |  | A | B | B | A | B |  | B |
| Copper |  | B | A | B | A | A |  | A |
| 316 Stainless steel |  | A | A | B | A | A |  | A |
| Titanium |  | B | A | B | A | A |  | A |
| miscellaneous |  |  |  |  |  |  |  |  |
| Carbon graphite (e.g. CFRP) |  | A | A | A | A | A |  | A |
| Al2O3 |  | A | A | A | A | A |  | A |
| SiC |  | A | A | A | A | A |  | A |
| Ratings on chemical effects @ 20°C: A: excellent ‐ no damage after 30 days of constant exposure B: resistant ‐ little or no damage after 30 days of constant exposure C: fair to poor ‐ some effect after 7 days of constant exposure D: not recommended ‐ immediate damage can occur Blank: No data available Sources: http://www.nalgenelabware.com http://www.coleparmer.com/techinfo/chemcomp.asp |  |  |  |  |  |  |  |  |

(informative) 
Particulate levels on surfaces

Standard method 1: Particle distribution

This can be done as per IEST-STD-CC1246E and ISO 14644-9:2022.

These documents give the size­number distribution function for particles on surfaces.

Levels are measured by counting the number and sizes of the particles on a known surface area.

Standard method 2: Obscuration factor

Overview

The obscuration factor (OF) is the ratio of the projected area of all particles to the total surface area on which they rest.

This OF is in principle independent of the number­size distribution of the particles and even independent of the shape and colour of the particles. In general the levels are expressed in parts per million (mm2/m2) and acceptable values are roughly between 10 mm2/m2 and 10 000 mm2/m2.

The OF has the advantage that a number of performance loss parameters are directly related to the particle coverage of the critical item.

Correlation for particles on surfaces

A correlation for particles on surface between levels of IEST STD 1246E and the obscuration factor is given in Table I-1.

This correlation is theoretically based on ideal distribution of IEST- STD-CC1246E (i.e. the slope factor of 0,926) and considering only particles between 1 m and 10 m.

: Correlation between ideal class of IEST-STD-CC1246E and obscuration factor

| IEST-STD-CC1246E PCL | ECSS-Q-ST-70-01C OF (mm2/m2) |
| --- | --- |
| 100 | 2 |
| 200 | 27 |
| 300 | 163 |
| 400 | 632 |
| 500 | 1891 |
| 600 | 4766 |
| 700 | 10636 |
| 750 | 15328 |
| 800 | 21650 |
| 900 | 41012 |
| 1000 | 73316 |

(informative) 
Effects of humidity on materials and components

: Effect of humidity on materials and components

| % RH Range | Effect |
| --- | --- |
| 0 - 30 | Serious static charge problems |
| 30 - 50 | Safe for highly polished metal surfaces or closed components |
| 50 - 65 | Marginally safe for humidity sensitive products Contaminated metal surfaces start to corrode |
| 65 - 80 | Corrosion rate increases largely Some plastics swell |
| 80 - 100 | Rapid corrosion Reduced electrical resistivity |
| NOTE For some materials, humidity has an effect on the material dimensional stability |  |

(informative) 
Cleaning methods

Removal of particulate contamination

Overview

The removal of particulate contamination can be performed with, but not limited to, the methods described in K.1.2 to K.1.4, also refer to ECSS-Q-ST-70-54 for more information.

Vacuum cleaning and wiping

Dust can be removed with the aid of an ordinary vacuum cleaner, combined with a good brush. Having the exhaust of the vacuum cleaner outside the cleanroom is preferred to avoid recontamination. Clean air supply to the item to be cleaned is used, otherwise the contamination of items to be cleaned can be increased by the relative dirty air which is extracted from the environment (e.g. when electrostatic attraction can occur). Only vacuum cleaners equipped with HEPA filters are used in a cleanroom and checked with a UV lamp while working.

Wiping is performed with extreme care, otherwise surfaces can be scratched and “dust” can simply be wiped onto other clean items in the vicinity.

Since, in any case, solvent leads particles to the bottom of cleaned part, those particles should be recovered with a vacuum cleaner at the end.

An effective form of wiping can be used of tissues dipped in methanol.

Gas jet cleaning

Another method of removal of particles is the very careful use of a jet of compressed gas, since contamination of the other clean items in the vicinity can result. Cleaning agents, such as brushes, wipe tissues or compressed gas, can themselves contaminate the item to be cleaned and can lead to dust scratching the surface during cleaning. Ionized air is a good approach in the removal of particles by air blowing.

Cleaning with dry ice (e.g. CO2 jet spray) can be very effective.

Tapes and films trapping

Larger particles can be removed by means of polyimide adhesive tape, eventually rolled around a metal or other appropriate tool (e.g. swabs).

The hardware to be cleaned can be coated with shrinkable polymer film and, after drying, the film can be removed with the contaminants. Use of this type of cleaning method needs to be carefully evaluated as it is known to have detrimental effects on some materials (e.g. gold coatings).

Removal of molecular contamination

Overview

The removal of molecular contamination can be performed with, but not limited to, the methods described in K.2.2 toK.2.8.

Mechanical cleaning

Dry wiping: clean lint­free cloth or lens paper is used, however, it has the disadvantage that it can scratch the surfaces.

Wet wiping: a clean cloth or paper is used in conjunction with organic solvents.

Other mechanical cleaning are grinding, brushing and blasting.

Solvent and detergent cleaning

Solvent cleaning: examples are washing, dipping, spraying, vapour cleaning and ultrasonic cleaning.

Detergent cleaning or soap cleaning: Detergent cleaning (or soap cleaning) for, for example, glass, rubbers, plastics, polyamides, PTFE, polypropylene and acrylates and all ferrous metals, including stainless steel. Such detergents also clean non­ferrous metals, such as aluminium and brass, but have an oxidizing effect on their surface. A detergent or soap cleaning is followed up by a final cleaning with solvent to remove all traces of detergent.

Chemical or electrochemical cleaning with, for example, acids, alkalines and salts for smoothing metal surfaces

Films trapping

Use of shrinkable polymer film, peeled after drying, can also be very effective for the removal of molecules (not for optical surfaces).

Gas jet cleaning

Cleaning with dry ice (CO2 jet spray): this is very effective for the removal of molecular layers.

Plasma cleaning

Cleaning with ionized inert low pressure gas: This is very effective for the removal of polymerized products.

Bakeout

Volatilizing under vacuum is especially successful for cleaning assembled units, or when solvent cleaning is too delicate an operation.

Ultra­violet­ozone cleaning

Molecules of an organic nature are activated by ultra­violet light, resulting in dissociation, after which they react with the ozone produced in the air by ultra­violet light.

(informative)
Bakeout Analysis Methodology

Introduction

The criteria at which a bake-out is considered sufficient are that the first and second derivative of the outgassing curves are below certain values. These values are set in such a way as to be reasonably difficult to reach but are somewhat arbitrary in that they are facility specific. The purpose of this note is to describe a more general criterion based on a robust analysis method.

Methodology

The outgassing from a material can usually be described with first order desorption kinetics. During isothermal bake-out, the outgassing characteristic is then given by a sum of exponential decay functions:

f(t)=iai⋅1-e-tτi. (1)

where f(t) is the outgassing characteristic that is being monitored, e.g. the frequency of a QCM kept at a certain temperature. ai gives the contribution of species i to the total outgassing characteristic, t is time and τ_i the residence time of outgassing species i. A single material can have many outgassing species, although at any given time only a few may be dominant. However, since flight hardware generally contains many different types of outgassing materials, the number of outgassing species can be very large at any given point in time.

A QCM cannot discriminate the different outgassing species and treats the outgassing from the hardware as a black box instead. However, Eq. (1) is still a useful way to describe the outgassing in a phenomenological way, as its mathematical properties match the physical properties of the outgassing process. That is, it is a continuously increasing function with a continuously decreasing rate and decreasing higher order derivatives.

From Eq. (1) it follows that the outgassing rate f' is given by

f'(t)=iaiτi⋅e-tτi. (2)

and the change of the rate is given by

f''t=-iaiτi2⋅e-tτi. (3)

In practice, the outgassing characteristic of flight hardware eventually appears linear in time, i.e. the rate becomes near constant. The linear and non-linear part of the outgassing characteristic can be separated using a Taylor expansion of the outgassing rate f':

f't+∆t=f't+f't∙∆t+…. (4)

The higher order terms can be safely ignored since we are interested in the bake-out time at which the deviation from linearity becomes small. Inserting Eq. (2) and Eq. (3) into Eq. (4) we get:

f't+∆t=iaiτi⋅e-tτi-∆t∙iaiτi2⋅e-tτi. (5)

We can define the total deviation from linearity ζ as:

ζ=f''(t)f'(t)∙∆t. (6)

or the deviation from linearity per unit of time ζ' (hereafter referred to as simply the deviation from linearity):

ζ'=f''(t)f'(t)=iaiτi2⋅e-tτiiaiτi⋅e-tτi. (7)

The inverse of ζ' is also a useful quantity which we define as the apparent residence time τapp according to

τapp=1ζ'=f'(t)f''(t). (8)

The apparent residence τapp can be considered as a weighted average residence time of all species outgassing after a time t.

The deviation from linearity or the apparent residence time are convenient quantities on which to base the stopping criterion for bake-outs. Regardless of the actual distribution of the outgassing species and their individual temperature dependence, at any given bake-out temperature the most volatile species always outgas first until their contribution becomes negligible compared to species with higher residence times. As follows from Eq. (1), for a single species i with residence time τi, the reservoir of species i after t = τi is just 37 % of the initial amount present. At t = 3∙τi this has decreased to just 5 % of the original, and less than 1% after t = 5∙τi. Thus, over time, the contribution of the most volatile species (smallest τi) decreases. As a result, the apparent residence time can increase and the deviation from linearity decreases. The time at which a deviation from linearity of less than x %/hour (or τapp > 100/x hours) is reached does depend on the distribution of outgassing species after a certain bake-out time. It does not depend on the total amount of outgassing mass from the hardware nor on the heating rate of the facility, since the analysis is valid only for the isothermal part of the bake-out. Setting a stopping criterion below a deviation from linearity of x %/hour thus ensures that a “best effort” has been reached.

The value at which the stopping criterion is set can be decided on a case-by-case basis, depending on the expected efficiency of the bake-out temperature w.r.t. the nominal flight temperature, with a maximum value of 1 %/hour. The actual time at which the stopping criterion ζ’ < x %/h (τapp > 100/x h) is reached varies depending on the outgassing characteristics of the species involved, but usually this time is somewhere between 0,5×(100/x) hours up to 3×(100/x) hours. The practical implementation of the method requires Eq. (1) to be curve-fitted to the QCM data of the isothermal bake-out phase to avoid interference from e.g. temperature fluctuations. The deviation from linearity and/or the apparent residence time can then be calculated from the first and second derivative of the fit function.

# Bibliography

| ECSS-S-ST-00 | ECSS system – Description, implementation and general requirements |  |
| --- | --- | --- |
| ECSS-E-ST-35-06 | Space engineering — Cleanliness requirements for spacecraft propulsion hardware |
| ECSS-Q-ST-10 | Space product assurance – Product assurance management |  |
| ECSS-Q-ST-70-54 | Space product assurance — Ultracleaning of flight hardware |
| ECSS-E-ST-10-04 | Space engineering – Space environment |  |
| ASTM E1216 87(1992) | Standard Practice for Sampling for Surface Particulate Contamination by Tape Lift |  |
| ASTM E1235M 95 | Standard Test Method for Gravimetric Determination of Nonvolatile Residue (NVR) in Environmentally Controlled Areas for Spacecraft |  |
| ASTM E1559 93 | Standard Test Method for Contamination Outgassing Characteristics of Spacecraft Materials |  |
| ISO 15388 | Contamination and Cleanliness Control |  |
| NASA SP 5076 | Contamination control handbook |  |
| MIL HDBK 406 | Contamination control technology cleaning materials for precision precleaning and use in cleanrooms and clean work |  |