Space engineering

ECSS Secretariat

ESA-ESTEC

Requirements & Standards Division

Noordwijk, The Netherlands

Product data exchange

Foreword

This document is one of the series of ECSS Technical Memoranda. Its Technical Memorandum status indicates that it is a non-normative document providing useful information to the space systems developers’ community on a specific subject. It is made available to record and present non-normative data, which are not relevant for a Standard or a Handbook. Note that these data are non-normative even if expressed in the language normally used for requirements.

Therefore, a Technical Memorandum is not considered by ECSS as suitable for direct use in Invitation To Tender (ITT) or business agreements for space systems development.

Disclaimer

ECSS does not provide any warranty whatsoever, whether expressed, implied, or statutory, including, but not limited to, any warranty of merchantability or fitness for a particular purpose or any warranty that the contents of the item are error-free. In no respect shall ECSS incur any liability for any damages, including, but not limited to, direct, indirect, special, or consequential damages arising out of, resulting from, or in any way connected to the use of this Standard, whether or not based upon warranty, business agreement, tort, or otherwise; whether or not injury was sustained by persons or property or otherwise; and whether or not loss was sustained from, or arose out of, the results of, the item, or any services that may be provided by ECSS.

Published by: 	ESA Requirements and Standards Division

ESTEC, P.O. Box 299,

2200 AG Noordwijk

The Netherlands

Copyright: 	2010© by the European Space Agency for the members of ECSS

Change log

| ECSS-E-TM-10-20A 16 April 2010 | First issue |
| --- | --- |

Table of contents

Change log	3

Introduction	6

1 Scope	7

2 Normative references	8

3 Terms, definitions and abbreviated terms	9

3.1	Terms from other documents	9

3.2	Terms specific to the present document	9

3.3	Abbreviated terms	10

4 Provisions	11

4.1	Principles	11

4.1.1	Disciplines	11

4.1.2	Structure of the provisions	13

4.2	General provisions for product data exchange	14

4.2.1	Units for physical quantities	14

4.2.2	Media for product data exchange	14

4.2.3	Specific limitation for product data exchange to and from the software engineering discipline	15

4.3	Specific requirements for product data exchange	15

4.3.1	Source: System engineering	15

4.3.2	Source: Configuration management	15

4.3.3	Source: Information - documentation management	16

4.3.4	Source: Integrated logistic support	16

4.3.5	Source: Mission analysis	17

4.3.6	Source: Space environmental effects	17

4.3.7	Source: Human factors engineering	17

4.3.8	Source: Electronics	17

4.3.9	Source: Electrical power	17

4.3.10	Source: Electromagnetic compatibility	18

4.3.11	Source: Radiofrequency systems	18

4.3.12	Source: Electrical interfaces and interconnections	18

4.3.13	Source: Optical	18

4.3.14	Source: Mechanical – CAD	18

4.3.15	Source: Mechanical – Thermal	20

4.3.16	Source: Mechanical – Structures	20

4.3.17	Source: Mechanical – Mechanisms	21

4.3.18	Source: Mechanical – ECLS	21

4.3.19	Source: Mechanical – Propulsion	21

4.3.20	Source: Mechanical – Pyrotechnics	21

4.3.21	Source: Software	21

4.3.22	Communication engineering	22

4.3.23	Source: Control engineering	22

4.3.24	Source: Manufacture, assembly, integration and test (MAIT)	22

4.3.25	Source: Materials, mechanical parts and processes	22

4.3.26	Source: Operations engineering	22

Annex A (informative)  References to electronic copies	23

Bibliography	24

Tables

Table 4-1  List of applicable disciplines for product data exchange and their associated ECSS documents	12

Table 4-2  Data transfer from a system engineering representation	15

Table 4-3  Data transfer from a configuration management representation	15

Table 4-4  Data transfer from an informationdocumentation management representation	16

Table 4-5  Data transfer from an ILS representation	16

Table 4-6  Data transfer from an electronic system representation	17

Table 4-7  Data transfer from an electrical power representation	17

Table 4-8  Data transfer from a mechanical – CAD representation	19

Table 4-9  Data transfer from a mechanicalthermal representation	20

Table 4-10  Data transfer from a mechanicalstructures representation	21

Table 4-11  Data transfer from an electrical power representation	22

Introduction

This document forms part of the system engineering branch (ECSS-E-10) of the space engineering area of the ECSS System.

Reliable and easytouse exchange of computerinterpretable product data between all parties, disciplines and persons involved in space projects, is generally recognized as one of the important means to enable efficient and effective project execution. Product data in this context comprises all data which specifies one or more aspects of a product throughout its life cycle.

Reliable electronic exchange of product data cannot be achieved without the establishment and adoption of formal protocols by the parties exchanging the data. The present document aims to facilitate the establishment of a set of applicable product data exchange protocols between the parties participating in a space project.

Throughout the present document, reference is made to many parts of ISO 10303 “Industrial automation systems and integration – Product data representation and exchange”, more commonly known as the “Standard for the Exchange of Product model data (STEP)”. ISO 10303 is the most important, global, crossindustry set of standards for the exchange of product data. The objective of ISO 10303 is explained in the Introduction of ISO 10303-1:

“ISO 10303 is an International Standard for the computerinterpretable representation and exchange of product data. The objective is to provide a mechanism that is capable of describing product data throughout the life cycle of a product, independent of any particular system. The nature of this description makes it suitable not only for neutral file exchange, but also as a basis for implementing and sharing product databases and archiving.”

When reading the present document it is very helpful if the reader has access to information on the ISO 10303 suite of standards. This information can be obtained from the national standard bodies representing ISO or from ISO itself and on the worldwide web from SC4ONLINE. See Clause 2 and Annex A for reference details.

The actual exchange of product data is performed via one of the following processes:

- sending and receiving selfcontained exchange datasets in offline mode in the form of computer files via appropriate media (for example, computer networks, computer tape cartridges, or CDROMs);

- by sharing data online through standardized data access interfaces to data stores that are connected to a computer network.

Space project engineering is a multidisciplinary activity employing a wide range of technologies, with no one person able to master all of the disciplines at the level of expertise required to ensure a successful outcome. Consequently, resources from a number of engineering disciplines generally contribute to the engineering process, at least at the higher levels of complexity.

# Scope

This document describes the methods and protocols for the exchange of computerinterpretable product data in space projects. Product data is defined as all data that specifies one or more aspects of a product throughout its life cycle. This document describes which standard protocol to use in what data exchange context by making reference to existing external standards. It does not cover the contents of any product data exchange protocol itself.

Two categories of product data exchange are distinguished:

- exchange of data between alternative tools used within one discipline, where the tools share similar functionality, and

- exchange of data between tools of different disciplines.

For the second category, in most cases only a subset of the product data represented in one discipline can be transferred meaningfully to a representation in another discipline.

Explicitly not within this document’s scope are:

- identification or availability of software tools to perform product data exchange,

- data compression standards, and

- data encryption standards.

This document can be applied to all participants in space programmes and throughout the complete product life cycle. The participants include customers, suppliers and partners at all levels and from all disciplines.

# Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this ECSS Technical Memorandum. For dated references, subsequent amendments to, or revision of any of these publications do not apply. However, parties to agreements based on this ECSS Technical Memorandum are encouraged to investigate the possibility of applying the more recent editions of the normative documents indicated below. For undated references, the latest edition of the publication referred to applies.

| ECSS-ST-S-00-01 | ECSS system – Glossary of terms |
| --- | --- |

# Terms, definitions and abbreviated terms

## Terms from other documents

For the purposes of this document, the terms and definitions given in ECSS-S-ST-00-01 apply.

## Terms specific to the present document

- application

group of one or more processes creating or using product data

[ISO 10303-1]

- application protocol

part of ISO 10303 that specifies a complete and consistent data model satisfying the scope and information requirements for a specific application

[adapted from ISO 10303-1]

- conformance class

subset of an application protocol for which conformance may be claimed

[ISO 10303-1]

- product data

representation of information about a product in a formal manner suitable for communication, interpretation or processing by human beings or by computers

[ISO 10303-1]

## Abbreviated terms

For the purpose of this document, the abbreviated terms from ECSS-S-ST-00-01 and the following apply:

| Abbreviation | Meaning |
| --- | --- |
| AP | application protocol |
| BREP | boundary representation |
| CAD | computer aided design |
| CAE | computer aided engineering |
| CAM | computer aided manufacturing |
| CC | conformance class |
| CSG | constructive solid geometry |
| ECLS | environmental control and life support |
| FTP | file transfer protocol |
| IETF | Internet Engineering Task Force |
| RFC | request for comment |
| MAIT | manufacturing, assembly, integration and test |
| PDE | product data exchange |
| PDF | portable document format |
| PDM | product data management |
| SI | international system of units - système internationale des unités |
|  | - It is specified in ISO 31 and ISO 1000. |
| STEP | standard for the exchange of product model data |
| . | - STEP is the commonly known name for ISO 10303 |

# Provisions

## Principles

### Disciplines

This document describes the protocols to be used for product data exchange between all relevant combinations of disciplines. The following eight groups of disciplines are recognized:

- system engineering

- electricalelectronic

- mechanical

- software

- communications engineering

- control engineering

- production engineering

- operations engineering

Each of these disciplines consists of a number of “subdisciplines” which take care of specific aspects of the overall system that constitutes a complete space product. For a presentation of the engineering disciplines involved in a space project see ECSS-S-ST-00.

Table 4-1 lists the disciplines addressed in the present document for product data exchange, with their associated ECSS Standards.

Table 4-1  List of applicable disciplines for product data exchange and their associated ECSS documents

| Discipline | Associated disciplinespecific ECSS documents |
| --- | --- |
| System engineering | ECSS-E-ST-10 Space engineering – System engineering general requirements |
| Configuration and information management | ECSS-M-ST-40 Space project management – Configuration and information management |
| Integrated logistic support | ECSS-M-70 Space project management – Integrated logistic support |
| Mission analysis | ECSS-E-ST-10 Space engineering – System engineering general requirements |
| Space environmental effects | ECSS-E-ST-10-04 Space engineering – Space environment |
| Human factors engineering | ECSS-E-ST-10-11 Space engineering – Human factors engineering |
| Electronics | ECSS-E-ST-20 Space engineering – Electrical and electronic |
| Electrical power | ECSS-E-ST-20 Space engineering – Electrical and electronic |
| Electromagnetic compatibility | ECSS-E-ST-20 Space engineering – Electrical and electronic |
| Radiofrequency systems | ECSS-E-ST-50 Space engineering – Communications |
| Electrical interfaces and interconnections | ECSS-E-ST-20 Space engineering – Electrical and electronic |
| Optical | ECSS-E-HB-21 Space engineering – Optical guidelines |
|  | ECSS-E-ST-32 Space engineering – Structural general requirements |
| Mechanical – CAD | ECSS-E-ST-10 Space engineering – System engineering general requirements |
|  | ECSS-E-ST-32-08 Space engineering – Materials |
| Mechanical – Thermal | ECSS-E-ST-31 Space engineering – Thermal control general requirements |
| Mechanical – Structures | ECSS-E-ST-32 Space engineering – Structural general requirements |
| Mechanical – Mechanisms | ECSS-E-ST-33-01 Space engineering – Mechanisms |
| Mechanical – ECLS | ECSS-E-ST-34 Space engineering – ECLS |
| Mechanical – Propulsion | ECSS-E-ST-35 Space engineering – Propulsion general requirements |
| Mechanical – Pyrotechnics | ECSS-E-ST-33-11 Space engineering – Explosive systems and devices |
| Software a) | ECSS-E-ST-40 Space engineering – Software |
| Communications | ECSS-E-ST-50 Space engineering – Communications |
| Control engineering | ECSS-E-HB-60 Space engineering – Control engineering handbook |
|  | ECSS-E-ST-20 Space engineering – Electrical and electronic |
| Manufacture, assembly, integration and test (MAIT) | ECSS-E-ST-10-02 System engineering – Verification |
|  | ECSS-E-10-03 b) System engineering – Testing |
|  | ECSS-E-ST-70 Space engineering – Ground systems and operations |
|  | ECSS-M-70 Space project management – Integrated logistic support |
|  | ECSS-Q-ST-70 Space product assurance – Materials, mechanical parts and processes |
| Materials, mechanical parts and processes | ECSS-Q-ST-70 Space product assurance – Materials, mechanical parts and processes |
|  | ECSS-E-ST-10 Space engineering – System engineering general requirements |
|  | ECSS-E-ST-32-08 Space engineering – Materials |
| Operations engineering | ECSS-E-ST-70 Space engineering – Ground systems and operations |
| a) Exchange of software product data is limited to configuration items as defined in 4.2.3. b) To be replaced by ECSS-E-ST-10-03. |  |

### Structure of the provisions

The provisions for product data exchange in the present document are split into two categories:

- general provisions pertaining to all product data exchange;

- specific provisions pertaining to product data exchange in a given product data exchange context.

A product data exchange context is defined by three elements:

- a source discipline, which holds the source representation of the product data;

- a destination discipline, which receives the data and converts it into its target representation of the product data;

- the intended purpose for the transfer, which describes the product data to be transferred.

The source and the destination discipline can be the same, in case of exchange of data between two tools used for one discipline or in case of long term data archiving for one tool.

The general provisions are given in the clause 4.2.

The specific provisions are given as clauses under clause 4.3, and written in a structured way. For each discipline listed in Table 1 there is a level 3 clause (4.3.x) which designates the source discipline. Each source discipline clause contains a collection of applicable destination disciplines, referenced as 4.3.x.y. The body of all specific provision clauses adheres to the following template:

| Table (example) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| 4.3.x.y |  |  |  |  |

The destination disciplines include a sequence number between parentheses in order to uniquely identify multiple exchange possibilities between a source and a destination discipline for different product data classes.

## General provisions for product data exchange

### Units for physical quantities

          - Throughout all product data exchanges the units for all values of all physical quantities shall be explicitly specified in the protocol.

          - Throughout all product data exchanges SI units should be used for all values of physical quantities.

          - The conversion rule from the actually used unit to the equivalent SI unit shall be explicitly included in the protocol, in case the values of physical quantities are not specified in SI units.

          - The conversion of units should conform to ISO 31 and ISO 1000.

          - If the conversion rule deviates from ISO 31 or ISO 1000 the origin of the conversion rule shall be specified.

          - The applicable units and conversion rules shall be specified and agreed prior to starting exchange of data by the parties involved the data exchange.

- The specification is usually initiated by the sending party.

### Media for product data exchange

          - The media for transfer of the product data to be exchanged shall be agreed by the parties involved.

- Example: Suitable media are, for example, CDROM, 4 mm DAT cartridge, and FTP server connected to the Internet.

### Specific limitation for product data exchange to and from the software engineering discipline

For software configuration items or software products the present document only takes into account the exchange of the associated configuration control data (i.e. item and version identification, and possibly a functional description).

Software items are considered to be parts of a product, implementing specified functionality in software. Software items are part of a larger containing product.

For the exchange of actual software specification, analysis and design models as well as software implementations see ECSS-E-ST-40.

## Specific requirements for product data exchange

### Source: System engineering

          - To transfer product data from a system engineering representation to the destination representation and for the product data stated in Table 4-2, the standard reference specified in such a table shall be used.

Table 4-2  Data transfer from a system engineering representation

| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| --- | --- | --- | --- | --- |
| 4.3.1.1 | System engineering | Mechanical - CAD (1) | Physical system breakdown | ISO 10303-203 CC1 |

### Source: Configuration management

          - To transfer product data from a configuration management representation to the destination representations and for the product data stated in Table 4-3, the standard references specified in such a table shall be used.

Table 4-3  Data transfer from a configuration management representation

| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| --- | --- | --- | --- | --- |
| 4.3.2.1 | Configuration management | Mechanical – CAD (1) | PDM data configuration controlled design information without shape | ISO 10303-203 CC1 |
| 4.3.2.2 | Configuration management | Configuration management (1) | PDM data configuration controlled design information without shape | ISO 10303-203 CC1 |

### Source: Information - documentation management

          - To transfer product data from an informationdocumentation management representation to the destination representations and for the product data stated in Table 4 , the standard references specified in such a table shall be used.

          - In the case specified in Table 4-4, reference 4.3.3.1, the applicable CC shall be mutually agreed.

Table 4-4  Data transfer from an informationdocumentation management representation

| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| --- | --- | --- | --- | --- |
| 4.3.3.1 | Information – documentation management | Information – documentation management (1) | Technical data package | ISO 10303-232 |
| 4.3.3.2 | Information – documentation management | Information – documentation management (2) | Document for readonly access | PDF |

### Source: Integrated logistic support

          - To transfer product data from an integrated logistic support representation to the destination representations and for the product data stated in Table 4-5, the standard references specified in such a table shall be used.

- PDM data typically contains the product assembly tree, bill of materials, and release status of all configuration items or parts.

Table 4-5  Data transfer from an ILS representation

| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| --- | --- | --- | --- | --- |
| 4.3.4.1 | Integrated logistic support | Integrated logistic support (1) | PDM data: configuration controlled design information without shape | ISO 10303-232 CC1 |
| 4.3.4.2 | Integrated logistic support | Mechanical – CAD (1) | PDM data: configuration controlled design information without shape | ISO 10303-232 CC1 |

### Source: Mission analysis

The standard reference to be used to exchange data from a mission analysis representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Space environmental effects

The standard reference to be used to exchange data from a space environmental effects representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Human factors engineering

The standard reference to be used to exchange data from a human factors engineering representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Electronics

          - To transfer product data from an electronics representation to the destination representations and for the product data stated in Table 4-6, the standard references specified in such a table shall be used.

          - The applicable CC shall be mutually agreed.

Table 4-6  Data transfer from an electronic system representation

| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| --- | --- | --- | --- | --- |
| 4.3.8.1 | Electronic systems | Mechanical – CAD (1) | Mechanical interfaces and layout | ISO 10303-210 |

### Source: Electrical power

          - To transfer product data from an electrical power representation to the destination representations and for the product data stated in Table 4-7, the standard references specified in such a table shall be used.

          - In all the cases specified in Table 4-7, the applicable CC shall be mutually agreed.

Table 4-7  Data transfer from an electrical power representation

| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| --- | --- | --- | --- | --- |
| 4.3.9.1 | Electronic systems | Mechanical – CAD (1) | Mechanical interfaces and layout | ISO 10303-210 |

### Source: Electromagnetic compatibility

The standard reference to be used to exchange data from a electromagnetic compatibility representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Radiofrequency systems

The standard reference to be used to exchange data from a radiofrequency systems representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Electrical interfaces and interconnections

The standard reference to be used to exchange data from a electrical interfaces and interconnections representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Optical

The standard reference to be used to exchange data from a optical representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Mechanical – CAD

          - To transfer product data from a mechanicalCAD representation to the destination representations and for the product data stated in Table 4-8, the standard references specified in such a table shall be used.

Table 4-8  Data transfer from a mechanical – CAD representation

| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| --- | --- | --- | --- | --- |
| 4.3.14.1 | Mechanical - CAD | Configuration management (1) | PDM data: configuration controlled design information without shape | ISO 10303-203 CC1 |
| 4.3.14.2 | Mechanical - CAD | Integrated logistic support (1) | PDM data: configuration controlled design information without shape | ISO 10303-203 CC1 |
| 4.3.14.3 | Mechanical - CAD | Mechanical – CAD (1) | Configuration controlled design information and shapes represented by manifold surface models with topology | ISO 10303-203 CC4 |
| 4.3.14.4 | Mechanical - CAD | Mechanical – CAD (2) | Configuration controlled design information and shapes represented by facetted boundary-representation models | ISO 10303-203 CC5 |
| 4.3.14.5 | Mechanical - CAD | Mechanical – CAD (3) | Configuration controlled design information and shapes represented by advanced boundary-representation models | ISO 10303-203 CC6 |
| 4.3.14.6 | Mechanical - CAD | Mechanical – CAD (4) | Component design with 3D shape | ISO 10303-214 CC1 |
| 4.3.14.7 | Mechanical - CAD | Mechanical – CAD (5) | Assembly design with 3D shape | ISO 10303-214 CC2 |
| 4.3.14.8 | Mechanical - CAD | Mechanical – CAD (6) | Component drawings with wireframe or surface shape representation | ISO 10303-214 CC3 |
| 4.3.14.9 | Mechanical - CAD | Mechanical - CAD (7) | Assembly drawings with wireframe, surface or solid shape representation | ISO 10303-214 CC4 |
| 4.3.14.10 | Mechanical - CAD | Thermal analysis (1) | 3D surface shape models | ISO 10303-203 CC4 |
| 4.3.14.11 | Mechanical - CAD | Thermal analysis (2) | 3D explicit shape models with advanced BREP for thermal radiative analysis | ISO 10303-203 CC6 |
| 4.3.14.12 | Mechanical - CAD | Mechanical – Structures (1) | 3D explicit shape model with advanced BREP | ISO 10303-203 CC6 |
| 4.3.14.13 | Mechanical - CAD | Mechanical – Structures (2) | 3D CSG model | ISO 10303-214 CC4 |
| 4.3.14.14 | Mechanical - CAD | Ground systems and operations (1) | PDM data: configuration controlled design information without shape | ISO 10303-203 CC1 |
| 4.3.14.15 | Mechanical - CAD | Ground systems and operations (2) | 3D explicit shape model with advanced BREP for shape representation | ISO 10303-203 CC6 |

### Source: Mechanical – Thermal

          - To transfer product data from a mechanicalthermal representation to the destination representations and for the product data stated in Table 4-9, the standard references specified in such a table shall be used.

Table 4-9  Data transfer from a mechanicalthermal representation

| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| --- | --- | --- | --- | --- |
| 4.3.15.1 | Thermal analysis | Mechanical – CAD (1) | 3D surface shape model | ISO 10303-203 CC4 |
| 4.3.15.2 | Thermal analysis | Mechanical – CAD (2) | 3D CSG model | ISO 10303-214 CC4 |
| 4.3.15.3 | Thermal analysis | Thermal analysis (1) | Thermalradiative analysis model with basic geometry | STEP-TAS CC1 |
| 4.3.15.4 | Thermal analysis | Thermal analysis (2) | Thermalradiative analysis model with basic geometry and kinematic model | STEP-TAS CC2 |
| 4.3.15.5 | Thermal analysis | Thermal analysis (3) | Thermalradiative analysis model with basic and constructive geometry | STEP-TAS CC3 |
| 4.3.15.6 | Thermal analysis | Thermal analysis (4) | Thermalradiative analysis model with basic and constructive geometry and kinematic model | STEP-TAS CC4 |
| 4.3.15.7 | Thermal analysis | Thermal analysis (5) | Thermalradiative analysis model with basic geometry, kinematic model and space mission aspects | STEP-TAS CC5 |
| 4.3.15.8 | Thermal analysis | Thermal analysis (6) | Thermalradiative analysis model with basic and constructive geometry, kinematic model and space mission aspects | STEP-TAS CC6 |

### Source: Mechanical – Structures

          - To transfer product data from a mechanicalstructures representation to the destination representations and for the product data stated in Table 4-10, the standard references specified in such a table shall be used.

          - In the case specified in Table 4-10, reference 4.3.16.3, the applicable CC shall be mutually agreed.

Table 4-10  Data transfer from a mechanicalstructures representation

| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| --- | --- | --- | --- | --- |
| 4.3.16.1 | Mechanical - Structures | Mechanical – CAD (1) | 3D explicit shape model with advanced BREP for structural analysis | ISO 10303-203 CC4 |
| 4.3.16.2 | Mechanical - Structures | Mechanical – CAD (2) | 3D CSG model for structural analysis | ISO 10303-214 CC4 |
| 4.3.16.3 | Mechanical - Structures | Mechanical – Structures (1) | Linear static finite element analysis model | ISO 10303-209 |

### Source: Mechanical – Mechanisms

The standard reference to be used to exchange data from a mechanical mechanisms representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Mechanical – ECLS

The standard reference to be used to exchange data from a mechanicalECLS representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Mechanical – Propulsion

The standard reference to be used to exchange data from a mechanicalpropulsion representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Mechanical – Pyrotechnics

The standard reference to be used to exchange data from a mechanicalpyrotechnics representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Software

          - To transfer product data from a Software representation to the destination representations and for the product data stated in Table 4-11, the standard references specified in such a table shall be used.

Table 4-11  Data transfer from an electrical power representation

| Reference | Source representation | Destination representation | Product data to be transferred | Standard reference to apply |
| --- | --- | --- | --- | --- |
| 4.3.21.1 | Software | Configuration management (1) | PDM data: Configuration controlled design information without shape | ISO 10303-203 CC1 |
| 4.3.21.2 | Software | Integrated logistic support (1) | PDM data: Configuration controlled design information without shape | ISO 10303-203 CC1 |

### Communication engineering

The standard reference to be used to exchange data from a communication engineering representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Control engineering

The standard reference to be used to exchange data from a control engineering representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Manufacture, assembly, integration and test (MAIT)

The standard reference to be used to exchange data from a manufacture, assembly, integration and test (MAIT) representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Materials, mechanical parts and processes

The standard reference to be used to exchange data from a materials, mechanical parts and processes representation to any applicable discipline listed in Table 4-1 is presently under consideration.

### Source: Operations engineering

The standard reference to be used to exchange data from an operations engineering representation to any applicable discipline listed in Table 4-1 is presently under consideration.

- (informative) 
References to electronic copies

At the time of the publication of the present document, electronic copies of referenced documents can be found at:

| ISOwebsite | The official website of ISO. For electronic copies of documents http://www.iso.ch |
| --- | --- |
| SC4ONLINE | The official website of ISO Technical Committee 184 “Industrial automation systems and integration” / Subcommittee 4 “Industrial data”, which is responsible for the ISO 10303 (STEP) standardization. http://www.tc184-sc4.org |
| STEP-TAS | STEPbased Application Protocol – Thermal Analysis for Space – ESA/ESTEC http://www.estec.esa.int/thermal/tools/standards.html |
| PDF | PDF Reference, 3rd Edition. Adobe Portable Document Format, Version 1.4, December 2001, Adobe Systems Incorporated, ISBN 0–201–75839-3 (size of pdffile around 9 MB, 987 pages) http://partners.adobe.com/asn/acrobat/docs/File_Format_Specifications/PDFReference.pdf |

Bibliography

| ECSS-S-ST-00 | ECSS system – Description, implementation and general requirements |
| --- | --- |
| ECSS-E-ST-10 | Space engineering - System engineering general requirements |
| ECSS-E-ST-10-02 | Space engineering - Verification |
| ECSS-E-10-03 | Space engineering - Testing |
| ECSS-E-ST-10-04 | Space engineering - Space environment |
| ECSS-E-ST-10-11 | Space engineering - Human factors engineering |
| ECSS-E-ST-20 | Space engineering - Electrical and electronic |
| ECSS-E-ST-31 | Space engineering - Thermal control general requirements |
| ECSS-E-ST-32 | Space engineering - Structural general requirements |
| ECSS-E-ST-32-08 | Space engineering - Materials |
| ECSS-E-ST-33-01 | Space engineering - Mechanisms |
| ECSS-E-ST-33-11 | Space engineering – Explosive systems and devices |
| ECSS-E-ST-34 | Space engineering - Environmental control and life support (ECLS) |
| ECSS-E-ST-35 | Space engineering - Propulsion general requirements |
| ECSS-E-ST-40 | Space engineering - Software |
| ECSS-E-ST-50 | Space engineering - Communications |
| ECSS-E-ST-70 | Space engineering - Ground systems and operations |
| ECSS-M-ST-40 | Space project management - Configuration and information management |
| ECSS-M-70 | Space project management - Integrated logistic support |
| ECSS-Q-ST-70 | Space product assurance - Materials, mechanical parts and processes |
| ECSS-E-HB-21 | Space engineering – Optical guidelines |
| ECSS-E-HB-60 | Space engineering - Control engineering handbook |
| ISO 31 | Quantities and units |
| ISO 1000 | SI units and recommendations for the use of their multiples and of certain other units |
| ISO 10303-1:1994 | Industrial automation systems and integration - Product data representation and exchange - Part 1: Overview and fundamental principles |
| ISO 10303-203:1994 | Industrial automation systems and integration - Product data representation and exchange - Part 203: Application protocol: Configuration controlled 3D designs of mechanical parts and assemblies |
| ISO 10303-209:2001 | Industrial automation systems and integration - Product data representation and exchange - Part 209: Application protocol: Composite and metallic structural analysis and related design |
| ISO 10303-210:2001 | Industrial automation systems and integration - Product data representation and exchange - Part 210: Application protocol: Electronic assembly, interconnection, and packaging design |
| ISO 10303-212:2001 | Industrial automation systems and integration - Product data representation and exchange - Part 212: Application protocol: Electrotechnical design and installation |
| ISO 10303-214:2001 | Industrial automation systems and integration - Product data representation and exchange - Part 214: Application protocol: Core data for automotive mechanical design processes |
| ISO 10303-232:2002 | Industrial automation systems and integration - Product data representation and exchange - Part 232: Application protocol: Technical data packaging core information and exchange |
| STEP-TAS | STEP-based Application Protocol - Thermal Analysis for Space - ESA/ESTEC) |
| PDF Reference, 3rd Edition 1) | PDF Reference, 3rd Edition, Adobe Portable Document Format, Version 1.4 |

) See download information in Annex A.