- (normative)
OTS item evaluation dossier - DRD

  - DRD identification

    - Requirement identification and source document

This DRD is called from requirement ECSS-Q-ST-20-10, requirement 5.1.3a.

    - Purpose and objective

The OTS item evaluation dossier provides a detailed incremental overview of the OTS item since the initial selection up to the OTS item qualification and approval for its use in the specific project.

In order to avoid loss of information or data, at the end of the characterization phase, when the final candidate has been selected, its evaluation dossier is formalized in a project document (report).

  - Expected response

    - Scope and contents

          - The OTS item evaluation dossier contents shall contain as a minimum:

            - Trade offs results.

            - Manufacturer datasheets, documentation and information data from / on the manufacturer, waivers, historical data.

            - Programmatic data.

- For example: Cost, procurement time, export limitation.

            - Compliance Matrix to the Equipment specification.

            - OTS item environmental characteristics.

- For example: Thermal, mechanical, EMI, EMC.

            - OTS Item status evaluation matrix (refer to provided example in point B.2.2).

            - RAMS information.

- For example: FMEA, Safety assessment, internal redundancy, reliability figures.

            - Safety Data Sheets and or Safety technical data.

            - Risk assessment.

            - PMP and EEE information and result of in-depth inspection.

            - Modifications needed and modifications implemented including PADs for self procured EEE parts.

            - Procurement document.

            - Operation and logistic information.

- For example: Life, storage, handling instruction, user’s manual.

            - All the data/information / analyses / test results obtained during the characterization phase.

            - All the data / information / analyses / test results obtained during the qualification phase, including any non conformance report.

            - European CE marking or equivalent internationally recognized marking.

    - Special remarks

          - The OTS item evaluation dossier shall be a living document in a form of a folder(s), established for each OTS item candidate.

    - Examples of OTS Item Status Evaluation Matrix

| OTS item status evaluation matrix |  |  |  |  |
| --- | --- | --- | --- | --- |
| Product name : |  | Supplier : |  |  |
| Part number (P/N) : |  | Country : |  |  |
| Documentation | OTS Item Original Documentation Title /Reference/Issue/Revision |  | Project Documentation Title /Reference/Issue/Revision | Difference |
| Equipment specification |  |  |  |  |
| Configuration item data list (CIDL) |  |  |  |  |
| Assembly Drawing |  |  |  |  |
| Interface Data sheet (IDS) |  |  |  |  |
| Interface Control Documents (including drawings) |  |  |  |  |
| Declared processes list (DPL) |  |  |  |  |
| Declared materials list (DML) |  |  |  |  |
| Declared Mechanical parts list |  |  |  |  |
| Part Approval Documents (PAD) |  |  |  |  |
| EEE part list (EEE PL) |  |  |  |  |
| Cleanliness data |  |  |  |  |
| User Manual |  |  |  |  |
| Qualification Status List (QSL) |  |  |  |  |
| Mechanical Analysis |  |  |  |  |
| Thermal analysis |  |  |  |  |
| Radiation analysis |  |  |  |  |
| EMC/ESD analysis |  |  |  |  |
| Reliability analysis |  |  |  |  |
| FMECA |  |  |  |  |
| Safety analysis/safety datasheets/safety procedures |  |  |  |  |
| Derating analysis |  |  |  |  |
| Parts Stress Analysis |  |  |  |  |
| Worst case Analysis |  |  |  |  |
| Verification Control Document |  |  |  |  |
| Test Reports |  |  |  |  |
| Inspection Reports |  |  |  |  |
| Certificate of Conformance (CoC) w.r.t. International Standards |  |  |  |  |
| NCRs/RFWs |  |  |  |  |
| Acceptance data package |  |  |  |  |

              - : Example of an OTS item status evaluation matrix

| COMPARISON OF EXISTING OTS ITEM QUALIFICATION Vs PROJECT REQUIREMENTS |  |  |  |
| --- | --- | --- | --- |
| Product name : |  | Supplier: |  |
| Part number (P/N) : |  | Country : |  |
| Qualification Parameters | OTS Item | Project requirements | Remarks |
| Random Vibration Normal to Mounting Surface | 20 - 100 Hz 6 dB/oct. 100 - 400 Hz 0,4 g2/Hz 400 - 2000 Hz -6 dB/oct. Total: 16,2 g(r.m.s.) (2 min/1 min per axis) | 20-100Hz 3 dB/oct. 100-300 Hz 1.42 g2/Hz 300-2000Hz -6 dB/oct. Total: (2min/1min per axis) | Proposed for EQM testing: 10 to 50 Hz: +6 dB/oct 50 to 355 Hz: 0,8 g2/Hz 355 to 1000 Hz: -4 dB/oct 1000 Hz to 1500 Hz: 0,2 g2/Hz 1500 Hz to 2000 Hz: -7 dB/oct Overall: 26 g(r.m.s.) |
| Random Vibration Parallel to Mounting Surface | 20 - 100 Hz 6 dB/oct. 100 – 400 Hz 0,15 g2/Hz 400 – 2000 Hz -6 dB/oct. Total: 9,9 g(r.m.s.) (2min/1min per axis) | 20 – 100 Hz 3 dB/oct. 100 - 300Hz 0,63 g2/Hz 300 - 2000Hz 6 dB/oct. Total: 9,4 g(r.m.s.) (1min/3min per axis) | Proposed for EQM testing: 10 Hz to 2000 Hz: 0.2 g2/Hz Overall: 19, 9 g(r.m.s.) |
| Sine Vibration (all 3 Axis | 5 -20 Hz +/- 9,3 mm 20 - 75 Hz 15,0 g 75 - 100 Hz 6 g (2/4 octaves per min) | out-of-plane: 5 - 20Hz 15 mm 20 - 100 Hz 24.0 g in plane: 5 - 20 Hz: 9.9 mm 20 - 100 Hz: 16 g (2/4 octaves per min) | Proposed for EQM testing: 12,5 mm up to 20 Hz 20g to 100Hz |

              - : Example of an comparison of existing OTS item qualification vs project requirements

OTS item dedicated data summary

| Name : |  |
| --- | --- |
| Qualification status: Q = Qualified, I = Qualification In progress, TBQ = To Be Qualified |  |
| In case of I or TBQ status, to precise the foreseen schedule and the open activities: |  |
| Development models to be manufactured: |  |
| Qualification plan to be implemented: |  |
| Design reviews to be hold: |  |
| Suppliers: |  |
| Equipment category: |  |
| Comments: |  |

              - : Example of OTS item dedicated data summary

- (informative)
ECSS-DRD Informational Reference

| Document | ECSS DRD Reference |
| --- | --- |
| Technical requirements specification (TA) | ECSS-E-ST-10-06, Annex A |
| C&CCP: Cleanliness and Contamination Control Plan | ECSS-Q-ST-70-01, Annex B |
| DCL: Declared Components List | ECSS-Q-ST-60, Annex B |
| DML: Declared Materials List | ECSS-Q-ST-70, Annex B |
| DPL: Declared Process List | ECSS-Q-ST-70, Annex D |
| DMPL: Declared Mechanical Parts List | ECSS-Q-ST-70, Annex C |
| PAD: Part Approval Document | ECSS-Q-ST-60, Annex D |
| Qualification Test Report | ECSS-Q-ST-70-10, Annex B |
| Certificate of Conformity | ECSS-Q-ST-20, Annex D |
| TRPT: Test Report | ECSS-E-ST-10-02, Annex C |
| VCD: Verification Control Document (including Verification Matrix) | ECSS-E-ST-10-02, Annex B |
| NCR Status List | ECSS-Q-ST-10-09, Annex B |
| List of RFDs and RFWs | ECSS-M-ST-40, Annex I/J |
| QSL: Qualification Status List | ECSS-Q-ST-10, Annex B |
| CIDL: Configuration Item Data List | ECSS-M-ST-40, Annex C |
| FMEA/FMECA Report: Failure Modes Effects (& Criticality) Analysis | ECSS-Q-ST-30-02, Annex A |
| WCCPA: Worst Case (circuit performance) Analysis | ECSS-Q-ST-30, Annex J |
| EMC Report / Analysis (= EMEVR: Electromagnetic Effects Verification Report) | ECSS-E-ST-20, Annex C |
| Radiation Test Report | ECSS-Q-ST-70-06, Annex C |
| Safety Analysis Report | ECSS-Q-ST-40, Annex D |
| Inspection Report | ECSS-E-ST-10-02, Annex E |

Bibliography

| ECSS-S-ST-00 | ECSS system - Description, implementation and general requirements |
| --- | --- |
| ECSS-E-ST-10-06 | Space engineering - Technical requirements specification |
| ECSS-M-ST-40 | Space project management - Configuration and information management |
| ECSS-Q-ST-10 | Space product assurance - Product assurance management |
| ECSS-Q-ST-10-09 | Space product assurance - Nonconformance control system |
| ECSS-Q-ST-30 | Space product assurance - Dependability |
| ECSS-Q-ST-30-02 | Space product assurance - Failure modes, effects (and criticality) analysis (FMEA/FMECA) |
| ECSS-Q-ST-40 | Space product assurance - Safety |
| ECSS-Q-ST-70-01 | Space product assurance - Cleanliness and contamination control |
| ECSS-Q-ST-70-06 | Space product assurance - Particle and UV radiation testing for space materials |
| ECSS-Q-ST-70-10 | Space product assurance - Qualification of printed circuit boards |