- (normative)
HSIA form - DRD

  - DRD identification

    - Requirement identification and source document

This DRD is called from ECSS-Q-ST-30-02, requirement 7.2h.

    - Purpose and objective

The purpose of the HSIA form is to document the analysis performed in a tabular form.

The HSIA check list is an aid for performing the analysis, see Annex I.

  - Expected response

    - Scope and content

- Subsystem or equipment

          - The HSIA form shall contain the identification of subsystem or equipment submitted to HSIA.

- HSIA sheet number

          - The HSIA form shall contain the HSIA running sheet number.

- FMEA/FMECA reference

          - The HSIA form shall contain the identification of the reference number of the failure mode in the design FMEA/FMECA.

- Failure mode

          - The HSIA form shall contain a summary of failure mode description.

- RB/TS reference

          - The HSIA form shall contain a reference to the software specification used for the HSIA (number, issue).

- Identification of parameters used to trigger the software

          - The HSIA form shall contain identification of the information processed by the software to notify the presence of the failure or initiate an isolation or corrective action in response.

          - The HSIA form shall contain the identification of corresponding health signal (health signal = result of comparison between detected and reference values).

- RB/TS requirement number for S/W triggering

          - The HSIA form shall contain the requirement number in the RB/TS corresponding to the information at D.2.1<6>.

- Description of software (S/W) action

          - The HSIA form shall contain a summary of the actions specified in RB/TS which are provided to negate the effects of or isolate the failure (isolation/recovery).

- RB/TS requirement number for S/W action

          - The HSIA form shall contain the requirement number in the RB/TS corresponding to the information at  D.2.1<8>.

- Description of the effect of the S/W action on the product functionality

          - The HSIA form shall contain a summary of the effects of the actions taken by S/W (as described in RB/TS) on the functions of the product and on interfacing items.

- Identified adverse effects on hardware (H/W)

          - The HSIA form shall contain a description of any identified adverse effect.

- Examples of adverse effects are overstress of H/W, or failure propagation.

- Assessment of the S/W action

          - The HSIA form shall contain an assessment of the S/W action.

- The answer “yes” summarizes that the S/W action on the product functionality is conforming to the FDIR requirements (where applicable) and the S/W action is acceptable for the product functioning. 	
In case of answer “no”, recommendations are reported in  D.2.1<13>.

- Recommendations

          - The HSIA form shall contain recommendations in case of insufficient S/W actions or in case of adverse effect on H/W.

- Remarks:

          - The HSIA form shall contain any additional remark where relevant.

    - Example of HSIA form

Figure D-1 gives an example of HSIA form.

| HARDWARESOFTWARE INTERACTION ANALYSIS (HSIA) |  |
| --- | --- |
| 1. Subsystem/Equipment: | 2. HSIA sheet number: |
| 3. FMEA/FMECA reference: | 4. Failure mode: |
| 5. RB/TS reference: |  |
| 6. Identification of parameters used to trigger the S/W action: | 7. RB/TS requirement number for S/W triggering: |
| 8. Description of S/W action: | 9. RB/TS requirement number for S/W action: |
| 10. Description of the effects of the S/W action on the H/W: | 11. Identified adverse effect on H/W |
| 12. Assessment of S/W action: Is the S/W action as expected? yes/no |  |
| 13. Recommendations: |  |
| 14. Remarks |  |

              - : Example of HSIA form

    - HSIA integrated in FMEA/FMECA worksheet

          - In case the HSIA is provided inside the FMEA/FMECA, the FMEA/FMECA worksheet shall be completed as follows:

            - add S/W Specification reference in the Reference document;

            - in each completed column: for each failure mode where software is involved enter “S/W”;

            - local/end effect: add points 10 and 11 of HSIA form;

            - failure detection: add points 6 and 7 of HSIA form;

            - recovery or compensation: add points 8 and 9 of HSIA form;

            - recommendation: add points 12 and 13 of HSIA form.