Space product assurance

ECSS Secretariat

ESA-ESTEC

Requirements & Standards Division

Noordwijk, The Netherlands

Derating - EEE components

Foreword

This Standard is one of the series of ECSS Standards intended to be applied together for the management, engineering and product assurance in space projects and applications. ECSS is a cooperative effort of the European Space Agency, national space agencies and European industry associations for the purpose of developing and maintaining common standards. Requirements in this Standard are defined in terms of what shall be accomplished, rather than in terms of how to organize and perform the necessary work. This allows existing organizational structures and methods to be applied where they are effective, and for the structures and methods to evolve as necessary without rewriting the standards.

This Standard has been prepared and maintained under the authority of the Space Components Steering Board (SCSB) in partnership with the European Space Agency (ESA), national space agencies (NSAs) and the European space industry, reviewed by the ECSS Product Assurance Panel and approved by the ECSS Steering Board.

The end-of-life parameter drifts are not covered by this Standard.

Disclaimer

ECSS does not provide any warranty whatsoever, whether expressed, implied, or statutory, including, but not limited to, any warranty of merchantability or fitness for a particular purpose or any warranty that the contents of the item are error-free. In no respect shall ECSS incur any liability for any damages, including, but not limited to, direct, indirect, special, or consequential damages arising out of, resulting from, or in any way connected to the use of this Standard, whether or not based upon warranty, business agreement, tort, or otherwise; whether or not injury was sustained by persons or property or otherwise; and whether or not loss was sustained from, or arose out of, the results of, the item, or any services that may be provided by ECSS.

Published by: 	ESA Requirements and Standards Division

ESTEC, P.O. Box 299,

2200 AG Noordwijk

The Netherlands

Copyright: 	2008 © by the European Space Agency for the members of ECSS

Change log

| ECSS-Q-60-11A 7 September 2004 | First issue |
| --- | --- |
| ECSS-Q-30-11A 24 April 2006 | Second issue |
| ECSS-Q-30-11B | Never issued |
| ECSS-Q-ST-30-11C 31 July 2008 | Third issue Minor editorial changes: - Document converted to new ECSS template, - Bibliography updated (reference to ECSS-S-ST-00 was added and reference to ECSS-E-10-12 and ECSS-Q-30 were deleted because not called from the text) |

Table of contents

Change log	3

Introduction	6

1 Scope	7

2 Normative references	8

3 Terms, definitions and abbreviated terms	9

3.1	Terms from other standards	9

3.2	Terms specific to the present standard	9

3.3	Abbreviated terms	10

4 User responsibility	12

5 Derating	13

5.1	Overview	13

5.2	Principles of derating	13

5.3	Applicability and component selection	14

5.4	Derating parameters	15

5.5	Additional rules and recommendations	16

6 Tables for load ratios or limits	17

6.1	Overview	17

6.2	Capacitors: ceramic – family-group code: 01-01 and 01-02	18

6.3	Capacitors: solid tantalum – family-group code: 01-03	19

6.4	Capacitors: non-solid tantalum – family-group code: 01-04	20

6.5	Capacitors: film – family-group code: 01-05	21

6.6	Capacitors: glass and porcelain – family-group code: 01-06	22

6.7	Capacitors: mica and reconstituted mica – family-group code: 01-07	23

6.8	Capacitors: feedthrough – family-group code: 01-10	24

6.9	Capacitors: semiconductor technology (MOS type) –  family-group code: 01-11	25

6.10	Capacitors: miscellaneous (variable capacitors) – family-group code: 01-99	26

6.11	Connectors – family-group code: 02-01, 02-02, 02-03, 02-07 and 02-09	27

6.12	Connectors RF – family-group code: 02-05	28

6.13	Piezo-electric devices: crystal resonator – family-group code: 03-01	29

6.14	Diodes – family-group code: 04-01, 04-02, 04-03, 04-04, 04-06, 04-08, 04-10 to 04-13 and 04-14	30

6.15	Diodes: RF/microwave-PIN – family-group code: 04-05, 04-15, 04-16 and 04-17	31

6.16	Feedthrough filters – family-group code: 05-01	32

6.17	Fuses: Cermet (metal film on ceramic) – family-group code: 06-01	33

6.18	Inductors and transformers – family-group code: 07-01 to 07-03	34

6.19	Integrated circuits: logic – family-group code: 08-10, 08-20, 08-21, 08-29 to 08-42, and 08-80	35

6.20	Integrated circuits: non-volatile memories – family-group code: 08-22, 08-23 and 08-24	36

6.21	Integrated circuits: linear – family-group code: 08-50 to 08-60 and 08-69	37

6.22	Integrated circuits: linear converters – family-group code: 08-61 and 08-62	38

6.23	Integrated circuits: MMICs – family-group code: 08-95	39

6.24	Integrated circuits: miscellaneous – family-group code: 08-99	40

6.25	Relays and switches – family-group code: 09-01, 09-02 and 16-01	41

6.26	Resistors – family-group code: 10-01 to 10-11	43

6.27	Thermistors – family-group code: 11-01 to 11-03	45

6.28	Transistors: bipolar – family-group code: 12-01 to 12-04 and 12-09	46

6.29	Transistors: FET – family-group code: 12-05 and 12-06	47

6.30	Transistors: RF: bipolar – family-group code: 12-10 and 12-13	48

6.31	Transistors: RF: FET – family-group code: 12-12, 12-14, 12-15(FET) and 12-16(FET)	49

6.32	Wires and cables – family-group code: 13-01 to 13 03	50

6.33	Opto-electronics – family-group code: 18-01 to 18-05	51

6.34	RF passive components: family-group-code: 30-01, 30-07, 30-09, 30-10 and 30-99	52

6.35	Fibre optic components: fibre and cable: family-group-code: 27-01	53

6.36	Hybrids	54

Annex A (informative) Family and group codes	55

Bibliography	59

Figures

Figure 5-1: Parameter stress versus strength relationship	14

Introduction

This Standard specifies derating requirements applicable to electronic, electrical and electromechanical components.

Derating is a long standing practice applied to components used on spacecrafts. Benefits of this practice are now proven, but for competitiveness reasons, it becomes necessary to find an optimized reliability. Too high a derating can lead to over-design, over-cost and over-sizing of components, the direct consequence being excess volume and weight. The aim is to obtain reliable and high performance equipment without over-sizing of the components. For this reason and if possible, this Standard provides derating requirements depending on mission duration and mean temperature, taking into account demonstrated limits of component capabilities.

# Scope

This Standard applies to all parties involved at all levels in the realization of space segment hardware and its interfaces.

The objective of this Standard is to provide customers with a guaranteed performance and reliability up to the equipment end-of-life. To this end, the following are specified:

- Load ratios or limits to reduce stress applied to components;

- Application rules and recommendations.

This standard may be tailored for the specific characteristics and constraints of a space project, in accordance with ECSS-S-ST-00.

# Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this ECSS Standard. For dated references, subsequent amendments to, or revisions of any of these publications do not apply. However, parties to agreements based on this ECSS Standard are encouraged to investigate the possibility of applying the most recent editions of the normative documents indicated below. For undated references the latest edition of the publication referred to applies.

| ECSS-S-ST-00-01 | ECSS system – Glossary of terms |
| --- | --- |
| ECSS-Q-ST-60 | Space product assurance – Electrical, electronic and electromechanical (EEE) components |

# Terms, definitions and abbreviated terms

## Terms from other standards

For the purpose of this Standard, the terms and definitions from ECSS-ST-00-01 apply.

## Terms specific to the present standard

- ambient temperature

temperature surrounding a component

- case temperature

temperature on the component package surface

- derating

intentional reduction in a parameter rating of a component in order to increase its useful life in terms of drift and reliability

- hot spot temperature

highest measured or predicted temperature within any component

- junction temperature

highest measured or predicted temperature at the junction within a semiconductor or micro-electronic device

- Predicted temperature can be taken as Tcase + thermal resistance between junction and case times actual power (Watt) of the device.

- load ratio

permissible operating level after derating has been applied; given as a percentage of a parameter rating

- operating conditions

parameter stress and environment (temperature, vibration, shock and radiation) in which components are expected to operate

- performance

operation of a component or an equipment with respect to specified criteria

- RadPack

package designed to provide some form of radiation protection

- rating

maximum parameter value specified and guaranteed by the component manufacturer and component procurement specification

- Rating is considered as a limit not to be exceeded during operation and constitutes in most cases the reference for derating.

## Abbreviated terms

For the purpose of this Standard, the abbreviated terms from ECSS-S-ST-00-01 and the following apply:

| Abbreviation | Meaning |
| --- | --- |
| AC | alternating current |
| A/D | analog to digital |
| AWG | American wire gauge |
| C | capacitance |
| DC | direct current |
| EMC | electromagnetic compatibility |
| EPPL | European preferred parts list |
| EOL | end-of-life |
| ESCC | European Space Component Coordination |
| ESR | equivalent series resistance |
| f | frequency |
| FET | field effect transistor |
| GaAs | gallium arsenide |
| GEO | geostationary orbit |
| ISO | International Organization for Standardization |
| LED | light emitting diode |
| LEO | low Earth orbit |
| MEO | medium Earth orbit |
| MOS | metal on silicon |
| MIL (spec) | specification of the US Department of Defense |
| NASA | National Aeronautics and Space Administration |
| P | power |
| RadHard | radiation hardened |
| Ri | insulation resistance |
| SEE | single event effect |
| SEBO | single event burn-out |
| SEGR | single event gate rupture |
| Si | silicon |
| SOA | safe operating area |
| Tj | junction temperature |
| Top | operating temperature |
| VCE | collector-emitter voltage |

# User responsibility

          - The user of this Standard shall verify that the ordered assurance level of procured components is compatible with the intended application.

# Derating

## Overview

The term derating refers to the intentional reduction of electrical, thermal and mechanical stresses on components to levels below their specified rating. Derating is a means of extending component life, increasing reliability and enhancing the end-of-life performance of equipment.

Derating participates in the protection of components from unexpected application anomalies and board design variations.

The load ratios or limits given in clause 6 were derived from information available at the time of writing this Standard and do not preclude further derating for specific applications.

This Standard also defines how to handle transients.

## Principles of derating

The component parameter strength defines the limits and the performance component technology in the particular application and varies from manufacturer to manufacturer, from type to type, and from lot to lot and can be represented by a statistical distribution. Likewise, component stress can be represented by a statistical distribution. Figure 5-1 illustrates the strength of a component and the stress applied at a given time, where each characteristic is represented by a probability density function.

A component operates in a reliable way if its parameter strength exceeds the parameter stress. The designer should ensure that the stress applied does not exceed the component parameter strength. This is represented by the intersection (shaded area) in Figure 5-1. The larger the shaded area, the higher the possibility of failure becomes.

There are two ways, which may be used simultaneously, in which the shaded area can be decreased:

- Decrease the stress applied (which moves the stress distribution to the left).

- Increase the component parameter strength (by selecting over-sized components) thereby moving the strength distribution to the right.

The goal is to minimize the stress-to-strength ratio of the component. Derating moves the parameter stress distribution to the left while the selection processes applied to the components for space applications contribute to moving the parameter strength distribution to the right. The selection processes also reduce the uncertainty associated with the component parameter strength.

Derating reduces the probability of failure, improves the end-of-life performance of components and provides additional design margins.

Another effect of derating is to provide a safety margin for design. It allows integrating parameter distribution from one component to another, and from one procurement to another.

Figure 5-1: Parameter stress versus strength relationship

## Applicability and component selection

### Overview

This Standard applies to all components, selected for space applications, that are used for a significant duration. The meaning of “significant duration” is a period that contributes to the component life, for instance, one month is considered to be a significant duration. These requirements apply to screened components procured in accordance with approved space specifications.

This Standard only applies to approved components for which quality was proven after rigorous testing in accordance with ECSS-Q-ST-60.

### Requirements

          - Derating shall be applied in consideration of temperature limits recommended.

          - The derating requirements of this Standard shall not be used as a justification to upgrade the quality level of components.

          - The derating requirements shall be taken into account at the beginning of the design cycle of an equipment for any consequential design trade-off to be made. Specific attention shall be paid to, for example, breadboards and engineering models where parameter derating was not considered.

          - Component families and groups excluded in this Standard are due to the lack of experimental data and failure history. For these components, the user shall consult a component design and reliability specialist to apply the requirements of this Standard.

          - Components may be excluded from this Standard if they are used for short durations of less than one month provided the device ratings are not exceeded; for example, components used in solar generator deployment systems, redundancy commutation and launchers (except in some specific cases, noted family by family). In these cases, the designer shall ensure that the applied stress level does not exceed the component maximum rating.

          - In the same way, during a test (e.g. qualification and EMC), derating requirements are applied on operating conditions and not on test conditions, even if during some periods, the temperature range reached is higher than the derated value. However, the specified maximum ratings shall not be exceeded.

          - Derating shall be applied on normal operating conditions and not on the fault condition.

          - Where components are required to operate in protection mode or in fail-safe mode in order to prevent failure propagation (e.g. short-circuit protection), the components concerned shall meet the derating requirements and application rules when performing the protection or fail-safe function under the worst failure case (i.e. highest stress applied to the components that can last throughout the mission).

## Derating parameters

### Overview

Derating requirements are provided in clause 6 for each component family.

For each category, the parameters to be derated are identified. The main parameters to be derated are:

- junction or case temperature at maximum operating conditions;

- power (rating, dissipation);

- voltage;

- current.

The parameters to be derated depend on component type.

A stress balancing concept offers flexibility between one stress versus another (voltage and temperature). In some cases, e.g. resistors, derating has a direct impact on component performance.

### Requirements

          - If ratings are provided for transients or surge conditions, the same derating figures as for steady-state equivalent parameters shall be used.

          - If ratings are not provided for transient or surge conditions, then it shall be assured that the transient or surge values are below the steady-state specified maximum ratings.

          - Repetitive transients, e.g. in a switching DC/DC converter operation, and clock transmission lines, shall not exceed the derated steady-state values, superseded by no more than 90 % of the rated manufacturer values if not compatible.

          - Where radiation sensitive components are identified, it is the user’s responsibility that the chosen component technologies are suitable and that the mitigation factors, such as shielding, meet the customer’s requirement. The electrical derating shall be in accordance with this Standard.

## Additional rules and recommendations

### Overview

In addition to strict derating requirements, some application rules and recommendations are given in this Standard to achieve the suitable reliability.

### Requirements

          - For hybrids, individual components shall be in conformance with their respective derating rules.

# Tables for load ratios or limits

## Overview

This clause provides the load ratios or limits.

They are also available on the World Wide Web at the following address:

https://escies.org

Abbreviations used in the tables are explained in clause 3.

Annex A contains a complete listing of the family and group codes for parts that are referred to in this Standard.

## Capacitors: ceramic – family-group code: 01-01 and 01-02

### General

          - The capacitor stress sum value of steady-state voltage, AC voltage shall not exceed the load ratios specified hereunder. For transients refer to clause 5.4.

          - Multilayer capacitors with a DC voltage rating less than 100 V may be used in low voltage (less than 10 V) continuous applications provided they have been submitted to a low voltage (1,5 V) 85 % humidity at 85 C test or other approved method.

          - Physical limitation for type II: maximum surge dV/dt approximate to 100 V/s. This should be verified with the manufacturer.

          - Internal heating due to ESR can increase ageing and should be taken into account by applying a margin in temperature. Where ESR is not known at the frequency of a ripple current, an extrapolation of the ESR value and resonance (from manufacturer’s or test data) should be made where possible.

### Derating

## Capacitors: solid tantalum – family-group code: 01-03

### General

          - The capacitor stress sum value of steady-state voltage and AC voltage shall not exceed the load ratio specified hereunder. For transients refer to clause 5.4.

          - 100 % surge current screening shall be applied for all surface mounted capacitors types.

          - Surge current shall be derated to 75 % of the Isurge max. Isurge max is defined as Vrated/(ESR+Rs). Vrated is the maximum rated voltage, ESR is the maximum specified value and Rs is the value of series resistance specified in the circuit for surge current testing as defined in the applicable procurement specification.

          - Reverse voltage shall not exceed 75 % of the manufacturer’s specified maximum value for the reverse voltage.

          - Ripple power shall never exceed 50 % of the manufacturer’s specified maximum value.

          - Internal heating due to ESR can increase ageing and should be taken into account by applying a margin in temperature. Where ESR is not known at the frequency of a ripple current, an extrapolation of the ESR value and resonance (from manufacturer’s or test data) should be made where possible.

### Derating

## Capacitors: non-solid tantalum – family-group code: 01-04

### General

          - Reverse voltage shall not exceed 75 % of the manufacturer’s specified maximum value for the reverse voltage.

          - Manufacturer’s ratings for ripple power or current shall never be exceeded.

          - Internal heating due to ESR can increase ageing and should be taken into account by applying a margin in temperature. Where ESR is not known at the frequency of a ripple current, an extrapolation of the ESR value and resonance (from manufacturer’s or test data) should be made where possible.

### Derating

## Capacitors: film – family-group code: 01-05

### General

          - Self healing requirements for polyethylene terepthalate (PET, polyester) film:

            - To have a clearing process in any case, the minimum energy available shall be greater than the manufacturer’s specified value (typical self healing energy is 5 μJ minimum).

            - To prevent any risk of destruction during self healing, capacitor banks shall not provide energy greater than 15 J.

          - For other films, clearing recommendations from manufacturers shall be followed.

          - Internal heating due to ESR can increase ageing and should be taken into account by applying a margin in temperature. Where ESR is not known at the frequency of a ripple current, an extrapolation of the ESR value and resonance (from manufacturer’s or test data) should be made where possible.

### Derating

## Capacitors: glass and porcelain – family-group code: 01-06

### General

          - Internal heating due to ESR can increase ageing and should be taken into account by applying a margin in temperature. Where ESR is not known at the frequency of a ripple current, an extrapolation of the ESR value and resonance (from manufacturer’s or test data) should be made where possible.

### Derating

## Capacitors: mica and reconstituted mica – family-group code: 01-07

### General

          - Internal heating due to ESR can increase ageing and should be taken into account by applying a margin in temperature. Where ESR is not known at the frequency of a ripple current, an extrapolation of the ESR value and resonance (from manufacturer’s or test data) should be made where possible.

### Derating

## Capacitors: feedthrough – family-group code: 01-10

### General

          - Internal heating due to ESR can increase ageing and should be taken into account by applying a margin in temperature. Where ESR is not known at the frequency of a ripple current, an extrapolation of the ESR value and resonance (from manufacturer’s or test data) should be made where possible.

### Derating

## Capacitors: semiconductor technology (MOS type) – 
family-group code: 01-11

### General

          - Internal heating due to ESR can increase ageing and should be taken into account by applying a margin in temperature. Where ESR is not known at the frequency of a ripple current, an extrapolation of the ESR value and resonance (from manufacturer’s or test data) should be made where possible.

### Derating

## Capacitors: miscellaneous (variable capacitors) – family-group code: 01-99

### General

          - Internal heating due to ESR can increase ageing and should be taken into account by applying a margin in temperature. Where ESR is not known at the frequency of a ripple current, an extrapolation of the ESR value and resonance (from manufacturer’s or test data) should be made where possible.

### Derating

## Connectors – family-group code: 02-01, 02-02, 02-03, 02-07 and 02-09

### General

          - For power connectors, power and return lines shall be separated by at least one unassigned contact to reduce the short-circuit risk.

          - Connector savers shall be used during testing of equipment to minimize number of mating and demating cycles.

          - When multi-pin connectors are close to one another, they shall be configured such that mating with a wrong connector is not possible or the contact assignments shall be chosen such that mating with a wrong connector does not cause damage to the unit itself nor to any other element of the system.

          - The connector and its constituent parts shall be from the same manufacturer.

### Derating

| Parameters | Load ratio or limit |
| --- | --- |
| Working voltage | 50 % of specified voltage at any altitude (pin-to-pin and pin-to-shell). |
| Current | 50 % |
| Maximum operating temperature | 30 C below maximum rated temperature. |
| Maximum mating and demating cycles | 50 |

## Connectors RF – family-group code: 02-05

### General

          - Connector savers shall be used during testing of equipment to minimize number of mating and demating cycles.

### Derating

| Parameters | Load ratio or limit |
| --- | --- |
| RF power | 75 % and shall be limited such that a 6 dB margin exists before the onset of multipactor. |
| Working voltage | 50 % of specified voltage at any altitude (pin-to-pin and pin-to-shell). |
| Maximum operating temperature | 30 C below maximum rated temperature. |
| Maximum mating and demating cycles | 50 |

## Piezo-electric devices: crystal resonator – family-group code: 03-01

### Derating

| Part type | Derating |
| --- | --- |
| Drive level | 25 % power rated drive level (superseded by manufacturer required minimum drive level if not compatible). |

## Diodes – family-group code: 04-01, 04-02, 04-03, 04-04, 04-06, 04-08, 04-10 to 04-13 and 04-14

### General

          - Some diodes can be radiation sensitive: the issue shall be recorded in the design file and the components selection shall be reviewed and approved as described in ECSS-Q-ST-60. Where power cycling is critical this should be considered.

### Derating

#### Diode (signal/switching, rectifier, transient suppression, varactor, pin, Schottky, step recovery) derating table

| Part type | Derating |
| --- | --- |
| Forward surge current (IFSM): - non-repetitive operation - repetitive operation | 80 % 60 % |
| Reverse voltage (VR) | 75 % |
| Dissipated power (PD) | 50 % (only if dissipated power is defined by the manufacturer) |
| Maximum junction temperature (Tj max) | 110 C or Tj max – 40 C (whichever is lower). |

#### Diode (Zener, reference) derating table

| Part type | Derating |
| --- | --- |
| Zener surge current (IZSM): - non-repetitive operation - repetitive operation | 80 % 60 % |
| Dissipated power (PD) | 65 % |
| Maximum junction temperature (Tj max) | 110 C or Tj max – 40 C (whichever is lower) |

## Diodes: RF/microwave-PIN – family-group code: 04-05, 04-15, 04-16 and 04-17

### General

          - Some diodes can be radiation sensitive: the issue shall be recorded in the design file and the components selection shall be reviewed and approved as described in ECSS-Q-ST-60.

### Derating

| Part type | Derating |
| --- | --- |
| Forward surge current | 50 % |
| Reverse voltage (VR) | 75 % |
| Dissipated power (PD) | 65 % |
| Maximum junction temperature (Tj max) | 110 C or Tj max – 40 C (whichever is lower) |
| NOTE 1: Forward current is not applicable to varactors. NOTE 2: Reverse voltage is not applicable to Gunn diodes. |  |

## Feedthrough filters – family-group code: 05-01

### Derating

## Fuses: Cermet (metal film on ceramic) – family-group code: 06-01

### General

          - Fuses shall be avoided whenever possible. The derating requirements in 6.17.2 (below) are only applicable to Cermet types. The application and the deratings of other fuse technologies shall be justified.

          - The largest fuse rating compatible with the source capability shall be used.

          - The power supply shall be capable of delivering three times the specified fuse rated current in order to obtain short fusing times.

### Derating

## Inductors and transformers – family-group code: 07-01 to 07-03

### General

          - For custom-made inductors and transformers, the maximum rated temperature shall be evaluated taking into consideration the temperature characteristics of the materials used.

### Derating

| Parameters | Load ratio or limit | Special conditions |
| --- | --- | --- |
| Maximum operating voltage (1) | 50 % of the applied insulation test voltage (2) | (1) Between winding-winding and between windings-case. The maximum operating voltage shall include DC, AC peak or combined. (2) Unless specified in the procurement specification, the minimum insulation test voltage applied shall be 500 V. For operating voltages greater than 200 V the insulation test voltage is equal to the partial discharge voltage (VPD), defined as the component qualification test level, where the partial discharge activity is detected, and with a test equipment sensitivity of no less than 1 pC. |
| Hot spot temperature | 20 C below maximum rated temperature of any material used. |  |

## Integrated circuits: logic – family-group code: 08-10, 08-20, 08-21, 08-29 to 08-42, and 08-80

### General

          - Some devices can be radiation sensitive: this shall be recorded and approved in accordance with ECSS-Q-ST-60.

### Derating

| Parameters | Load ratio or limit | Special conditions |
| --- | --- | --- |
| Supply voltage (VCC) | Manufacturer recommended value ±5 % or 90 % of maximum rating. | Supply voltage - Turn on transient peaks or other peaks shall not exceed the maximum rating. - The input voltage shall not exceed the supply voltage (unless otherwise stated in the device specification). |
| Output current (Iout) | 80 % |  |
| Maximum junction temperature (Tj max) | 110 C or Tj max – 40 C (whichever is lower) | Junction temperature If the maximum junction temperature is not specified, derate from the maximum operating temperature. |

## Integrated circuits: non-volatile memories – family-group code: 08-22, 08-23 and 08-24

### General

          - Some devices can be radiation sensitive: this shall be recorded and approved in accordance with ECSS-Q-ST-60.

### Derating

| Parameters | Load ratio or limit | Special conditions |
| --- | --- | --- |
| Supply voltage (VCC) | Manufacturer’s recommended value ±5 % or 90 % of maximum rating. | Supply voltage - Turn on transient peaks or other peaks shall not exceed the maximum rating. - The input voltage shall not exceed the supply voltage (except adapted component design). |
| Output current (Iout) | 80 % |  |
| Maximum junction temperature (Tj max) | 110 C or Tj max – 40 C (whichever is lower) | Junction temperature If the maximum junction temperature is not specified, derate from the maximum operating temperature. |
| Endurance and data retention |  | Endurance The endurance (number of write and erase cycles) and the retention time-to-failure of EPROM, EEPROM and Flash devices can be derated from the manufacturer specification case by case. An acceleration model, such as Arrhenius’s law with an activation energy of 0,6 eV, or lower, shall be used to determine the equivalent time for space flight. |

## Integrated circuits: linear – family-group code: 08-50 to 08-60 and 08-69

### General

          - Some linear circuits can be radiation sensitive: the issue shall be recorded in the design file and the components selection shall be reviewed and approved as described in ECSS-Q-ST-60.

          - Additional margins can be applied for radiation effects.

### Derating

| Parameters | Load ratio or limit | Special conditions |
| --- | --- | --- |
| Supply voltage (VCC) | 90 % of the maximum rated value | Supply voltage shall include DC + AC ripple. |
| Input voltage (VIN) | 70 % (or 50 % on the input current) for operational amplifiers. 100 % or derated circuit supply voltage, whichever is less, for comparators. 90 % for regulators. |  |
| Output current (Iout) | 80 % |  |
| Transients | Shall not exceed the specified maximum ratings. |  |
| Maximum junction temperature (Tj max) | 110 C or Tj max – 40 C, whichever is lower. |  |

## Integrated circuits: linear converters – family-group code: 08-61 and 08-62

### General

          - Some linear circuits can be radiation sensitive: the issue shall be recorded in the design file and the components selection shall be reviewed and approved as described in ECSS-Q-ST-60.

### Derating

| Parameters | Load ratio or limit | Special conditions |
| --- | --- | --- |
| Maximum junction temperature (Tj max) | 110 C or Tj max – 40 C, whichever is lower. |  |
| Supply voltage (VCC) | 90 % | Supply voltage shall include DC + AC ripple. |
| Input voltage (VIN) | 100 % or derated circuit supply voltage, whichever is less. |  |
| Output current (Iout) | 80 % (D/A converters only) |  |

## Integrated circuits: MMICs – family-group code: 08-95

### General

          - Some MMICs can be radiation sensitive: the choice of MMICs shall be based on suitability and application. Justification shall be in accordance with ECSS-Q-ST-60 and provided in accordance with this Standard, and at the design reviews.

### Derating

          - Each discrete cell (capacitors, resistors, diodes and transistors) constituting analogue custom MMICs shall be derated in accordance with this document’s requirements for the applicable family. For digital cells, apply the derating rules applicable to integrated circuits.

          - When operational reliability data is available, the compression level shall be derated to 2 dB under the highest compression level showing no drift. MMICs having no compression data shall not be submitted to more than 1 dB of compression.

## Integrated circuits: miscellaneous – family-group code: 08-99

### General

          - For all ICs not considered in the previous subgroups, the following derating rules shall be followed:

- Manufacturer’s derating values.

- Maximum junction temperature: 110 C or Tj max – 40 C, whichever is lower.

- For the part of the IC similar to logic ICs, apply the derating rules for logic subgroups, for the part similar to linear ICs, apply the derating rules for linear subgroups and so forth.

          - Some integrated circuits can be radiation sensitive: this shall be recorded and approved in accordance with ECSS-Q-ST-60.

## Relays and switches – family-group code: 09-01, 09-02 and 16-01

### General

          - The coil supply voltage shall be within the specified voltage range or between the specified rated and the maximum coil voltage.

          - The minimum coil pulse duration for latching relays shall be 10 times the sum of latch time (tL) and bounce time (tB) or 30 ms, whichever is greater.

          - Current derating does not apply to contacts that only carry current and do not switch it or to contacts that switch at zero current. In the latter case, the number of operations is limited to the qualified number of operations.

          - Rated contact load voltage should not be exceeded since it has a strong impact on the contact current: this shall be recorded and approved in accordance with ECSS-Q-ST-60.

          - A double throw contact shall not be used to switch a load (movable contact) between a power supply and ground (stationary contacts). This type of configuration may be accepted in the following conditions:

- when switching off the power supply the other stationary contact is not connected to ground, or

- the potential difference between stationary contacts is less than 10 V and the switched current less than 0,1 A.

          - Paralleled relays shall not be used to increase current switching capabilities of contacts. When relays are paralleled for redundancy, in order to increase the system’s reliability, the sum of the paralleled currents shall not exceed the contact current rating.

          - Suppression diodes shall not be used inside relays.

### Derating

## Resistors – family-group code: 10-01 to 10-11

### Derating (all resistors except heaters)

#### General

          - The mentioned temperatures cited below refer to case temperatures.

#### Metal film precision resistor (type RNC, except RNC 90) derating table

| Part type | Derating |
| --- | --- |
| Voltage | 80 % |
| Power | 50 % up to 125 C and further decreasing to 0 % at 150 C |

#### Metal film semi-precision resistor (type RLR) derating table

| Part type | Derating |
| --- | --- |
| Voltage | 80 % |
| Power | 50 % up to 70 C and further decreasing to 0 % at 125 C |

#### Foil resistor (type RNC 90) derating table

| Part type | Derating |
| --- | --- |
| Voltage | 80 % |
| Power | 50 % up to 70 C and further decreasing to 0 % at 125 C |

#### Wire-wound high precision resistor (type RBR 56) derating table

| Part type | Derating |
| --- | --- |
| Voltage | 80 % |
| Power (type RBR 56) | Wire-wound for all tolerances: 50 % up to 115 C, decreasing to 0 % at 130 C |

#### Wire-wound power resistor (type RWR, RER) derating table

| Part type | Derating |
| --- | --- |
| Voltage | 80 % |
| Power | 60 % up to 25 C, decreasing to 0 % at 175 C |

#### Chip resistor (RM), network resistor derating table

| Part type | Derating |
| --- | --- |
| Voltage | 80 % |
| Power | 50 % up to 85 C, decreasing to 0 % at 125 C |

#### Microwave load resistor derating table

| Part type | Derating |
| --- | --- |
| Voltage | 80 % |
| Power | 50 % |

#### Pulse power rating - repetitive pulses

          - Average power < 50 % of nominal rating

Paverage=Vp2R×tpT

where

Vp	is the pulse amplitude

R	is the resistance

tp	is the pulse width

T	is the cycle duration

#### Pulse power rating (single pulse)

          - If ratings are provided for transient or surge condition, the same derating figures as for steady-state equivalent parameters shall be used.

          - If ratings are not provided for transient or surge condition, then it shall be assured that the transient or surge values are below the steady-state specified ratings.

### Derating for heaters

| Parameters | Load ratio or limit |
| --- | --- |
| Actual rated power (W) | 50 % |

          - Actual rated power shall be specified in the applicable heater design drawing. It shall be determined from the specified heating area (s) in cm2 taking into account the thermal properties of the mounted heater in the application.

## Thermistors – family-group code: 11-01 to 11-03

### Derating

| Parameters | Load ratio or limit |
| --- | --- |
| Power | 50 % of the maximum power |

## Transistors: bipolar – family-group code: 12-01 to 12-04 and 12-09

### General

          - Some transistors can be radiation sensitive: the issue shall be recorded in the design file and the components selection shall be reviewed and approved as described in ECSS-Q-ST-60.

### Derating

| Part type | Derating |
| --- | --- |
| Collector-emitter voltage (VCE0) | 75 % |
| Collector-base voltage (VCB0) | 75 % |
| Emitter-base voltage (VEB0) | 75 % |
| Collector current (IC max) | 75 % |
| Base current (IB max), if specified | 75 % |
| Power (PD) | 65 % of maximum power. a b |
| Maximum junction temperature (Tj max) | 110 C or Tj max - 40 C (whichever is lower). |
| a The designer should refer to the SOA. b Where power cycling is critical this should be considered. |  |

## Transistors: FET – family-group code: 12-05 and 12-06

### General

          - Only SEE radiation characterized MOSFETs shall be used in space applications.

          - The user shall consider power cycling when this is critical to his application.

### Derating

| Parameters | Load ratio or limit |
| --- | --- |
| Drain to source voltage (VDS) | 80 % of rated, or the SEE safe operating area (VDS versus VGS), whichever is lower |
| Gate to source voltage (VGS) | 75 %, taking into account the SEE Safe Operating Area |
| Drain current (IDS) | 75 % |
| Power dissipation (PD) max | 65 % max |
| Maximum junction temperature (Tj max) | 110 C or Tj max - 40 C (whichever is lower) |

## Transistors: RF: bipolar – family-group code: 12-10 and 12-13

### General

          - Some transistors can be radiation sensitive: the issue shall be recorded in the design file and the components selection shall be reviewed and approved as described in ECSS-Q-ST-60.

### Derating

| Part type | Derating |
| --- | --- |
| Collector-emitter voltage (VCE) | 75 % |
| Collector-base voltage (VCB) | 75 % |
| Emitter-base voltage (VEB) | 75 % |
| Collector current (IC) | 75 % |
| Base current (IB), if specified | 75 % |
| Power dissipation (PD) | 65 % or limited by the derating on operating temperature. |
| Maximum junction temperature (Tj max) | 110 C or Tj max – 40 C (whichever is lower) for Si bipolar transistors. |

## Transistors: RF: FET – family-group code: 12-12, 12-14, 12-15(FET) and 12-16(FET)

### General

          - Some transistors can be radiation sensitive: this issue shall be recorded in the design file and the components selection shall be reviewed and approved as described in ECSS-Q-ST-60.

### Derating

| Part type | Derating |
| --- | --- |
| Drain to source voltage (VDS) | 75 % |
| Gate to source voltage (VGS) | 75 % |
| Gate to drain voltage (VDS) | 75 % |
| Drain current (IDS) | 75 % |
| Power dissipation (PD) | 80 % or limited by the derating on operating temperature. |
| Maximum junction temperature (Tj max) | 110 C or Tj max – 40 C (whichever is lower) for Si. 115 C or Tj max – 40 C (whichever is lower) for GaAs. |
| NOTE When supported by reliability data, the compression level (including worst case modulation peak compression) is derated to 2 dB under the highest compression level showing no drift. No compression levels exceeding 1 dB are applied to FETs without compression data. |  |

## Wires and cables – family-group code: 13-01 to 13 03

### Derating

| Parameters | Load ratio or limit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage | 50 % |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wire size (AWG) | 32 | 30 | 28 | 26 | 24 | 22 | 20 | 18 | 16 | 14 | 12 | 10 | 8 | 6 | 4 |
| Maximum current Cu (I) (A) a b | 1,2 | 1,3 | 1,5 | 2,5 | 3,5 | 5 | 7,5 | 10 | 13 | 17 | 25 | 32 | 45 | 60 | 81 |
| Maximum current Al (I) (A) a b |  |  |  |  |  | 4 | 6 | 8 | 10,4 | 13,6 | 18,4 | 25,6 | 36 |  |  |
| Wire surface temperature | Manufacturer’s maximum rating – 40 C or 120 C, whichever is lower. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a Maximum applied current according to the wire size (AWG) resulting in a maximum temperature of 120 C. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b The derating on current for bundles with N wires is calculated as follows IBW = ISW × K |  | Wires AWG 0 to AWG 12 |  |  |  |  |  |  | Wires AWG 14 to AWG 32 |  |  |  |  |  |  |
|  |  | Number of wires (N) |  |  | K |  |  |  | Number of wires (N) |  |  | K |  |  |  |
|  |  | 1 < N  3 3 < N  7 7 < N  19 19 < N  331 |  |  | 1,1 – (0,1 × N) 1,01 – (0,07 × N) 0,81 – [0,15 × ln(N)] 0,59 – [0,076 × ln(N)] |  |  |  | 1 < N  3 3 < N  7 7 < N  52 52 < N  331 |  |  | 1,1 – (0,1 × N) 1,01 – (0,07 × N) 0,81 – [0,15 × ln(N)] 0,467 – [0,0632 × ln(N)] |  |  |  |
|  |  | IBW: maximum current for an individual wire in a bundle. ISW: maximum current for a single wire as given in the derating table above. ln: Natural log. |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Opto-electronics – family-group code: 18-01 to 18-05

### General

          - Light emitting diodes can be radiation sensitive, in particular, there is a high sensitivity to proton displacement damage: this issue shall be recorded in the design file and the components selection shall be reviewed and approved as described in ECSS-Q-ST-60.

          - Opto-couplers can be radiation sensitive, in particular, operation at low diode currents increases radiation sensitivity: this issue shall be recorded in the design file and the components selection shall be reviewed and approved as described in ECSS-Q-ST-60.

          - Photo-transistors can be radiation sensitive: this issue shall be recorded in the design file and the components selection shall be reviewed and approved as described in ECSS-Q-ST-60.

### Derating

| Parameters | Load ratio or limit |
| --- | --- |
| Light emitting diode: |  |
| Forward current | Manufacturer recommended value, or derate to 50 % if not available |
| Reverse voltage | Derate to 75 % |
| Photo transistor: |  |
| Maximum collector current | Derate to 80 % |
| Maximum collector-emitter voltage | Derate to 75 % |
| Light emitting diode and photo transistor: |  |
| Maximum junction temperature (Tj max) | 110 C or Tj max-40 C (whichever is lower) |

## RF passive components: family-group-code: 30-01, 30-07, 30-09, 30-10 and 30-99

### General

          - For connectorized components, connector savers shall be used during testing of equipment to minimize number of mating and demating cycles.

### Derating

| Parameters | Load ratio or limit |
| --- | --- |
| RF power | 75 % and shall be limited such that a 6 dB margin exists before the onset of multipactor. |
| Maximum operating temperature | 30 C below maximum rated temperature. |
| Maximum mating and demating cycles | 50 |

## Fibre optic components: fibre and cable: family-group-code: 27-01

### Derating

| Parameters | Load ratio or limit |
| --- | --- |
| Bend radius | 200 % of the minimum value |
| Cable tension | 50 % of the rated tensile strength |
| Fibre tension | 20 % of the proof test |

## Hybrids

          - For hybrids, individual components shall be in conformance with their respective derating rules.

- (informative)
Family and group codes

This annex contains an extract from the European preferred parts list (EPPL) and it lists all the parts referred to in this Standard providing their family and group codes.

| Family code | Group code | Family | Group |
| --- | --- | --- | --- |
| 01 | 01 | Capacitors | Ceramic |
| 01 | 02 | Capacitors | Ceramic Chip |
| 01 | 03 | Capacitors | Tantalum solid |
| 01 | 04 | Capacitors | Tantalum non-solid |
| 01 | 05 | Capacitors | Plastic metallized |
| 01 | 06 | Capacitors | Glass |
| 01 | 07 | Capacitors | Mica and reconstituted mica |
| 01 | 10 | Capacitors | Feedthrough |
| 01 | 11 | Capacitors | Semiconductor |
| 01 | 99 | Capacitors | Miscellaneous |
| 02 | 01 | Connectors | Circular |
| 02 | 02 | Connectors | Rectangular |
| 02 | 03 | Connectors | Printed circuit board |
| 02 | 07 | Connectors | Microminiature |
| 02 | 09 | Connectors | Rack and panel |
| 03 | 01 | Piezo-electric devices | Crystal resonator |
| 04 | 01 | Diodes | Switching |
| 04 | 02 | Diodes | Rectifier |
| 04 | 03 | Diodes | Voltage regulator |
| 04 | 04 | Diodes | Voltage reference/zener |
| 04 | 05 | Diodes | RF/microwave Schottky - Si |
| 04 | 06 | Diodes | Pin |
| 04 | 08 | Diodes | Transient suppression |
| 04 | 10 | Diodes | High voltage rectifier |
| 04 | 11 | Diodes | Microwave varactor - GaAs |
| 04 | 12 | Diodes | Step recovery |
| 04 | 13 | Diodes | Microwave varactor - Si |
| 04 | 14 | Diodes | Current regulator |
| 04 | 15 | Diodes | Microwave Schottky - GaAs |
| 04 | 16 | Diodes | RF/microwave - PIN |
| 04 | 17 | Diodes | Microwave Gunn - GaAs |
| 05 | 01 | Filters | Feedthrough |
| 06 | 01 | Fuses | All |
| 07 | 01 | Inductors | RF coil |
| 07 | 02 | Inductors | Cores |
| 07 | 03 | Inductors | Chip |
| 08 | 10 | Microcircuits | Microprocessors/microcontrollers/ peripherals |
| 08 | 20 | Microcircuits | Memory SRAM |
| 08 | 21 | Microcircuits | Memory DRAM |
| 08 | 22 | Microcircuits | Memory PROM |
| 08 | 23 | Microcircuits | Memory EPROM |
| 08 | 24 | Microcircuits | Memory EEPROM |
| 08 | 29 | Microcircuits | Memory others |
| 08 | 30 | Microcircuits | Programmable logic |
| 08 | 40 | Microcircuits | ASIC technologies digital |
| 08 | 41 | Microcircuits | ASIC technologies linear |
| 08 | 42 | Microcircuits | ASIC technologies mixed analogue/digital |
| 08 | 50 | Microcircuits | Linear operational amplifier |
| 08 | 51 | Microcircuits | Linear sample and hold amplifier |
| 08 | 52 | Microcircuits | Linear voltage regulator |
| 08 | 53 | Microcircuits | Linear voltage comparator |
| 08 | 54 | Microcircuits | Linear switching regulator |
| 08 | 55 | Microcircuits | Linear line driver |
| 08 | 56 | Microcircuits | Linear line receiver |
| 08 | 57 | Microcircuits | Linear timer |
| 08 | 58 | Microcircuits | Linear multiplier |
| 08 | 59 | Microcircuits | Linear switches |
| 08 | 60 | Microcircuits | Linear multiplexer/demultiplexer |
| 08 | 61 | Microcircuits | Linear analog to digital converter |
| 08 | 62 | Microcircuits | Linear digital to analogue converter |
| 08 | 69 | Microcircuits | Linear other functions |
| 08 | 80 | Microcircuits | Logic families |
| 08 | 95 | Microcircuits | MMIC |
| 08 | 99 | Microcircuits | Miscellaneous |
| 09 | 01 | Relays | Non-latching |
| 09 | 02 | Relays | Latching |
| 10 | 01 | Resistors | Metal oxide |
| 10 | 02 | Resistors | Wire-wound precision - including surface mount |
| 10 | 03 | Resistors | Wire-wound chassis mounted |
| 10 | 04 | Resistors | Variable trimmers |
| 10 | 05 | Resistors | Composition |
| 10 | 07 | Resistors | Shunt |
| 10 | 08 | Resistors | Metal film |
| 10 | 09 | Resistors | Chip - all |
| 10 | 10 | Resistors | Network - all |
| 10 | 11 | Resistors | Heaters, flexible |
| 11 | 01 | Thermistors | Temperature compensating |
| 11 | 02 | Thermistors | Temperature measuring |
| 11 | 03 | Thermistors | Temperature sensor |
| 12 | 01 | Transistors | Low power, NPN - < 2 W |
| 12 | 02 | Transistors | Low power, PNP - < 2 W |
| 12 | 03 | Transistors | High power, NPN - > 2 W |
| 12 | 04 | Transistors | High power, PNP - > 2 W |
| 12 | 05 | Transistors | FET N channel |
| 12 | 06 | Transistors | FET P channel |
| 12 | 09 | Transistors | Switching |
| 12 | 10 | Transistors | RF/microwave NPN low power/low noise |
| 12 | 12 | Transistors | RF/microwave FET N-channel/P-channel |
| 12 | 13 | Transistors | RF/microwave bipolar power |
| 12 | 14 | Transistors | RF/microwave FET power - Si |
| 12 | 15 | Transistors | Microwave power - GaAs |
| 12 | 16 | Transistors | Microwave low noise - GaAs |
| 13 | 01 | Wires and cables | Low frequency |
| 13 | 02 | Wires and cables | Coaxial |
| 13 | 03 | Wires and cables | Fibre optic |
| 16 | 01 | Switches | Standard DC/AC power toggle |
| 18 | 01 | Opto-electronics | Opto-coupler |
| 18 | 02 | Opto-electronics | LED |
| 18 | 03 | Opto-electronics | Phototransistor |
| 18 | 04 | Opto-electronics | Photo diode/sensor |
| 18 | 05 | Opto-electronics | Laser diode |
| 27 | 01 | Fibreoptic components | Fibre/cable |
| 30 | 01 | RF passive devices | Coaxial couplers |
| 30 | 07 | RF passive devices | Isolator/circulator |
| 30 | 09 | RF passive devices | Coaxial power dividers |
| 30 | 10 | RF passive devices | Coaxial attenuators/loads |
| 30 | 99 | RF passive devices | Miscellaneous |

Bibliography

| ECSS-S-ST-00 | ECSS system - Description and implementation and general requirements |
| --- | --- |