Space product assurance

ECSS Secretariat

ESA-ESTEC

Requirements & Standards Division

Noordwijk, The Netherlands

Sneak analysis - Part 2: Clue list

Foreword

This document is one of the series of ECSS Technical Memoranda. Its Technical Memorandum status indicates that it is a non-normative document providing useful information to the space systems developers’ community on a specific subject. It is made available to record and present non-normative data, which are not relevant for a Standard or a Handbook. Note that these data are non-normative even if expressed in the language normally used for requirements.

Therefore, a Technical Memorandum is not considered by ECSS as suitable for direct use in Invitation To Tender (ITT) or business agreements for space systems development.

Disclaimer

ECSS does not provide any warranty whatsoever, whether expressed, implied, or statutory, including, but not limited to, any warranty of merchantability or fitness for a particular purpose or any warranty that the contents of the item are error-free. In no respect shall ECSS incur any liability for any damages, including, but not limited to, direct, indirect, special, or consequential damages arising out of, resulting from, or in any way connected to the use of this Document, whether or not based upon warranty, business agreement, tort, or otherwise; whether or not injury was sustained by persons or property or otherwise; and whether or not loss was sustained from, or arose out of, the results of, the item, or any services that may be provided by ECSS.

Published by: 	ESA Requirements and Standards Division

ESTEC, P.O. Box 299,

2200 AG Noordwijk

The Netherlands

Copyright: 	2010© by the European Space Agency for the members of ECSS

Change log

| ESS-Q-40-04A Part 1 14 October 1997 | First issue |
| --- | --- |
| ECSS-Q-TM-40-04 Part 2A 16 April 2010 | Second issue Conversion of ECSS-Q-40-04A - Part 2: Clue list (14 October 1997) into a Technical Memorandum according to ECSS drafting rules for Technical Memorandum. |

Table of contents

Change log	3

1 Scope	5

2 Normative references	6

3 Terms, definitions and abbreviated terms	7

3.1	Terms from other standards	7

3.2	Abbreviated terms	7

4 Clue list structure	8

4.1	Taxonomy of items	8

4.2	Structure of the entries of the clue list	8

Annex A (informative)  Taxonomy	10

Annex B (normative) Clues for electrical, electromechanical and electronic equipment	16

Annex C (normative) Clues for software in ADA language	72

Annex D (informative) Clues for hydraulic equipment	91

Index	97

# Scope

The aim of sneak analysis is to identify sneak circuits, i.e. unexpected paths for a flow of mass, energy, data or logical sequence that under certain conditions can initiate an undesired function or inhibit a desired function. Sneak circuits are not the result of failure, but are latent conditions, inadvertently designed into the system.

This Technical Memorandum establishes a procedure for performing sneak analysis and specifies the required output.

The Technical Memorandum is composed of two parts:

- Part 1 (i.e. this document ECSS-Q-TM-40-04- Sneak analysis - Methods and procedures) that contains the methods and procedures for performing sneak analysis;

- Part 2 (i.e. the document ECSS-Q-TM-40-04 - Sneak analysis - Clue list) that contains a basic clue list to be used during sneak analysis.

This technical memorandum is applicable when the performance of sneak analysis is required by ECSS-Q-ST-40 or by the business agreement between the customer and the supplier.

Alternative sneak analysis procedures proposed by the supplier may be accepted by the customer provided that equivalence, for the intended application, with the one presented in this Technical Memorandum is shown by the supplier.

This Technical Memorandum may be tailored for the specific characteristic and constrains of a space project in conformance with ECSS-S-ST-00.

# Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this ECSS Standard. For dated references, subsequent amendments to, or revision of any of these publications do not apply, However, parties to agreements based on this ECSS Standard are encouraged to investigate the possibility of applying the more recent editions of the normative documents indicated below. For undated references, the latest edition of the publication referred to applies.

| ECSS-ST-S-00-01 | ECSS system - Glossary of terms |
| --- | --- |
| ECSS-Q-TM-40-04 | Space product assurance - Sneak analysis – Part 1: Principles and requirements |

# Terms, definitions and abbreviated terms

## Terms from other standards

For the purpose of this document, the terms and definitions from ECSS-S-ST-00-01, ECSS-Q-ST-30, ECSS-Q-ST-40 and ECSS-Q-ST-40-04 apply.

## Abbreviated terms

For the purpose of this Standard, the abbreviated terms from ECSS-S-ST-00-01 and ECSS-Q-ST-40-04 apply:

# Clue list structure

## Taxonomy of items

Annex A contains the taxonomy of “items” namely EEE and hydraulic components, software instructions) relevant to the clues listed in this document.

For EEE equipment and software written in the ADA language the clues contained in Annexes B and C shall be used.

When Sneak Analysis is to be applied to a hydraulic system or subsystem the clues contained in Annex D may be used.

The Annexes of this document may be obtained, upon request from the ECSS secretariat, either as word-processor file or as a database for personal computers.

## Structure of the entries of the clue list

A clue is defined in ECSS-Q-TM-40-04A - Part 1, 3.2.1 as a: “question pointing at a possible way through which design errors associated with one or more items of a system can lead to a system malfunction”.

To apply a particular clue, it is necessary to clearly identify:

- the text of the question;

- to which items the questions applies (the taxonomy of these items is contained in Annex A).

To facilitate the management of the clue list and its application other attributes are attached to each clue, i.e.:

- an unambiguous alphanumeric identifier (clues for EEE equipment have identifiers in the range 1-999, clues for software have identifiers in the range 1000-1999, clues for hydraulic equipment have identifiers in the range 2000-2999);

- a “clue short title” which provides a short description of the content of the clue;

the “clue explanation”, i.e. a short explanation of the clue text;

To restrict the use of the various clues to the sneak analysis procedure for which they are intended (see ECSS-Q-TM-40-04 - Part 1, 4.1), the attribute “clue type” has been associated to each entry of the list.

- If a clue type is “path-component” then it is intended for use during the application of the sneak path analysis;

- If a clue type is “component-only” then it is intended for use during the application of the design concern analysis.

It is noted that some clues are intended for use both during sneak path analysis and design concern analysis and these are typed by two keywords “path-component” and “component-only”.

Some “component-only” clues need to be applied only once to each member of the taxonomy and not to each component in the system under analysis. These clues are identified by the keyword “once-only” and are to be used only during the design concern analysis. For an example, see clue 60 in Annex B.

The “path” clues (see ECSS-Q-TM-40-04 - Part 1, 4.1) can be derived, according to the approach described in Annex B of ECSS-Q-TM-40-04 - Part 1, from the causal relation between “source” and “target” that is of interest. Therefore the “path” clues are not listed in this document.

The coding of an entry of the clue list is illustrated by the following example:

EXAMPLE

| Identifier  | 30 |
| --- | --- |
| Title  | protection against reverse voltage |
| Clue type  | component-only |
| Item  | regulator+OpAmp |
| Clue Text  | Is a regulator protected against reverse voltage? |

Explanation:	If a voltage drop or a short circuit occurs at the regulator input, the reverse voltage applied to the regulator can exceed the breakdown voltage. If the regulator has no internal protection diode, an external protection diode shall be added.

The table entries are:

 = 	univocal alphanumeric identifier

 =	clue short title

 =	clue type

 = 	items to which the question applies

 =  	the question pointing at a possible way through which design errors can lead to a system malfunction

 = 	clue explanation

- (informative)
 Taxonomy

This Annex presents the taxonomy of three classes of “item”, EEE and hydraulic components, ADA language instructions to which the clues contained in this document apply.

The taxonomy is present in the form of a tree in which the “hierarchical level” (currently ranging from 0 till 7) is given next to the name of the item. The level of a particular item within the hierarchy is shown by the amount of indentation from the left hand margin, lower level items being more indented than an item in the level above.

When a clue pertains to a parent item, it also pertains to the daughter items.

0,“Component”,

1,“Part”,

2,“signal”

2,“label”

3,“Offpage”		(connection symbol between different drawings)

2,“Electronic”

3,“Active”

4,“IC”		(integrated circuits)

5,“Digital”

6,“driver”

6,“tristate”

6,“gate”

7,“buffer”

7,“and”

7,“nand”	(negated AND)

7,“or”

7,“nor”		(negated OR)

7,“not”

7,“xor”		(exclusive OR)

6,“memory”

7,“RAM”	(random access memory)

8,“DRAM”	(dynamic RAM)

8,“SRAM”	(static RAM)

7,“ROM”	(read-only memory)

8,“PROM”	(Programmable ROM)

8,“EPROM”	(Erasable PROM)

8,“EEPROM”	(Electrically EPROM)

6,“Array”

7,“PLA”	(Programmable logic array)

7,“LCA”	(Logic cell array)

7,“EPLD”	(Electrically programmable logic device)

7,“PAL”	(Programmable array logic)

6,“Static”

7,“latch”

7,“JK”	(J-K flipflop)

7,“register”

7,“flipflop”

7,“monoflop”

7,“bistable”

7,“RS”	(Reset-Set flipflop)

5,“Counting”

6,“counter”

6,“mux”		(multiplexer)

6,“shift”

6,“adder”

6,“Clock”

6,“Oscillator”

5,“Processing”

6,“micropro”	(microprocessor)

6,“ASIC”		(Application specific IC)

6,“DAC/ADC”	(digital to analog converter/analog to digital 										converter)

6,“regulator”	(voltage regulator)

6,“communication”

5,“Analogue”

6,“amplifier”

6,“OpAmp”		(operational amplifier)

6,“comparator”

4,“Transistor”

5,“bipolar”

6, “PNP”

6, “NPN”

5,“Darlington”

5,“MOS”		(metal-oxide semiconductor)

5,“FET”			(field effect transistor)

4,“Diode”

5,“Zener”

5,“LED”		(light emitting diode)

5,“optocoupler”

4,“Rectifier”

5,“Thyristor”

5,“Triac”

5,“GTO”		(gate turn-off thyristor)

5,“Thyratron”

3,“Passive”

4,“Resistor”

4,“Capacitor”

4,“Heater”

4,“Thermistor”

4,“Potentiometer”

4,“Varistor”			(variable resistor)

4,“Inductor”

5,“coil”

2,“Electromechanic”

3,“Contactor”

4,“Breaker”

4,“Limiter”

4,“Relay”

5,“bistabrel”		(bistable relay)

5,“monostable”	(monostable relay)

5,“monofloprel”	(monoflop relay)

5,“Relaycoil”

5,“Relaypole”

6,“Singlepole”

6,“Multipole”

4,“Switch”

4,“Contact”

4,“Button”

3,“Transducer”

4,“Motor”

5,“Motorcoil”

5,“Actuator”

4,“EED”	(electroexplosive device)

5,“Squib”

5,“Explosive”

5,“Detonator”

5,“Initiator”

5,“Exbolt”	(explosive bolt)

4,“Speaker”

5,“Loudspeaker”

5,“Buzzer”

5,“Bell”

4,“Microphone”

4,“Detector”

4,“Sensor”

5,“CSens”	(current sensor)

5,“FSens”	(fire sensor)

5,“VSens”	(voltage sensor)

5,“TSens”	(temperature sensor)

5,“PSens”	(pressure sensor)

5,“PoSens”	(position sensor)

5,“XSens”	(concentration sensor)

5,“LSens”	(level sensor)

4,“Threshold”

2,“Electric”

3,“Binding”

4,“Fuse”

4,“Connector”

5,“Multiconnector”

5,“Interlock”

5,“Malecon”		(male connector)

5,“Femalecon”	(female connector)

5,“Plug”

5,“Terminal”

5,“Strap”

5,“Testpoint”

5,“Testgap”

4,“Wire”

4,“Cable”

4,“Bundle”

4,“Splice”

4.“Node”		(node of the electrical schematic)

3,“Transformation”

4,“Transformer”	(“trafo” is a synonym of  transformer)

4,“Autotransfo”	(auto transformer)

4,“Lamp”

5,“Neon”

5,“Indicator”

4,“Tube”

4,“Antenna”

3,“Earth”

4,“Ground”

4,“Chassis”

3,“Energy”

4,“power”

5,“Battery”

5,“Supply”

4,“Load”

4,“Regulator”

1,“Software”

2,“OUTPUT”

2,“OUTREG”	(output register)

2,“INPUT”

2,“INREG”		(input register)

2,“LOOP”

3,“FOR”

3,“REVERSE”

3,“END LOOP”

3,“EXIT”

3,“EXIT WHEN”

2,“GOTO”

2,“RETURN”

2,“CALL”

3,“PACKAGE”

3,“END”

3,“PROCEDURE”

3,“FUNCTION”

3,“EXCEPTION”

3,“RAISE”

3,“TASK”

3,“BEGIN”

3,“GENERIC”

3,“SEPARATE”

3,“SEMAPHORE”

2,“BOX” 	(software instruction implementing algorithmic operations, e.g. additions, 						divisions)

2,“ASSIGN”	(software instruction that assign values to variables)

2,“DIAMOND”

3,“IF”

3,“ELSE IF”

3,“CASE”

3,“WHEN”

3,“WHILE”

3,“OTHERS”

2,“ENDCONDITIONAL”

3,“ELSE”

3,“END IF”

3,“END CASE”

3,“END LOOP”

3,“NULL”

2,“THEN”

2,“DECLARATION”

3,“CONSTANT”

3,“TYPE”

4,“NEW”

4,“DIGITS”

4,“DELTA”

4,“RANGE”

4,“SUBTYPE”

4,“IS”

4,“RECORD”

4,“ACCESS”

4,“PRIVATE”

4,“RANGE”

3,“RENAME”

3,“IS SEPARATE”

1,“hydraulic”

2,“tank”

3,“tankpres”	(sealed container)

3,“tankbladder”

3,“vessel”

2,“valve”

3,“sv”		(relief valve)

3,“cvalve”	(control valve)

3,“checkv”	(check valve)

2,“pump”

2,“turbine”

2,“vent”

2,“drain”

2,“pregulator”		(pressure regulator)

- (normative)
Clues for electrical, electromechanical and electronic equipment

| Identifier | 23B |
| --- | --- |
| Title | place of the switching items |
| Clue type | component-only path-component |
| Item | switch+contact+breaker |
| Clue Text | Is the switch placed in the lower branch that closes the circuit for two or more upper (power dome) branches |

Explanation:	If a switching item is placed in the common branch, it may lead to a general current interruption in the multiple branches.

| Identifier | 24 |
| --- | --- |
| Title | transient exceeding max current |
| Clue type | component-only |
| Item | transistor |
| Clue Text | Can a current transient exceed transistor max current? |

Explanation:	During power transition or switching for instance, high inrush current may occur (capacitor charging): this current transient can induce a voltage transient, exceeding a breakdown voltage of the transistor. If a transistor is used to switch an inductive load without appropriate transient suppression measures being taken, there is a risk that the transistor may fail. When a resistor is used as a current path, and is connected in parallel with the emitter and the base of a transistor, if a reverse current flows through the resistor (temporary short circuit), the reverse base-emitter breakdown voltage can easily be exceeded (about 5 Volts).

| Identifier | 24A |
| --- | --- |
| Title | transient exceeding breakdown voltage |
| Clue type | component-only |
| Item | transistor |
| Clue Text | Can a voltage transient exceed transistor breakdown voltage? |

Explanation:	If a transistor is used to switch off an inductive load without appropriate transient suppression measures being taken, there is a risk that the transistor may fail.

| Identifier | 24B |
| --- | --- |
| Title | reverse current through collector |
| Clue type | component-only |
| Item | transistor |
| Clue Text | Can a voltage transient exceed emitter breakdown voltage? |

Explanation:	When a resistor is used as a current path, and is connected in parallel with the emitter and the base of a transistor, if a reverse current flows through the resistor (temporary short circuit), the reverse base-emitter breakdown voltage can easily be exceeded (about 5 Volts).

| Identifier | 25 |
| --- | --- |
| Title | permanent electrical link across a contact |
| Clue type | once-only |
| Item | relay+switch+contact+button |
| Clue Text | Does a permanent low current flow through the contact? |

Explanation:	The contact resistance may become significant if the current is low and the current won’t be sufficient for the contact regeneration; RESET and SET cycles shall be scheduled for the contact regeneration if no other possibility exists.

| Identifier | 26 |
| --- | --- |
| Title | parallel MOSFET transistors |
| Clue type | component-only |
| Item | transistor |
| Clue Text | Are the oscillations generated by two parallel MOSFET transistors damped out by an appropriate circuit? |

Explanation:	Two parallel MOSFET transistors can create an oscillating circuit.

| Identifier | 27A |
| --- | --- |
| Title | voltage compatibility between different IC families |
| Clue type | component-only |
| Item | IC |
| Clue Text | Is the voltage compatibility between circuits of different families respected? |

Explanation:	A problem of voltage compatibility can occur between different circuit families and it may lead to a sneak current path. Particular attention should be paid during power transition or when different voltage values are used at interfaces. Voltage differences at interface often occur:

-	during power on/off, some parts of the circuit may have reached their operational voltage while others are still changing

-	during power on/off, within a circuit supplied with separated voltages, one voltage may have reached its operational value while others are still changing

-	separated parts of a circuit can be independently supplied, one being powered while the others are not.

| Identifier | 27B |
| --- | --- |
| Title | collector-base junction reverse biased |
| Clue type | component-only |
| Item | transistor |
| Clue Text | Can the collector-base junction be reverse biased? |

Explanation:	A transistor receiving on its base an active signal while it is not nominally powered may induce a sneak current path towards a load, a power source or the ground through the collector-base junction if reverse biased. Particular attention should be paid during power transition or when different voltage values are used at interface. Voltage differences at interface often occur:

-	during power on/off, some parts of the circuit may have reached their operational voltage while others are still changing

-	during power on/off, within a circuit supplied with separated voltages, one voltage may have reached its operational value while others are still changing

-	separated parts of a circuit can be independently supplied, one being powered while the others are not.

| Identifier | 27C |
| --- | --- |
| Title | excessive reverse bias |
| Clue type | component-only |
| Item | diode |
| Clue Text | Can the diode junction be biased beyond breakdown? |

Explanation:	This can particularly occur if there are multiple voltages, or possibilities for voltage spikes.

| Identifier | 28 |
| --- | --- |
| Title | non-linear transfer function |
| Clue type | component-only |
| Item | OpAmp+transistor |
| Clue Text | Is a compensation network implemented to correct a non-linear transfer pattern, that supplies the lower branches? |

Explanation:	Either a non-linear function is designed to be non-linear (pre-accentuation of high frequencies for instance), either the non-linearity is unexpected and then a compensation network should be added. Multiple circuits in series (OpAmp) generally induce non linear transfer functions.

| Identifier | 29 |
| --- | --- |
| Title | feedback network on separated boards |
| Clue type | component-only |
| Item | signal+node+amplifier+OpAmp+transistor |
| Clue Text | Is the feedback network implemented on separated boards? |

Explanation:	The wire length used to connect the separated boards can induce variable inductive and capacitative effects, modifying the feedback characteristics. The response for the circuit may be altered and oscillation may arise.

| Identifier | 30 |
| --- | --- |
| Title | protection against reverse voltage |
| Clue type | component-only |
| Item | regulator+OpAmp |
| Clue Text | Is a regulator protected against reverse voltage? |

Explanation:	If a voltage drop or a short circuit occurs at the regulator input, the reverse voltage applied to the regulator can exceed the breakdown voltage. If the regulator has no internal protection diode, an external protection diode shall be added.

| Identifier | 32A |
| --- | --- |
| Title | filtering of a power source |
| Clue type | component-only |
| Item | power+supply+battery |
| Clue Text | Is there a filter at the output(s) of the power source? |

Explanation:	Filters shall be added in order to smooth ripple and avoid spikes to propagate outside the power source through the wiring.

| Identifier | 32AA |
| --- | --- |
| Title | filtering of a power source |
| Clue type | component-only |
| Item | power+supply+battery |
| Clue Text | Is there a filter at the output(s) of the power source? |

Explanation:	Filters shall be added in order to smooth ripple and avoid spikes to propagate outside the power source through the wiring.

| Identifier | 32B |
| --- | --- |
| Title | filtering of cables and long lines |
| Clue type | component-only |
| Item | buffer+connector |
| Clue Text | Is there a radio frequency (RF) filter provided on circuit output(s) interfacing cables and long lines? |

Explanation:	A RF filter shall be added in order to avoid RF to propagate through the wiring.
Note: you will need to check wire or cable length for this clue.

| Identifier | 33 |
| --- | --- |
| Title | frequency pass band for measurement |
| Clue type | component-only |
| Item | sensor+OpAmp+comparator |
| Clue Text | Is the frequency pass band of the measurement circuit adapted to the physical phenomenon? |

Explanation:	The acquisition frequency of a measurement / instrumentation circuit shall be at least twice the frequency of the measured phenomenon (Shannon theorem)

| Identifier | 34 |
| --- | --- |
| Title | delay of a signal |
| Clue type | component-only |
| Item | signal+cable+connector+offpage+malecon+femcon |
| Clue Text | Can the time constant of a signal path induce a critical delay for the signal? |

Explanation:	A long signal path between circuit boards can cause signal delays and resultant timing sneaks. Digital devices, analog switches comparators and transistors may exhibit unpredictable behaviour due resulting glitches.

| Identifier | 34AA |
| --- | --- |
| Title | delay of a signal |
| Clue type | path-component component-only |
| Item | signal+cable+connector+offpage+malecon+femcon |
| Clue Text | Can the time constant of a signal path induce a critical delay for the signal? |

Explanation:	A long signal path between circuit boards can cause signal delays and resultant timing sneaks. Digital devices, analog switches comparators and transistors may exhibit unpredictable behaviour due resulting glitches.

| Identifier | 34A |
| --- | --- |
| Title | RC/LC time constant |
| Clue type | path-component component-only |
| Item | capacitor+inductor |
| Clue Text | Can the time constant of a resistor-capacitor or inductor-capacitor circuit induce a critical delay for the signal? |

Explanation:	If a resistor-capacitor network is implemented on the signal path, a delay is induced that may introduce a sneak timing. The rise and fall times may be affected, especially for digital circuits, and the operating frequency may not be achieved. Moreover, with low rise and fall times, digital circuit may deliver erroneous signals. Digital devices, analog switches comparators and transistors may exhibit unpredictable switching times and excessive power dissipation due to imprecise switching caused by excessive input time constants.

| Identifier | 4 |
| --- | --- |
| Title | command input implementation |
| Clue type | component-only |
| Item | bistable+flip-flop+register+latch+counter+JK+RS+bistabrel |
| Clue Text | Are both the SET/START command and the RESET/STOP command implemented? |

Explanation:	As an example, if only the SET/START command is implemented, once the function has been activated, it will always remain in the same state as the RESET/STOP command is not available!

| Identifier | 5A |
| --- | --- |
| Title | maximum frequency delays |
| Clue type | component-only |
| Item | IC+gate+and+or+nand+nor+not |
| Clue Text | Have the propagation time, the rise time and fall time been taken into account for the maximum frequency calculation? |

Explanation:	There may be sneak timing problems if the expected signal frequency is close to a component maximum frequency limit.

| Identifier | 34B |
| --- | --- |
| Title | delay due to parasitic capacitance |
| Clue type | path-component component-only |
| Item | diode |
| Clue Text | Can the time constant of a signal path induce a critical delay for the signal due to parasitic capacitance? |

Explanation:	The forward resistance and the parasitic capacitance of a diode can induce a delay on the signal path.

| Identifier | 34C |
| --- | --- |
| Title | delay of a signal, high output resistance |
| Clue type | path-component component-only |
| Item | battery+power+supply+psens+tsens+lsens+fsens+xsens |
| Clue Text | Can the time constant of a signal path induce a critical delay for the signal due to high output resistance? |

Explanation:	A high output impedance state can cause slow operation due to the long RC time constant. Digital devices, analog switches comparators and transistors may exhibit unpredictable switching times and excessive power dissipation due to imprecise switching caused by excessive input time constants.

| Identifier | 35 |
| --- | --- |
| Title | voltage protection on inputs |
| Clue type | component-only |
| Item | OpAmp+comparator |
| Clue Text | Is the component protected against erroneous voltages on its inputs? |

Explanation:	When multiple signals are connected to the same input through resistors (summator, subtractor), a sneak current path can occur between these multiple signals if one of them delivers a voltage higher than expected: 
(Zener) diodes can be connected across  each signal input. 
Protection diodes can also be implemented to limit the differential voltage between inputs.

| Identifier | 36A |
| --- | --- |
| Title | location of monitoring circuit |
| Clue type | component-only |
| Item | sensor+detector+indicator+lamp+psens+fsens+tsens+xsens+lsens+posens |
| Clue Text | Is a monitoring circuit located and connected so that it delivers the status of the function to be monitored? |

Explanation:	The monitoring circuit shall indicate only the status of the monitored function, not the status of another function.

| Identifier | 40A |
| --- | --- |
| Title | disconnection from signal source |
| Clue type | path-component component-only |
| Item | sensor+detector+lamp+indicator+psens+fsens+xsens+lsens+csens |
| Clue Text | Can the monitoring circuit be disconnected from the monitored signal or device? |

Explanation:	The monitoring circuit should not indicate the opposite state of the monitored function.

| Identifier | 41 |
| --- | --- |
| Title | indication stability during transition |
| Clue type | component-only |
| Item | sensor+indicator+lamp+detector |
| Clue Text | Is the information delivered by the monitoring circuit stable and consistent during signal or state transitions? |

Explanation:	During power or mode transitions, the monitoring circuit shall give appropriate information to the system.

| Identifier | 42 |
| --- | --- |
| Title | digital distributed signal |
| Clue type | path-component component-only |
| Item | clock+oscillator |
| Clue Text | Are the rise and fall times of a clock signal acceptable? |

Explanation:	For instance, if a clock signal is distributed to many circuits, the input capacitances of each circuit add and the clock generator may deliver a degraded clock signal with a slow leading and trailing edges and possibly a distorted waveform and no clear leading edge.

| Identifier | 43 |
| --- | --- |
| Title | unused inputs |
| Clue type | component-only |
| Item | IC+and+or+nand+nor+not+JK+RS+flipflop+register+latch |
| Clue Text | Are integrated circuits unused inputs correctly polarized or grounded? |

Explanation:	Some integrated circuits include internal polarization on their inputs/outputs. For others, integrated circuit unused inputs shall not be left floating: open inputs cause decreased noise immunity, unnecessary power dissipation due to instability. For instance:

-	feedback between the output “-” of an OpAmp and “+” correctly polarized through a resistor

-	unused digital circuit inputs connected to the ground/power through a resistor.

| Identifier | 43A |
| --- | --- |
| Title | unused inputs tied to other inputs |
| Clue type | component-only |
| Item | IC+and+or+nand+nor+not+JK+RS+flipflop+register+latch |
| Clue Text | Are unused inputs tied to other inputs? |

Explanation:	Tying unused inputs to used inputs of the same circuit is not advisable as it reduces AC noise immunity due to increased input coupling capacitance.

| Identifier | 44A |
| --- | --- |
| Title | protection against transient voltage |
| Clue type | component-only |
| Item | power+supply+battery |
| Clue Text | Is there a protection against transient voltages? |

Explanation:	Fast protective diodes (TRANSIL) can be used to protect against transient overvoltages which may cause latch-up or even permanent failure. Zener diodes can also be used. A voltage protective device on each voltage source shall be implemented to prevent the circuits using these voltages from being damaged due to overvoltage.

| Identifier | 3B |
| --- | --- |
| Title | overflow |
| Clue type | component-only |
| Item | counter+shift+adder |
| Clue Text | Is there an overflow detection circuit? |

Explanation:	The out of limits detection circuit should ensure detection if the nominal range of measurement is saturated.

| Identifier | 44C |
| --- | --- |
| Title | protection against transient currents |
| Clue type | component-only |
| Item | power+supply+battery |
| Clue Text | Is there a protection against current transients? |

Explanation:	During switching, high currents may flow, especially if the load is partly capacitative, or if there are large motors. Sneak conditions may mean that more than the planned number of loads of this type are connected at one time.

| Identifier | 45 |
| --- | --- |
| Title | tristate outputs for multi-user line/bus |
| Clue type | component-only |
| Item | buffer+driver |
| Clue Text | Are interface circuits of tristate type when used on a bus? |

Explanation:	To avoid a short circuit between the different users sharing a single line or bus, tristate outputs shall be used.

| Identifier | 45A |
| --- | --- |
| Title | tristate outputs for multi-user line/bus |
| Clue type | component-only |
| Item | buffer+driver |
| Clue Text | Can tristate outputs to a bus be activated at the same time? |

Explanation:	To avoid a short circuit between the different users sharing a single line or bus, tristate outputs shall be only be activated one at a time.

| Identifier | 46 |
| --- | --- |
| Title | pull-up resistor for multi-user line/bus |
| Clue type | component-only |
| Item | buffer+and+or+nand+nor+not+IC+RAM+ROM |
| Clue Text | Is there one pull-up/down resistor attached to tristate outputs? |

Explanation:	There shall be a pull-up/down resistor attached to tristate outputs in order to polarize the line output during the high impedance state. When there is a bus, only one pull-up/down resistor is necessary per bus line in complex circuits, duplicate pull-up/down resistors may have been implemented on separated drawing sheets.

| Identifier | 47 |
| --- | --- |
| Title | pull-up resistor impedance |
| Clue type | component-only |
| Item | buffer+driver |
| Clue Text | Is the pull-up resistor value low compared with the high impedance value of the line/bus? |

Explanation:	If the pull-up resistor value is high or close to the high impedance value of the line/bus, the tri-state outputs connected to the line/bus may be polarized to a middle voltage value during the high impedance state of the bus and it may lead to a short circuit, especially for CMOS devices. The pull-up resistor value shall be lower than one tenth of the high impedance value of the line/bus.

| Identifier | 47A |
| --- | --- |
| Title | pull-up resistor impedance |
| Clue type | component-only |
| Item | buffer+driver |
| Clue Text | Is the pull-up resistor value low compared with the high impedance value of the line/bus? |

Explanation:	If the pull-up resistor value is high or close to the high impedance value of the line/bus, the tri-state outputs connected to the line/bus may be polarized to a middle voltage value during the high impedance state of the bus and it may lead to a short circuit, especially for CMOS devices. The pull-up resistor value shall be lower than one tenth of the high impedance value of the line/bus.

| Identifier | 50 |
| --- | --- |
| Title | decoupling capacitor values |
| Clue type | component-only |
| Item | IC+and+nand+or+nor+not+JK+RS+flipflop+register+latch |
| Clue Text | Has a decoupling capacitor been provided? |

Explanation:	The high processing frequency of the logic circuits leads to current spikes and then to voltage spikes because of the inductive coupling between conductors at high frequency. Only a decoupling capacitor close to the power pins can prevent the spikes from propagating to the other circuits.

| Identifier | 51 |
| --- | --- |
| Title | timing compatibility |
| Clue type | component-only |
| Item | IC+and+nand+or+nor+not+JK+RS+flipflop+register+latch |
| Clue Text | Is the timing compatibility between circuits respected? |

Explanation:	This problem is likely to occur between circuits of different families or when an analogue output is connected to a digital input. If one of the specified parameters cannot be guaranteed, it can lead to a sneak timing. For CMOS digital circuits, rise and fall time shall not exceed 15 microseconds; otherwise, an erroneous output can be generated, and power dissipation increases.

| Identifier | 52 |
| --- | --- |
| Title | energy storage/release |
| Clue type | path-component component-only |
| Item | capacitor |
| Clue Text | Can the energy stored by the component be released without damage? |

Explanation:	As capacitors may retain stored charge once the power is removed, capacitors can lead to latch up, destroy sensitive circuits, delay the power off of the function or create sneak current paths. A discharge circuit (bleed resistor in parallel with a high value capacitor) may be provided to drain away the charge after the circuit is switched off. 
The system or circuit shall be designed such that the connection between the capacitor and its bleed resistor cannot be broken except during repair.
When a circuit output connected to a capacitor changes state (digital circuit), the capacitor may generate an unexpected voltage spike, dangerous for the components receiving this spike.
A capacitor may keep circuit inputs powered while the circuit is not powered. This can induce sneak current paths through internal circuits protection diodes, resistors, transistors, ...

| Identifier | 52A |
| --- | --- |
| Title | energy storage/release |
| Clue type | path-component component-only |
| Item | inductor+coil+relay+transformer+motor+trafo |
| Clue Text | Can the energy stored by the component be released without causing damage? |

Explanation:	A magnetic field is the energy source that can produce large voltage transients when it collapses, particularly when an equipment is  shut down or switched to standby.

| Identifier | 52B |
| --- | --- |
| Title | lack of surge resistor |
| Clue type | path-component component-only |
| Item | capacitor |
| Clue Text | No surge resistor to protect an IC? |

Explanation:	Integrated circuits should be protected against large capacitances connected to their input by placing a resistor in series.

| Identifier | 52C |
| --- | --- |
| Title | diode prevents energy release |
| Clue type | path-component component-only |
| Item | capacitor |
| Clue Text | Can a diode prevent energy release from the component? |

Explanation:	If a diode is connected in series with a capacitor (to protect it against reverse voltage perhaps), check that it does not prevent the capacitor from releasing its energy.

| Identifier | 53 |
| --- | --- |
| Title | activation by leakage current |
| Clue type | path-component component-only |
| Item | diode+resistor+transistor+gate+and+or+nand+nor+capacitor |
| Clue Text | Can the leakage current of the component activate or inhibit digital devices and lead to (un)expected functions or loss of function? |

Explanation:	For instance, during unusual operating modes, the relay coil may be fed from a low voltage or a high impedance source and can lead to a sneak current path. Sensitive amplifiers, FETs, and high input impedance devices are particularly susceptible to problems.

| Identifier | 54 |
| --- | --- |
| Title | acceptable charge/discharge current |
| Clue type | component-only |
| Item | capacitor |
| Clue Text | Is the charge/discharge current flowing through the capacitor within the specified rating of the capacitor and other involved components to which it is connected? |

Explanation:	Depending on the capacitor technology, the current flowing through the capacitor shall not exceed a specified value. As this current may reach high intensity, its influence on other items (fuses, transistor, voltage spikes,...) shall be carefully studied in order to respect the appropriate ratings.

| Identifier | 54A |
| --- | --- |
| Title | high switching current for tantalum capacitors |
| Clue type | component-only |
| Item | capacitor |
| Clue Text | Is there a possibility for a high switching current, and is the capacitor a tantalum type? |

Explanation:	For polarized solid tantalum capacitors, the charge and the discharge current flowing through the capacitor should be limited under 0.1 W/V (a resistor can be used to limit the current).
Current through ceramic capacitors should not exceed 50mA.

| Identifier | 55 |
| --- | --- |
| Title | voltage delay during power transition |
| Clue type | component-only |
| Item | capacitor |
| Clue Text | Are different capacitor charge and discharge times within a circuit acceptable? |

Explanation:	If some capacitors reach operating voltage before others, the designer shall ensure that it will not lead to an unexpected current path or a temporary wrong processing of a function.

| Identifier | 55A |
| --- | --- |
| Title | capacitor used as status indicator |
| Clue type | component-only |
| Item | capacitor |
| Clue Text | Is a capacitor voltage used to feed a status indicator or sensor? |

Explanation:	If a capacitor voltage is used (directly or not) as a status information by a monitoring circuit, the capacity may induce a delay in monitoring.

| Identifier | 56 |
| --- | --- |
| Title | protection against reverse voltage |
| Clue type | component-only |
| Item | inductor+coil+relay+transformer+motor |
| Clue Text | Is the overvoltage generated when switching a winding suppressed? |

Explanation:	If the current drift/direction changes or if the current is switched off in inductors, coils or transformers, an unlimited reverse voltage is generated, with possible oscillations. This reverse overvoltage is likely to destroy any sensitive components located nearby.
To suppress this overvoltage, a diode (with a reverse coupled Zener diode in series) is usually implemented in parallel as a protection. The Zener diode increases the forward resistance of the diode in order to lower the oscillation time constant. Without Zener diode, the time constant is increased by a factor of 5 to 10, and the reverse voltage burn-out of the diode shall be carefully considered.

| Identifier | 56A |
| --- | --- |
| Title | protection against reverse voltage |
| Clue type | component-only |
| Item | inductor+coil+relay+transformer+motor |
| Clue Text | Is the overvoltage generated when switching a winding suppressed? |

Explanation:	If the current drift/direction changes or if the current is switched off in inductors, coils or transformers, an unlimited reverse voltage is generated, with possible oscillations. This reverse overvoltage is likely to destroy any sensitive components located nearby.
To suppress this overvoltage, a diode (with a reverse coupled Zener diode in series) is usually implemented in parallel as a protection. The Zener diode increases the forward resistance of the diode in order to lower the oscillation time constant. Without Zener diode, the time constant is increased by a factor of 5 to 10, and the reverse voltage burn-out of the diode shall be carefully considered.

| Identifier | 57 |
| --- | --- |
| Title | correct signal for mode |
| Clue type | path-component |
| Item | switch+capacitor+battery+power+supply |
| Clue Text | Does the signal remain within its expected voltage range for the expected switching conditions? |

Explanation:	During particular phases (e.g. testing) or change of modes, some switching combinations may be activated with a sequence or order quite different from that expected by the designer. Then, the signal may:
1) exceed (positive or negative) the voltage supply of a circuit dealing with this signal, leading to a sneak current path or an erroneous processing (operational amplifier, comparator, digital circuit, transistor)
2) remain within a medium voltage range unacceptable for the circuit (digital circuit, MOSFET,...)
3) remain floating, indeterminate
4) simply activate the wrong function(s) for this mode. Reference signal or polarization signals shall be checked as well. Normally, bias conditions should not be dependent to circuit configuration and switching modes, except after particular and careful study. The collector junction of a bipolar transistor shall not be inadvertently forward biased under some switching, operational or testing modes.

| Identifier | 58 |
| --- | --- |
| Title | fan out/fan in rating |
| Clue type | component-only |
| Item | IC+and+or+nand+nor+not+RS+JK+flipflop+register+latch+ buffer+active+EED |
| Clue Text | Does the component loading remain within the specified ratings? |

Explanation:	The common assumption that a TTL device can drive ten devices is not always true. Fan-out limits should be calculated by summing input current requirements and comparing with the output source current

| Identifier | 10B |
| --- | --- |
| Title | spike on the initialization signal |
| Clue type | component-only |
| Item | digital+IC+and+nand+or+nor+not+gate+RS+JK+flipflop+register+latch+tristate |
| Clue Text | Can a spike occur on the initialization signal? |

Explanation:	The initialization signal shall be protected against spikes to prevent spurious initialization of the equipment.

| Identifier | 13 |
| --- | --- |
| Title | transient current through a relay contact |
| Clue type | component-only |
| Item | relay+contact |
| Clue Text | Is an unregulated flow of current through the relay contacts acceptable? |

Explanation:	During power transition or switching for instance, high inrush current may occur when capacitive loads are switched. The relay shall be operated within the manufacturer’s specified ratings.

| Identifier | 58AA |
| --- | --- |
| Title | load rating |
| Clue type | component-only |
| Item | IC+and+or+nand+nor+not+RS+JK+flipflop+register+latch+ buffer+active+EED |
| Clue Text | Does the component load remain within the specified ratings? |

Explanation:	Is the fan out rating exceeded.

| Identifier | 58A |
| --- | --- |
| Title | load ratings |
| Clue type | component-only |
| Item | flipflop+register+latch+buffer+OpAmp+IC |
| Clue Text | Are load ratings exceeded during abnormal or unusual operational states? |

Explanation:	During unscheduled sequences and modes (e.g. testing) or switching, a load with an impedance lower than expected or with different characteristics (inductive instead of resistive) can lead to permanent failure or a sneak current path with potential hazards.
Particular attention should be paid to integrated circuits (e.g. OpAmp, comparator) or semiconductors connected to capacitors; during power transition for instance, if a capacitor is used as a feedback with an OpAmp/comparator, the drive capability of the OpAmp/comparator output may be exceeded. The same is true if the capacitor is connected from input to ground, or from output to ground. To provide a series limiting resistance is a solution in some cases.

| Identifier | 58AAA |
| --- | --- |
| Title | load ratings |
| Clue type | component-only |
| Item | flipflop+register+latch+buffer+OpAmp+IC |
| Clue Text | Are load ratings exceeded during abnormal or unusual operational states? |

Explanation:	During unscheduled sequences and modes (testing, ...) or switching, a load with an impedance lower than expected or with different characteristics (inductive instead of resistive) can lead to the circuit destruction or a sneak current path, with potential hazards.
Particular attention should be paid to integrated circuits (OpAmp, comparator,...) or semiconductors connected to capacitors; during power transition for instance, if a capacitor is used as a feedback with an OpAmp/comparator, the drive capability of the OpAmp/comparator output may be exceeded. The same is true if the capacitor is connected from input to ground, or from output to ground. To provide a series limiting resistance is a solution in some cases.

| Identifier | 58B |
| --- | --- |
| Title | load ratings for EED |
| Clue type | component-only |
| Item | active+EED |
| Clue Text | Can the circuit supply sufficient energy to blow the EED under all operational conditions? |

Explanation:	For a squib component, sufficient energy shall be delivered by the output to blow it. For some switching conditions, there may be too much resistance or inductance in the activating circuit.

| Identifier | 59 |
| --- | --- |
| Title | internal unexpected link of ICs pins |
| Clue type | once-only |
| Item | IC+path+JK+buffer+RS+flipflop+register+counter |
| Clue Text | Has a possible internal link of ICs pins been taken into account for the design? |

Explanation:	An unexpected internal link between pins of an integrated circuit can lead to a circuit malfunction or a sneak current path.

| Identifier | 60 |
| --- | --- |
| Title | low voltage drift with time |
| Clue type | once-only |
| Item | capacitor |
| Clue Text | Have appropriate capacitors been used when a circuit needs a low voltage drift with time? |

Explanation:	For instance, for sample & hold amplifiers with low voltage drift with time, teflon, polyethylene or polycarbonate capacitors are the best suited.

| Identifier | 61 |
| --- | --- |
| Title | temperature stability problems |
| Clue type | once-only |
| Item | capacitor |
| Clue Text | Is an appropriate capacitor type used if the circuit needs a good temperature stability? |

Explanation:	For instance, for low passing active filter needing a good temperature stability, silvered mica capacitors are the best suited.

| Identifier | 62 |
| --- | --- |
| Title | balanced pair of transistors |
| Clue type | component-only |
| Item | transistor+PNP+NPN |
| Clue Text | If there are differential instrumentation entrance or push-pull output stage made with transistors, are the transistors perfectly balanced? |

Explanation:	The transistors shall be balanced to avoid saturation or clamping.

| Identifier | 63 |
| --- | --- |
| Title | connected tristate outputs enabled |
| Clue type | component-only |
| Item | buffer+driver |
| Clue Text | Can connected tristate outputs be enabled at the same time? |

Explanation:	On multi-user line/bus, only one user provide an output to the line; otherwise, if two users provide simultaneous outputs, erroneous information may be transmitted and a short circuit may also occur. 
Tying totem-pole outputs together may cause a power-to-ground short-circuit, if one output drives a “1” logic level and another a “0” logic level.

| Identifier | 65 |
| --- | --- |
| Title | correct state during initialization |
| Clue type | component-only |
| Item | breaker+relay+switch |
| Clue Text | During power on or initialization, is the component in a correct predefined state? |

Explanation:	During initialization, the relay state shall be clearly established.

| Identifier | 65A |
| --- | --- |
| Title | correct state during initialization |
| Clue type | component-only |
| Item | transistor |
| Clue Text | During power on or initialization, is the component in a correct, predefined state? |

Explanation:	During initialization, high impedance outputs shall have appropriate polarization resistors: e.g. not to bias a NPN base.

| Identifier | 20D |
| --- | --- |
| Title | switching item dedicated to a function |
| Clue type | component-only |
| Item | switch+contact+breaker |
| Clue Text | When a protective device is activated, are other functions than those expected affected? |

Explanation:	With elaborate equipments, some combinations may lead to unexpected behaviour of functions or inhibition of functions.

| Identifier | 21 |
| --- | --- |
| Title | saturated or clipped signal |
| Clue type | component-only |
| Item | OpAmp+amplifier+transistor |
| Clue Text | Can an output signal be saturated or clipped? |

Explanation:	For instance, with an integrator, the saturation of the feedback capacitor induces a saturated signal and an erroneous output signal. 
When amplifiers are used (OpAmp, transistor), if the gain is not correctly adjusted, the output signal may be saturated. If the amplifier polarization is not symmetrical, the output signal may saturate asymmetrically.

| Identifier | 21A |
| --- | --- |
| Title | saturated or clipped signal input |
| Clue type | component-only |
| Item | OpAmp+amplifier+transistor |
| Clue Text | Can an input signal be saturated or clipped? |

Explanation:	For instance, with an integrator, the saturation of the feedback capacitor induces a saturated signal and an erroneous output signal.

| Identifier | 22 |
| --- | --- |
| Title | self-healing of plastic film capacitors |
| Clue type | component-only |
| Item | capacitor |
| Clue Text | Is there enough energy in the capacitor for self healing? |

Explanation:	Recommended minimum energy for self-healing is about 500mJ.

| Identifier | 65B |
| --- | --- |
| Title | correct state during initialization |
| Clue type | component-only |
| Item | IC+latch+flipflop |
| Clue Text | During power on or initialization, is each item/circuit in a correct predefined state? |

Explanation:	During initialization, latch/register inputs shall have appropriate polarization resistors in order to retain the correct logic state when the initialization phase is completed and the computer has still not accessed to the latch/register, or they shall be initialised dynamically.

| Identifier | 66 |
| --- | --- |
| Title | correct initialization time |
| Clue type | component-only |
| Item | JK+RS+latch+register+counter+flipflop+and+RAM+ROM+micropro |
| Clue Text | Is the initialization time long enough to be compatible with each component signal specification and the longest transition time of the power sources? |

Explanation:	Some circuit pins need a minimum initialization time (RESET, CLEAR, SET,...). A too short initialization time may not correctly reset a microcontroller or a register state ; it may enable outputs which are connected to circuits not still nominally powered.

| Identifier | 66A |
| --- | --- |
| Title | correct power settling time |
| Clue type | component-only |
| Item | JK+RS+latch+register+counter+flipflop+and+RAM+ROM+micropro |
| Clue Text | Is the initialization time long enough to be compatible with the settling time of the power supply? |

Explanation:	The initialization signal (RESET, CLEAR, SET,...) needs to be held until the power supply level is steady at all parts of the circuit. A too short initialization time may not correctly reset a microcontroller or a register state ; it may enable outputs which are connected to circuits not still nominally powered.

| Identifier | 67 |
| --- | --- |
| Title | switching item shared by several function |
| Clue type | path-component component-only |
| Item | switch+contact+relay+and+gate+breaker |
| Clue Text | Is a change of state of a switching item for one function acceptable for correct performance off other functions? |

Explanation:	A switching item can be shared through a same contact, or through multiple contacts activated at the same time.
If a circuit-breaker is triggered, its change of state will affect all the functions connected to it. Check that it does not steal power from a function that should still be powered.

| Identifier | 19A |
| --- | --- |
| Title | sufficient available power |
| Clue type | component-only |
| Item | transformer+power+supply+battery |
| Clue Text | Can the power source meet the maximum demand? |

Explanation:	Under some particular conditions (e.g. test, reconfiguration), more power than expected may be consumed by the loads:

-	in the case where multiple loads are to be supplied, an unfavourable combination of loads may exceed the capability of the power supply

-	a load may draw much more power than normal under anomalous circumstances (resistive torque on a motor). If the power equipment contains a current limiter, the power source shall also be able to support the activation of the current limiter, specially if the source is shared by other equipments.

| Identifier | 69 |
| --- | --- |
| Title | false indication during switching |
| Clue type | component-only |
| Item | relay+switch+breaker+button |
| Clue Text | During switching, can a false indication occur or an unintended function be activated? |

Explanation:	If the contact “chatters” or “bounces” during switching, the signal may be detected as multiple inputs. Using an electronic switch (such as a transistor) instead of a relay may provide a solution.
If a monitoring circuit checks a relay status just after the relay has changed of state, erroneous information may be transmitted as the relay contacts can rebound for a short time after switching. A delay in the status acquisition is necessary to monitor a stable status.
A capacitance filter or a monostable circuit may be fitted to produce a reliable deterministic switching signal.

| Identifier | 70 |
| --- | --- |
| Title | label ambiguity |
| Clue type | component-only |
| Item | label+lamp+indicator |
| Clue Text | Have each electrical indicator, command and signal its own unique label? |

Explanation:	To allow an easier management of the electrical drawing and sneak label detection, each signal and connection shall have its own unique label.
Labelling shall be precise : for instance, the wires attached to either side of a fuse shall have different labels.
Quasi-similar labels for independent indicators, commands and signals shall be avoided.

| Identifier | 71 |
| --- | --- |
| Title | unterminated leads |
| Clue type | component-only |
| Item | electric+electromechanical+electronic |
| Clue Text | Are there any open ended or nonterminated outputs? |

Explanation:	In a circuit each extremity of a wire shall normally be connected to a component. If a lead is connected to a flag that makes it available for another sheet of the drawing, the lead shall continue on the new sheet under the same label.

| Identifier | 71A |
| --- | --- |
| Title | unconnected leads due to switching |
| Clue type | component-only |
| Item | IC+OpAmp+JK+RS+flipflop+component |
| Clue Text | Is there any open ended or nonterminated outputs for some switching state? |

Explanation:	Component terminal wires shall be connected in all switch states.

| Identifier | 72 |
| --- | --- |
| Title | consistent labelling |
| Clue type | component-only |
| Item | label+indicator+lamp+speaker |
| Clue Text | Is the label wording consistent with others labels? |

Explanation:	The wording labels should have uniform style, be unique, and identifiable. As a suggestion, the master equipment which delivers the signal can be used as a common root for the label wording.

| Identifier | 73 |
| --- | --- |
| Title | exhaustive labelling |
| Clue type | component-only |
| Item | label+lamp+indicator+speaker |
| Clue Text | Are all the circuit applications clearly indicated in a label name? |

Explanation:	To allow an easier management of the electrical drawing and sneak label detection, each indicator, command and signal shall be clearly labelled to indicate all of its functions. 
For instance, if a switch is on a ground return of several loads (Y pattern or power dome configuration), its command label shall indicate all of the loads affected.

| Identifier | 74B |
| --- | --- |
| Title | independence of timing of switching functions |
| Clue type | path-component component-only |
| Item | switch+button+relay+gate+and |
| Clue Text | Is independent timing of switching items justified? |

Explanation:	When multiple switches shall be activated, if a confusion in the activation order can have critical impacts, then the temporal independence between these switches shall be suppressed, or safety barriers shall be added.
If several switches shall be activated at the same moment (to cut off of the 3 phases of the main power for instance), these switches should be activated by the same command or signal.

| Identifier | 75 |
| --- | --- |
| Title | coherent labelling of inter-board leads |
| Clue type | component-only |
| Item | label+connector |
| Clue Text | Is an interface label used on separated boards the same on all boards? |

Explanation:	If the labels are different, there may be problems during the routing on the board ; signals may not be connected together.

| Identifier | 76 |
| --- | --- |
| Title | uniform logic conventions |
| Clue type | component-only |
| Item | lamp+switch+button |
| Clue Text | Do the indications provided by all the monitoring circuits following the same common logic conventions? |

Explanation:	Usually, the level “1” corresponds to “power presence” and the level “0” to “lack of power”. A red lamp lights up for a problem and a green lamp indicated that the monitored function is OK.

| Identifier | 77 |
| --- | --- |
| Title | impedance matching |
| Clue type | component-only |
| Item | buffer+amplifier+driver |
| Clue Text | Are interface lines terminated correctly? |

Explanation:	If the rise time of a digital signal is short compared with the propagation time of the signal (on a long line or through an interface), signal reflections may induce parasite voltages and erroneous information.

| Identifier | 78 |
| --- | --- |
| Title | truth table for digital function |
| Clue type | component-only |
| Item | IC+and+or+nand+nor+not+ASIC+PLA |
| Clue Text | Has each state of a truth table for a digital function been checked? |

Explanation:	The truth table allows an exhaustive check of every possible input state.

| Identifier | 81 |
| --- | --- |
| Title | undesired state inhibition |
| Clue type | component-only |
| Item | IC+flipflop+JK+RS+register+latch+counter |
| Clue Text | Is the change of the component to an undesired state inhibited? |

Explanation:	For instance, resetting a counter may be undesired due to its effects on the equipment: consequently, the counter reset shall be inhibited and additional circuits may be necessary to prevent the counter from passing by the “0” state when counting.

| Identifier | 1 |
| --- | --- |
| Title | simultaneous activation of mutually exclusive inputs |
| Clue type | component-only |
| Item | bistable+flip-flop+register+latch+counter+JK+RS+bistabrel |
| Clue Text | Can mutually exclusive commands (STOP/START & RESET/SET) be generated at the same time? |

Explanation:	The RESET/SET & STOP/START shall not occur at the same time otherwise the item state will be undetermined.

| Identifier | 2 |
| --- | --- |
| Title | transient current through a fuse or breaker |
| Clue type | component-only |
| Item | fuse+breaker |
| Clue Text | Is a transient current flowing through a fuse (a breaker) within specified limits? |

Explanation:	A fuse shall withstand without rapturing the high transient currents which may occur during power switching or during a power transition. The fuse should be chosen in accordance with the manufacturer’s recommendations.

| Identifier | 3A |
| --- | --- |
| Title | saturation detection |
| Clue type | component-only |
| Item | sensor+fsens+psens+tsens+vsens |
| Clue Text | Is there an out-of-limits or saturation detection circuit? |

Explanation:	The sensors should be selected in such a way that the detection circuit ensures detection of the out-of-limits or saturation in the system.

| Identifier | 15A |
| --- | --- |
| Title | discharge circuit for a power-source |
| Clue type | path-component component-only |
| Item | power+battery+supply |
| Clue Text | Is the power source provided with a circuit that can isolate it from its load? |

Explanation:	For safety reasons and to avoid sneak current paths at interface between circuits with different supplies, once an equipment has been switched off, the power source shall be prevented from continuing to supply power.

| Identifier | 16 |
| --- | --- |
| Title | stray capacitance |
| Clue type | component-only |
| Item | IC+and+or+nand+nor+not+flipflop+JK+RS+register+latch |
| Clue Text | Are circuits using capacitors at pF levels sensitive to the capacitor tolerance? |

Explanation:	When the circuit is to be implemented on a PCB, the parasitic capacitance of the PCB tracks may alter the proper functioning of the circuit. The input capacitance of circuits may also induce parasitic capacitance. Parasitic capacitance may cause delay in functioning, failure to switch, poor signal form, oscillation.

| Identifier | 165 |
| --- | --- |
| Title | secondary load viewed from the transformer primary |
| Clue type | component-only |
| Item | transformer |
| Clue Text | Is the secondary load viewed from the transformer primary acceptable? |

Explanation:	To determine the load seen by the transformer primary, the transformation ratio shall be taken into account. 
If a capacitor is connected in parallel with a secondary winding of a transformer (filter in a direct current converter), during power on, the capacitor charging induces a “short circuit” at the transformer primary: a current limiter may be implemented to cancel this problem.

| Identifier | 166 |
| --- | --- |
| Title | energy received by an indicator |
| Clue type | component-only |
| Item | lamp+LED+loudspeaker+speaker+buzzer+bell |
| Clue Text | Is the energy received by the indicator correct? |

Explanation:	If a low current flows through a lamp or LED, the indicator may not be seen if lights up, depending of the room brightness. An audible alarm (loud-speaker, buzzer) shall have enough power available for the transducer, especially if it is placed in a noisy room. This may have critical effects if it is an alarm indicator! 
On the other hand, a limitation resistor may sometimes be implemented (LED).

| Identifier | 167 |
| --- | --- |
| Title | command duration adapted to circuit characteristics |
| Clue type | component-only |
| Item | relay+flipflop+JK+RS+register+monoflop |
| Clue Text | Is the command duration matched to the circuit characteristic? |

Explanation:	For monostable relays, the command shall be maintained as long as the change of state is desired. For bistable relays, once the state has changed, the command is no longer needed; but to change state of bistable relay, a minimum duration command is necessary to reach a sufficient magnetic field on the coil.

| Identifier | 168 |
| --- | --- |
| Title | current limiters in series |
| Clue type | component-only |
| Item | limiter+fuse+relay+breaker |
| Clue Text | Are current limiters in series correctly set? |

Explanation:	Both the maximum current limit and the reaction delay shall be considered. Normally, down-stream current limiter shall first react before other up-stream current limiters.

| Identifier | 82 |
| --- | --- |
| Title | multiple paths of a signal |
| Clue type | path-component component-only |
| Item | connector+malecon+femcon |
| Clue Text | Does the same signal flow through separated lines or connectors? |

Explanation:	This arrangement shall be avoided as the separated wires of the signal can recombine through other boards and it can lead to sneak current paths. Also, low current and high current returns connected in parallel shall be avoided in case of the high current disconnection.

| Identifier | 82A |
| --- | --- |
| Title | multiple earth returns for a signal |
| Clue type | path-component component-only |
| Item | earth+chassis |
| Clue Text | Is there an earth return flow through separated lines or connectors? |

Explanation:	This arrangement shall be avoided as the separated wires in the earth return can lead to inductance effects, conduction path overload, and errors during disconnection.

| Identifier | 82AA |
| --- | --- |
| Title | multiple earth returns for a signal |
| Clue type | path-component component-only |
| Item | earth+chassis |
| Clue Text | Is there an earth return flow through separated lines or connectors? |

Explanation:	This arrangement shall be avoided as the separated wires in the earth return can lead to inductance effects, conduction path overload, and errors during disconnection.

| Identifier | 82B |
| --- | --- |
| Title | connector in earth return line |
| Clue type | path-component component-only |
| Item | connector+malecon+femalecon |
| Clue Text | Is there a connector in the earth return line, and a parallel path which bypasses it? |

Explanation:	This arrangement shall be avoided as the parallel earth return can cause a sneak timing or a sneak circuit.

| Identifier | 82BB |
| --- | --- |
| Title | connector in earth return line |
| Clue type | path-component component-only |
| Item | earth+chassis+ground |
| Clue Text | Is there a connector in the earth return line, and a parallel path which bypasses it? |

Explanation:	This arrangement shall be avoided as the parallel earth return can cause a sneak timing or a sneak circuit.

| Identifier | 83 |
| --- | --- |
| Title | signal on a power line |
| Clue type | path-component component-only |
| Item | and+or+nand+nor+gate+flipflop+RS+JK+register+amplifier+ transistor+comparator |
| Clue Text | Is the signal perturbation due to the power line acceptable? |

Explanation:	Spikes occurring on a power line can generate high frequency interference which can alter signal characteristics. The potential difference due to the interference current flowing through the line may not be negligible compared to the signal.

| Identifier | 84A |
| --- | --- |
| Title | power and signal lines in separated connectors |
| Clue type | component-only |
| Item | multiconnector |
| Clue Text | Does the connector contain both power and signal lines? |

Explanation:	It is necessary to route signal and power lines through separate connectors. It can lead to the complete destruction of the equipment if a short circuit between a signal and a power line occurs. The electromagnetic interferences may induce random/false data on digital signals and modify an analogue signal characteristic.

| Identifier | 84B |
| --- | --- |
| Title | disconnecting separated connectors |
| Clue type | component-only |
| Item | multiconnector+connector+interlock+malecon+femalecon |
| Clue Text | Is it possible to disconnect power lines in separated connectors? |

Explanation:	Opening just one connector from a set of power and earth or return connectors is a frequent cause of sneaks.

| Identifier | 86A |
| --- | --- |
| Title | irreversible state after switching |
| Clue type | path-component component-only |
| Item | relay+and+nor+flipflop |
| Clue Text | Can switching lead to an irreversible state? |

Explanation:	For instance, the command circuit of a relay may be powered through the relay; once the relay has been activated, the command circuit is no more powered and can no longer be activated, so the relay remains in an irreversible state. A relay contact or gate on a single path branch shall not be commanded by a circuit supplied through another branch: once the relay contact is open, the command circuit of the relay is no more supplied and the relay contact can never be closed.

| Identifier | 89 |
| --- | --- |
| Title | decoupling capacitor for each board |
| Clue type | component-only |
| Item | supply+battery+power |
| Clue Text | Has a decoupling capacitor been provided for each direct current power signal at the entrance of each board? |

Explanation:	Install a decoupling capacitor for each power line at the point where it enters each board to ensure a “clean” power supply free of spikes (mF) and radio-frequency parasites (nF).

| Identifier | 89A |
| --- | --- |
| Title | decoupling capacitor for each board |
| Clue type | component-only |
| Item | connector+multiconnector+malecon+femcon |
| Clue Text | Has a decoupling capacitor been provided for each direct current power signal at the entrance of each board? |

Explanation:	To install a decoupling capacitor for each power signal at the point where it enters each board ensure a “clean” power supply free of spikes (mF) and radio-frequency (nF).

| Identifier | 92 |
| --- | --- |
| Title | quality of the contact material |
| Clue type | once-only |
| Item | relay+switch+interlock+contact+button |
| Clue Text | Is the contact material correctly chosen with respect to the signal characteristics and the relay application? |

Explanation:	For low loads (e.g., 1 mA, 5 V) gold plated contacts are required. For very low loads (e.g. a few mA, 1 Volt), use gold or silver/gold contacts (e.g. thermo-couple switching).
For repetitive functioning, use silver/cadmium contacts.

| Identifier | 93 |
| --- | --- |
| Title | contacts and inductive load switching |
| Clue type | component-only |
| Item | switch+contact+breaker |
| Clue Text | Are the contacts protected against high voltage spikes? |

Explanation:	If the load seen by the contacts is inductive, a resistor with a capacitor in series or a voltage suppression diode can be added in parallel to the relay contacts; but magnetic blowing relays or high cut distance relays are preferable.

| Identifier | 98 |
| --- | --- |
| Title | low impedance between power sources |
| Clue type | path-component component-only |
| Item | power+supply+battery |
| Clue Text | Can a low impedance tie two power sources? |

Explanation:	Under some switching conditions, a low impedance load may be connected between two power sources and create a short circuit: relay-coil, inductor, motor, transformer, diode, wire, ...

| Identifier | 99 |
| --- | --- |
| Title | temperature limit for multi layer tantalum capacitors |
| Clue type | once-only |
| Item | capacitor |
| Clue Text | Can the temperature of a multi layer tantalum capacitor exceed 38 °C? |

Explanation:	The anode wire of multi-layer tantalum capacitors intersects when temperature is above 38 °C.

| Identifier | 100 |
| --- | --- |
| Title | reverse voltage on a polarized capacitor |
| Clue type | component-only |
| Item | capacitor |
| Clue Text | Can a reverse voltage be applied to the polarized capacitor? |

Explanation:	If a reverse voltage is applied to a polarized capacitor, the capacitor will burst. A protection diode against reverse voltage in parallel with the capacitor can be added.

| Identifier | 101 |
| --- | --- |
| Title | capacitive feedback between an amplifier’s input and output |
| Clue type | component-only |
| Item | OpAmp+amplifier |
| Clue Text | For an amplifier with a capacitive feedback, does the amplifier output remain stable? |

Explanation:	Capacitive feedback between the input and the output of an OpAmp can lead to instability in an unexpected frequency range. These oscillations may superpose upon the nominal signal and induce an erroneous processing.

| Identifier | 102 |
| --- | --- |
| Title | compensation of amplifier input capacitance |
| Clue type | component-only |
| Item | OpAmp+transistor+amplifier |
| Clue Text | Has the amplifier input capacitance been compensated? |

Explanation:	Capacitive feedback compensates for input capacitance of an OpAmp or a transistor (FET), but also increases the risk of instability. A check is necessary to prevent this risk.

| Identifier | 103 |
| --- | --- |
| Title | hysteresis for noise immunity |
| Clue type | component-only |
| Item | comparator+OpAmp |
| Clue Text | Has hysteresis been implemented to improve noise immunity? |

Explanation:	For a comparator, an hysteresis can be implemented by positive feedback (usually a resistor) between the output and the non-inverting input. If no hysteresis is implemented, depending of the signal noise at the inputs, the output may “bounce” during the change of state.

| Identifier | 104 |
| --- | --- |
| Title | adjustment of input impedance |
| Clue type | component-only |
| Item | OpAmp+transistor+amplifier |
| Clue Text | Has the input impedance been adjusted? |

Explanation:	For instance, to compensate the input offset errors of an amplifier, equalizing input resistors shall be implemented to allow adjustment. If the offset has an effect on the correct circuit processing, these input resistors are imperative. Voltage converters or current converters made with OpAmps should have a resistor on the non-inverting input adjusted with the feedback resistor. 
If the feedback is made via a complex network, a complex network should also be provided on the input impedance.

| Identifier | 105 |
| --- | --- |
| Title | commutation time of transistor |
| Clue type | component-only |
| Item | transistor |
| Clue Text | Are the turn-on and turn-off times for the transistor low compared with the stable state time? |

Explanation:	During a commutation in a (power) transistor, the power dissipation partly comes from the switching times, as both voltage and current are applied to the transistor during the switching. To improve the efficiency, turn-on and turn-off times shall be as small as possible: use commutation accelerators. To reduce the switching times also allows to lower the transistor case temperature.

| Identifier | 109 |
| --- | --- |
| Title | plug |
| Clue type | path-component component-only |
| Item | latch+register+flip-flop+monoflop+bistable+JK+RS+counter+ transistor+rectifier+breaker+limiter+relay+monostable+switch+ contact+button+fuse+interlock |
| Clue Text | Can switching lead to an unstable state? |

Explanation:	Feedback around a loop of digital components can lead to instability if the number of signal reversals is zero or even.

| Identifier | 109A |
| --- | --- |
| Title | unstable or irreversible state |
| Clue type | path-component component-only |
| Item | relay |
| Clue Text | Can a switching lead to an unstable/irreversible state? |

Explanation:	The switching of a relay can activate an unwanted series of events in which another relay changes of state and subsequently activates the first one.
If the relays are monostable, this situation leads to an unstable state. If one relay is bistable, the switching leads to an irreversible state.

| Identifier | 110 |
| --- | --- |
| Title | continuous current path via an autotransformer |
| Clue type | path-component component-only |
| Item | autotransfo |
| Clue Text | Can an unexpected continuous current flow through the autotransformer? |

Explanation:	An autotransformer has no galvanic isolation between input and output. A continuous current can flow through the windings and create an unexpected/sneak current path.

| Identifier | 111 |
| --- | --- |
| Title | parallel diodes for redundancy/power sharing |
| Clue type | component-only |
| Item | diode |
| Clue Text | Is the current flowing through parallel diodes equally divided or does it flow only through one diode? |

Explanation:	-	Either one diode can carry all the current itself and the other diode is for redundancy: then no current should (not obligatory) flow through the second diode to improve its life time as a “cold” redundancy.

-	Or one diode cannot carry all the current itself and the current shall be shared with the other diode: half the current flowing through the two diodes and then a balance device shall be implemented as the two diodes will not have the same diode voltage. One shall insure that unequal division will not occur.

| Identifier | 112 |
| --- | --- |
| Title | correct biasing of Zener diodes |
| Clue type | component-only |
| Item | Zener |
| Clue Text | Is the Zener diode correctly biased? |

Explanation:	When a Zener diode is used as voltage reference it shall conduct a sufficiently large current to ensure that it operates in the breakdown region of its characteristic sufficiently far from the “knee”. This ensures a stable reference voltage. However, the power dissipated in the Zener diode should not exceed the maximum permitted value.

| Identifier | 113 |
| --- | --- |
| Title | inductance of a wire wound resistor |
| Clue type | component-only |
| Item | resistor |
| Clue Text | Has the inductive effect of the wound wire resistor been taken into account? |

Explanation:	For a quasi direct current signal, the inductance should have negligible effect. At high frequencies, the inductance will not be negligible and will add to the impedance of the resistor, possibly leading to unexpected results.

| Identifier | 115A |
| --- | --- |
| Title | connected power sources |
| Clue type | path-component component-only |
| Item | power+supply+battery |
| Clue Text | Is it acceptable to connect one reference voltage of the power source to a reference voltage of an other power source? |

Explanation:	Some power sources may be separated by galvanic isolation. To connect these power sources (power-to-power or ground-to-ground) may undo the benefit of galvanic isolation. It can also create unexpected high current paths as the reference point of each power source may not have exactly the same voltage.

| Identifier | 5B |
| --- | --- |
| Title | maximum signal frequency |
| Clue type | component-only |
| Item | IC+gate+and+or+nand+nor+not |
| Clue Text | Have the maximum signal frequency been taken into account? |

Explanation:	There may be problems of low output signal levels if the expected signal frequency is close to the component maximum frequency limit.

| Identifier | 7B |
| --- | --- |
| Title | location of the protection device |
| Clue type | component-only |
| Item | load |
| Clue Text | Are the protection devices missing? |

Explanation:	When a signal or input is distributed to circuits, the first component attached to the input line shall be a protective device in order to protect the input against a short circuit on the component.

| Identifier | 116 |
| --- | --- |
| Title | correct signal attachment on drawing |
| Clue type | component-only |
| Item | signal+connector+label+offpage |
| Clue Text | Is each circuit using the right signal on its inputs and delivering the right signal at the correct output? |

Explanation:	-	As a system is usually documented on several drawings, flags are used to identify corresponding signal lines on the various drawing.

-	When using a register to latch the data of a bus (for instance), the input <i> of the latch shall deliver on the corresponding output a signal with the same number <i>.

-	Sometimes, labels seem to be at the correct place on the drawing, but within computer, labels may be mis-attached.

-	In case of hierarchical design, the flags of a level shall be coherent with the flags of the upper and lower level

| Identifier | 117A |
| --- | --- |
| Title | one path for separate power returns |
| Clue type | component-only |
| Item | earth+ground+chassis |
| Clue Text | Are independent power supplies sharing a current return path or chassis return? |

Explanation:	The return lines for digital and low-level analogue signals shall be kept separate. Spikes due to digital circuits will not then propagate to analogue circuits through a common power line. High inrush currents due to power switching will not affect the voltage stability of other sensitive circuits.

| Identifier | 117B |
| --- | --- |
| Title | one earth path for separate power source |
| Clue type | component-only |
| Item | earth+chassis |
| Clue Text | Are separate power source lines mixed? |

Explanation:	Independent power supplies should not share a common line/conductor to supply their loads.
The return lines for digital and low-level analogue signals shall be kept separate. Spikes due to digital circuits will not then propagate to analogue circuits through a common power line. High inrush currents due to power switching will not affect the voltage stability of other sensitive circuits. 
Usually, CAD provides the designer with a “star” component. It allows several separate signals with different labels to be created from a single signal. It shall be used to distribute a power source with a star net.

| Identifier | 117D |
| --- | --- |
| Title | one path for separate signal returns |
| Clue type | component-only |
| Item | earth+ground+chassis |
| Clue Text | Is a single earth return long or high resistance? |

Explanation:	Independent signal return paths should be kept separate, to avoid cross talk until they reach a common low resistance earth point. Cross talk can in some cases lead to feedback oscillations.

| Identifier | 119A |
| --- | --- |
| Title | monitoring circuit independent of monitored function |
| Clue type | component-only |
| Item | sensor+psens+fsens+tsens+lsens |
| Clue Text | Does the monitoring circuit depend upon the function it monitors? |

Explanation:	A failure of the monitored circuit shall not prevent the monitoring circuit from operating correctly: for instance through a common power source if the monitored function is shorted. The monitoring circuit should not depend on the monitored function for its proper functioning. On the other hand, means shall be provided to detect a failure of the monitoring circuit.

| Identifier | 121 |
| --- | --- |
| Title | ICs needing external component |
| Clue type | component-only |
| Item | IC+gate+and+or+nand+nor+not+buffer+clock+JK+RS+flipflop+ register+driver+counter |
| Clue Text | For ICs needing an external component, has the appropriate component been implemented? |

Explanation:	Some TTL outputs are “open -collector”: A pull-up resistor shall be added between the output (transistor collector) and + VCC. Some TTL outputs are also sometimes “open emitter”: a pull-down resistor then shall be added. The value of the resistor depends of the load to drive.
Tristate outputs also require a pull-up or pull-down resistor. A transistor base-driven by an open collector transistor should also have a pull-up/down resistor to assure proper switching. If an open collector TTL output drives a CMOS input, a pull-up resistor shall be added.

| Identifier | 122 |
| --- | --- |
| Title | noise compatible with device noise immunity |
| Clue type | component-only |
| Item | IC |
| Clue Text | Is the signal noise acceptable for the circuit dealing with this signal? |

Explanation:	When a signal contains extraneous noise, its upper and lower bounds may exceed specified limits, making the signal unacceptable.
To increase the noise immunity of a receiver, Schmitt trigger gates may be used on inputs. On the same principle, a hysteresis cycle may be implemented in comparators. In any case, a noise margin should always be maintained.

| Identifier | 123 |
| --- | --- |
| Title | internally unsuppressed pins |
| Clue type | component-only |
| Item | IC |
| Clue Text | For ICs with internally unsuppressed pins, has a suppression device been implemented? |

Explanation:	Some integrated circuits do not have internal protection diodes fitted to the inputs and outputs. These integrated circuits are vulnerable to transient voltages which may exceed maximum permitted limits causing latch-up or even permanent damages.
External protection devices (diodes, resistors in series, ...) shall be attached to pins with such “dangerous” signals.

| Identifier | 124 |
| --- | --- |
| Title | load characteristic after use |
| Clue type | path-component component-only |
| Item | EED+squib+explosive+detonator |
| Clue Text | Are the after-use EED characteristics compatible with the circuit implementation? |

Explanation:	After firing, squib components may be open- or short-circuited. Depending of this state, the circuit implementation will not be the same, nor the protection devices (current limiter). A short circuit in a squib shall not cause damage other circuit components. Capacitor voltage discharge sources are not as likely to burn a squib “open” as is a battery or generator supply.

| Identifier | 125A |
| --- | --- |
| Title | static energy protection |
| Clue type | component-only |
| Item | EED+squib+detonator |
| Clue Text | Is a static energy protection for electroexplosive device implemented? |

Explanation:	Without static energy protection, the electroexplosive device may ignite or detonate without being commanded. For instance, a pull-down resistor to ground of 100kW may be used.

| Identifier | 125B |
| --- | --- |
| Title | disconnection of a static energy protect |
| Clue type | component-only |
| Item | EED+squib+detonato |
| Clue Text | Can a static energy protection for electroexplosive device be disconnected? |

Explanation:	Without static energy protection, the electroexplosive device may ignite or detonate without being commanded. Unintended disconnection or switching out the protection (for test purposes) may introduce a hazard.

| Identifier | 126 |
| --- | --- |
| Title | “no fire” current protection |
| Clue type | component-only |
| Item | EED+squib+detonator+exbolt |
| Clue Text | Is a “no fire” current protection for the device implemented? |

Explanation:	Depending on the duty, EED’s may require: 
- an out-of-line firing chain 
- a separate mechanical arming device 
- diverse redundant switches in the firing chain 
- sterilisation or guaranteed cut-out circuits.

| Identifier | 127 |
| --- | --- |
| Title | unexpected activation |
| Clue type | component-only |
| Item | rectifier+thyristor+thyratron+triac+GTO |
| Clue Text | Can a rectifier change state without being commanded? |

Explanation:	Some rectifiers need a command to turn-on (thyristor, triac) and to turn-off (GTO). Turn-on occurs in the presence of a short-duration, low-valued gate current. Such low-valued current shall not be generated by a spike. However, they can also activate without being commanded, if for instance they see a significant voltage transition (dV/dt too high). Such conditions may be obtained during testing, for instance.

| Identifier | 129 |
| --- | --- |
| Title | drawing consistency with part list |
| Clue type | component-only |
| Item | part |
| Clue Text | Does the parts list correspond to the components used in the circuit? |

Explanation:	As CAD tools are usually generating automatically the parts list, each component of the circuit shall be listed in the parts list. The acronym associated to each part shall also correspond to the expected symbol.

| Identifier | 130 |
| --- | --- |
| Title | component direction |
| Clue type | component-only |
| Item | transformer+diode+transistor+IC |
| Clue Text | Is the component implemented with the correct polarisation on the drawing? |

Explanation:	Some components are not polarised: resistor, inductor, not polarized capacitor (they are bidirectional). On the other hand, some components can only be used in one way: polarized capacitors, diode, transistor, integrated circuit, transformer (with primary and secondary windings,...).

| Identifier | 132 |
| --- | --- |
| Title | current limiter shared by multiple loads |
| Clue type | component-only |
| Item | limiter+regulator+csens+breaker+fuse |
| Clue Text | Is the current limiter sensitive to each load? |

Explanation:	Whatever the considered load, a short circuit shall trigger the current limiting circuit. No load pattern should exist which can mask a short circuit.

| Identifier | 133 |
| --- | --- |
| Title | operating range of a limiting device |
| Clue type | component-only |
| Item | fuse+breaker+Zener+regulator |
| Clue Text | Is the operating range of a limiting device correctly adjusted? |

Explanation:	Spurious activation of a limiting device shall be avoided: during power switching of an equipment, the spike of inrush current shall not trigger the current limiter otherwise the equipment shall never be turned on! On the other hand, any permanent short circuit shall be detected.

| Identifier | 136A |
| --- | --- |
| Title | response time of a monitoring circuit |
| Clue type | component-only |
| Item | sensor+threshold |
| Clue Text | Is the response time of the monitoring circuit acceptable for the system? |

Explanation:	If a capacitor voltage is used (directly or not) as a status information by a monitoring circuit, the capacity may induce a delay. 
If a monitoring circuit checks the status of a relay  just after the relay has changed of state, erroneous information may be transmitted if the relay contacts suffer “contact bounce”after switching. A delay for the status acquisition is necessary to monitor a stable status.

| Identifier | 136B |
| --- | --- |
| Title | start up latency |
| Clue type | component-only |
| Item | sensor+threshold |
| Clue Text | Is the start up latency of the sensor acceptable for the system? |

Explanation:	Many sensors for physical variables, particularly concentration and temperature, have a latency time before they register correctly.

| Identifier | 138 |
| --- | --- |
| Title | transient current path through switches |
| Clue type | path-component component-only |
| Item | transistor+rectifier+breaker+relay+switch+contact+button+ interlock+plug |
| Clue Text | During change of state of switches, can transient current paths exist? |

Explanation:	As switches have not the same speed (when switching):

-	a switch can still be in the same position whereas another has already changed of state.

-	a switch can be “Open” during changing of state, whereas another has contacts touching.

Make-before-break switches create a transient current path between contacts during switching. If a short circuit occurs through contacts during a transitory current path, it may weld the contacts and maintain the short circuit. If there is a switch in an upper branch and in a lower branch, a short circuit may appear if both switches let the current flow at the same time (due to the switching reaction delay).

| Identifier | 138A |
| --- | --- |
| Title | transient current path through switches |
| Clue type | path-component component-only |
| Item | transistor+rectifier+breaker+relay+switch+contact+button+interlock+plug |
| Clue Text | During change of state of switches, can transient current paths exist? |

Explanation:	As switches have not the same speed (when switching):

-	a switch can still be in the same position whereas another has already changed of state.

-	a switch can be “Open” during changing of state, whereas another has contacts touching.

Make-before-break switches create a transient current path between contacts during switching. 
If a short circuit occurs through contacts during a transient current path, it may weld the contacts and maintain the short circuit. 
If there is a switch in an upper branch and in a lower branch, a short circuit may appear if both switches let the current flow at the same time (due to the switching reaction delay).

| Identifier | 139 |
| --- | --- |
| Title | transient current gap while switching |
| Clue type | path-component component-only |
| Item | transistor+rectifier+breaker+relay+switch+contact+button+ interlock+plug |
| Clue Text | During change of state of switches, can transient current gap exist? |

Explanation:	While the pole of the relay moving piece is in transit between two contacts, both contacts are momentarily floating. Then the load may not be supplied during a brief moment, which may not be acceptable, or may create the possibility for another current path to occur.

| Identifier | 140 |
| --- | --- |
| Title | (un)expected switch/command activation |
| Clue type | path-component component-only |
| Item | transistor+rectifier+breaker+relay+switch+contact+button+ interlock+plug |
| Clue Text | Can a switch or its command be activated when not desired or inhibited when expected? |

Explanation:	During power transition, the transient current which flows (for instance to charge capacitors) can briefly activate a switch command. If a part of a circuit is powered while another part is not, it can inhibit a current path or create new ones. 
Unexpected sequences (testing, check-out, contingency,..) can lead to an unscheduled switch combination. Conduction may be required for one load, but may be unwanted for other loads.

| Identifier | 140A |
| --- | --- |
| Title | (un)expected switch/command activation |
| Clue type | path-component component-only |
| Item | transistor+rectifier+breaker+relay+switch+contact+button+ interlock+plug |
| Clue Text | Can a switch or its command be activated when not desired or inhibited when expected? |

Explanation:	During power transition, the transient current which flows (for instance to charge capacitors) can briefly activate a switch command. If a part of a circuit is powered while another part is not, it can inhibit a current path or create new ones.
Unexpected sequences (testing, check-out, contingency,..) can lead to an unscheduled switch combination. Conduction may be required for one load, but may be unwanted for other loads.

| Identifier | 140AA |
| --- | --- |
| Title | (un)expected switch/command activation |
| Clue type | path-component component-only |
| Item | transistor+rectifier+breaker+relay+switch+contact+button+ interlock+plug |
| Clue Text | Can a switch or its command be activated when not desired or inhibited when expected? |

Explanation:	During power transition, the transient current which flows (for instance to charge capacitors) can briefly activate a switch command. If a part of a circuit is powered while another part is not, it can inhibit a current path or create new ones.
Unexpected sequences (testing, check-out, contingency,..) can lead to an unscheduled switch combination. Conduction may be required for one load, but may be unwanted for other loads.

| Identifier | 140AAA |
| --- | --- |
| Title | (un)expected switch/command activation |
| Clue type | path-component component-only |
| Item | and+nor |
| Clue Text | Can a switch or its command be activated when not desired or inhibited when expected? |

Explanation:	During power transition, the transient current which flows (for instance to charge capacitors) can briefly activate a switch command. If a part of a circuit is powered while another part is not, it can inhibit a current path or create new ones.
Unexpected sequences (testing, check-out, contingency,..) can lead to an unscheduled switch combination. Conduction may be required for one load, but may be unwanted for other loads.

| Identifier | 141A |
| --- | --- |
| Title | power transient |
| Clue type | path-component component-only |
| Item | gate+AND+mux |
| Clue Text | Can a transient pick-up or drop-out voltage unintentionally energize/de-energize a load or a switching item? |

Explanation:	During a power transition a capacitor behaves as a low impedance: a capacitor implemented as a static protection (against direct current) may allow an unexpected current flow through a load in dynamic (transient) conditions.
Voltage differences within a circuit often occur:

-	during power on/off, some parts of the circuit may have reached their operational voltage while others are still changing

-	during power on/off, within a circuit supplied with separated voltages, one voltage may have reached its operational value while others are still changing

-	separated parts of a circuit can be independently supplied, one being powered while the others are not.

Such voltage differences, normal or unexpected, can create sneak current paths through diodes, transistor base-collector junction, circuit interfaces where the voltage difference occurs, rectifier due to a high dV/dT, transformer, internal protections (diodes) or connected pins of integrated circuits.

| Identifier | 142 |
| --- | --- |
| Title | disconnection of electrical interlocks |
| Clue type | path-component component-only |
| Item | interlock+connector+switch+contact |
| Clue Text | Can the disconnection arising from electrical interlocks create sneak current paths, depending of the disconnection order? |

Explanation:	If simultaneous disconnections are scheduled, one disconnection may occur before others creating, for a brief moment, unexpected current paths: for instance, suppression of a ground return path.

| Identifier | 143 |
| --- | --- |
| Title | connection of testing circuits |
| Clue type | path-component component-only |
| Item | testpoint+connector |
| Clue Text | Can (external) testing circuits energize parts other than those intended to be tested? |

Explanation:	Usually, protective devices are installed on lines carrying test signals: e.g. diodes connected to VCC either externally or internally to integrated circuits. If an erroneous voltage is applied to the test signal input, a sneak current path through the test interface and the protective devices may energize unexpected functions.

| Identifier | 146A |
| --- | --- |
| Title | loss of ground |
| Clue type | path-component component-only |
| Item | ground+earth+chassis |
| Clue Text | Does loss of ground allow feedback into the path? |

Explanation:	If the ground is used to determine the potential of a branch, the loss of a ground contact (after switching or disconnection for instance) will leave the junction point floating, allowing an unexpected current to flow through the other branches. It may also mis-bias some circuits and destroy them.

| Identifier | 146B |
| --- | --- |
| Title | loss of ground |
| Clue type | path-component component-only |
| Item | node |
| Clue Text | Does loss of ground allow feedback into the path? |

Explanation:	If the ground is used to determine the potential of a branch, the loss of a ground contract (after switching or disconnection for instance) will leave the junction point floating, allowing an unexpected current to flow through the other branches. It may also mis-bias some circuits and destroy them.

| Identifier | 147 |
| --- | --- |
| Title | loss of power source |
| Clue type | component-only path-component |
| Item | transistor+OpAmp+diode+Zener+capacitor |
| Clue Text | Does loss of one power source mis-bias the circuit? |

Explanation:	Losing a power source (unexpected switching or disconnection) may create an intermediate bias voltage, allowing an unwanted current path in the other branches or component malfunction.

| Identifier | 148 |
| --- | --- |
| Title | high value resistor |
| Clue type | component-only |
| Item | resistor |
| Clue Text | Is a high-value resistor compatible with a low parasitic capacitance? |

Explanation:	Parasitic capacitor is generated by a circuit input and the board wiring which, when combined with a high value resistor can create an unexpected time constant.

| Identifier | 151 |
| --- | --- |
| Title | turn-on delay of a diode |
| Clue type | component-only |
| Item | diode |
| Clue Text | Is the turn-on delay acceptable for the diode? |

Explanation:	Sometimes, a current versus time curve gives information on a necessary turn-on delay for the diode.

| Identifier | 153 |
| --- | --- |
| Title | reference potential for chassis |
| Clue type | component-only |
| Item | chassis |
| Clue Text | Is the chassis reference tied to the adequate reference potential? |

Explanation:	If the chassis ground is not tied to the equipment ground, some components using the chassis as a heat sink and current return path may not be operational. The metallic case of transistors is sometimes used for a current path as it is tied to the transistor collector.

| Identifier | 153A |
| --- | --- |
| Title | reference potential for power supply |
| Clue type | component-only |
| Item | battery+power |
| Clue Text | Is the power supply reference tied to the adequate ground? |

Explanation:	If the power return is not tied to the equipment ground, some components using the chassis as a heat sink and current return path may not be operational. The metallic case of transistors is sometimes used for a current path as it is tied to the transistor collector.

| Identifier | 155 |
| --- | --- |
| Title | reverse current in a relay coil |
| Clue type | component-only |
| Item | relay |
| Clue Text | Has a protection against reverse current been implemented in series with a relay coil command? |

Explanation:	A relay coil may receive several commands (through a wired “OR”). To avoid unexpected reverse current from one command to another, a diode is generally added in series with the relay coil on the current path of the command signal.

| Identifier | 156 |
| --- | --- |
| Title | bias/polarization signal |
| Clue type | component-only |
| Item | transistor+Zener+MOS+FET+OpAmp+analogue |
| Clue Text | Is a bias/polarization signal correct? |

Explanation:	Part of the current used to fix a bias/polarization signal may be derived by a component using this reference signal (transistor, OpAmp, Zener, diode). The bias/polarization current shall be high enough to provide a stable reference with the correct voltage. A capacitor may also be added for decoupling. 
Ensure that a (scheduled) disconnection or switching will not modify the correct functioning of the circuit.

| Identifier | 157 |
| --- | --- |
| Title | reaction delay of one-shot items |
| Clue type | component-only |
| Item | EED |
| Clue Text | Has delay time of one-shot items or electroexplosive devices been considered during design? |

Explanation:	The following topics should be checked: 
- sufficient pulse duration 
- explosion duration

| Identifier | 159 |
| --- | --- |
| Title | protective and polarization device |
| Clue type | component-only |
| Item | buffer+driver+connector+plug+malecon+femalecon |
| Clue Text | Have interfaces with the outside world a protective and polarization device? |

Explanation:	To prevent component damage or erroneous information, unterminated inputs at interface may require a polarization resistor (pull-up or pull-down).
Resistor in series with the inputs/outputs shall protect the components from static discharge. Protection diodes may also be added on unsuppressed inputs/outputs. In case an erroneous signal is applied to the circuit through a test interface, a protection shall prevent the circuit from being damaged.

| Identifier | 161 |
| --- | --- |
| Title | splitting and recombining of a digital signals |
| Clue type | path-component component-only |
| Item | digital+connector+interlock |
| Clue Text | Does a digital signal sharing a common source and load split and later recombine? |

Explanation:	Glitches often occur as a result of the separating and later recombining of digital signals having a common source and load.
Glitches may results in false logic causing sneak-timing. Glitches are usually detected by drawing a timing diagram.

| Identifier | 162 |
| --- | --- |
| Title | switch function for target |
| Clue type | path-component component-only |
| Item | switch+breaker+fuse+button+relay+contact+path |
| Clue Text | Is the switch function directly related to the target operation? |

Explanation:	The switch may have other functions not related to the target or may be intended for some other purpose entirely. The other function can be activated/deactivated at a time when the target is required/not required.

| Identifier | 164 |
| --- | --- |
| Title | correct polarization with coupling capacitor |
| Clue type | component-only |
| Item | capacitor |
| Clue Text | Are circuits using a signal coming through a joining (coupling) capacitor correctly polarized? |

Explanation:	Sometimes, coupling capacitors are used between circuits to transmit an a.c. signal without any direct current component. In such cases, the direct current component shall be restored by connecting the secondary side of the capacitor to ground via a suitable resistor.

| Identifier | 7C |
| --- | --- |
| Title | location of the protection device |
| Clue type | component-only |
| Item | load |
| Clue Text | Is the circuit located upstream of protection device(s) on a power bus? |

Explanation:	When power is distributed to circuits, the first item on the power path shall be a protective device in order to protect the power bus against a short circuit in the sensor.

| Identifier | 9 |
| --- | --- |
| Title | resynchronization of a sequential logic |
| Clue type | component-only |
| Item | IC+and+or+nand+nor+not+flipflop+JK+RS |
| Clue Text | Is a sequential digital signal safely resynchronized? |

Explanation:	When a digital signal passes through many logic gates, a delay occurs. Within systems/circuits using a common clock, sequential digital signals shall be resynchronized as soon as the propagation time uncertainty may lead to sneak timing problems. The resynchronisation signal may be missing in some modes.

| Identifier | 9A |
| --- | --- |
| Title | resynchronization of sequential logic with long logic train |
| Clue type | component-only |
| Item | IC+and+or+nand+nor+not+flipflop+JK+RS |
| Clue Text | Is a sequential digital signal safely resynchronized for a long logic train? |

Explanation:	When a digital signal passes through many logic gates, a delay occurs. Within systems/circuits using a common clock, sequential digital signals shall be resynchronized as soon as the propagation time uncertainty may lead to sneak timing problems. This is especially true if the logic train is very long.

| Identifier | 169 |
| --- | --- |
| Title | explicit wording of a label |
| Clue type | component-only |
| Item | label+lamp+indicator |
| Clue Text | Is each label wording explicit? |

Explanation:	Instead of writing for example “function 2” for a label, it is preferable to identify the function precisely: “door opening”. This is important for indicators or command

| Identifier | 171 |
| --- | --- |
| Title | indicator correct function label |
| Clue type | path-component component-only |
| Item | lamp+indicator+switch+button |
| Clue Text | Does the label of the indicator reflect the function of the TARGET? |

Explanation:	If the label does not properly reflect the function, the TARGET may be activated or deactivated at the wrong time.

| Identifier | 172 |
| --- | --- |
| Title | indicator for a unique function |
| Clue type | path-component component-only |
| Item | lamp+indicator+switch+button |
| Clue Text | Does the label of the indicator reflect other functions than the one(s) intended? |

Explanation:	If the label reflects other functions, some equipment may be activated or deactivated at the wrong time.

- (normative)
Clues for software in ADA language

| Identifier | 1001 |
| --- | --- |
| Title | memory mapped register |
| Clue type | path-component component-only |
| Item | outreg |
| Clue Text | Is the register memory mapped? If so, can it be addressed in error as a result of array Identifier errors? |

Explanation:	Memory addressing errors can cause false output, and require data flow tracing to potential erroneous array addressing locations and pointer assignments.

| Identifier | 1002 |
| --- | --- |
| Title | address code control |
| Clue type | path-component component-only |
| Item | outreg |
| Clue Text | Are the codes addressing the register modifiable by several parts of the software? |

Explanation:	If the address codes are present in other places or can be created by modification, the register values could be modified erroneously.

| Identifier | 1003 |
| --- | --- |
| Title | register number modification |
| Clue type | path-component component-only |
| Item | outreg |
| Clue Text | Are any register numbers treated as parameters, or used as updatable table entries? |

Explanation:	If so, it may be possible to modify the number so that the register is addressed wrongly, or is not addressed when it should be.

| Identifier | 1004 |
| --- | --- |
| Title | synthesis of control codes |
| Clue type | path-component component-only |
| Item | outreg |
| Clue Text | Are the operation codes for the register fixed codes, or are they constructed from constants by bit manipulation? |

Explanation:	If they are constructed, they may be incorrect, resulting in the incorrect output signal.

| Identifier | 1005A |
| --- | --- |
| Title | register sequence dependency |
| Clue type | path-component component-only |
| Item | outreg |
| Clue Text | Are there constraints on the register operation sequence? |

Explanation:	It is often the case that separate outputs to the same device shall be in the correct sequence in order to establish the correct sequence of system states. If this is not done it may lead to stop of operations, or to incorrect system operation.

| Identifier | 1006 |
| --- | --- |
| Title | memory mapped register |
| Clue type | path-component component-only |
| Item | inreg |
| Clue Text | Is the register memory mapped? If so, is it possible to read from the wrong register due to an addressing error? |

Explanation:	If so a spurious input value will be obtained.

| Identifier | 1007 |
| --- | --- |
| Title | register address modification |
| Clue type | path-component component-only |
| Item | inreg |
| Clue Text | Are the register addresses hard coded, or are they stored in a table? Is the table updated? |

Explanation:	If so, the wrong register may be addressed.

| Identifier | 1008 |
| --- | --- |
| Title | register timing |
| Clue type | path-component component-only |
| Item | inreg |
| Clue Text | Is it possible to read register values while they are partially updated? |

Explanation:	In some cases, registers are updated gradually, for example one bit at a time. If this occurs, then it is important to synchronise read operations so that they occur when an input cycle has been completed.

| Identifier | 1009 |
| --- | --- |
| Title | register use coordination |
| Clue type | path-component component-only |
| Item | inreg |
| Clue Text | Are there two programs which read the same register values? |

Explanation:	If so, are the read operations coordinated, or can the programs disagree about the register value due to differences in the read time?

| Identifier | 1010 |
| --- | --- |
| Title | fixed data read cycle |
| Clue type | path-component component-only |
| Item | inreg |
| Clue Text | For data acquisition, is the cycle of data reading fixed? Is the delay time between read operations fixed? |

Explanation:	If not, then data series will not be made according to a fixed time base and noise will be introduced into data. Interrupts and variable length loops can cause variations in read cycle.

| Identifier | 1011 |
| --- | --- |
| Title | data read frequency |
| Clue type | path-component component-only |
| Item | inreg |
| Clue Text | Is the delay time between read operations always short compared with the natural time constants for the system being monitored? |

Explanation:	As a rule of thumb the delay should be less than one tenth of the shortest significant time constant in the controlled system.

| Identifier | 1012 |
| --- | --- |
| Title | input register read timing |
| Clue type | path-component component-only |
| Item | inreg |
| Clue Text | Can exceptions, overflows or special handling upset the timing or sequence of input? |

Explanation:	If there are unusual operations, there may be excessive time delays in treating time critical input, or in providing safety critical outputs.

| Identifier | 1013 |
| --- | --- |
| Title | register overflow |
| Clue type | path-component component-only |
| Item | inreg |
| Clue Text | Can the input register overflow? |

Explanation:	Register overflow will cause erroneous input data values.

| Identifier | 1015 |
| --- | --- |
| Title | function side effects |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Can expression evaluation order cause variations in function side effects? |

Explanation:	Is the order of evaluation of parts of the expression fixed? If not, does the sequence of evaluation of terms in the expression affect the result? This can occur if there are functions in the expression which have side effects.

| Identifier | 1015A |
| --- | --- |
| Title | exception side effects |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Can exceptions cause variations in function side effects? |

Explanation:	Is the order of evaluation of parts of the expression fixed? If not, does the sequence of evaluation of terms in the expression affect the result? This can occur if there evaluation can result in exceptions are functions in the expression.

| Identifier | 1016 |
| --- | --- |
| Title | type coercion |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Does expression evaluation or assignment involve type coercion? |

Explanation:	If so, is there any loss of precision or potential for overflow?

| Identifier | 1017 |
| --- | --- |
| Title | overflow |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Can evaluation of the expression cause arithmetic overflow or underflow? |

Explanation:	Overflow or underflow may cause errors in output, spikes or steps in control signals, and delays in output or loss of processing.

| Identifier | 1018 |
| --- | --- |
| Title | division by zero |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Is there divide operation in the statement? Can division by zero occur? |

Explanation:	Division by zero will cause an exception, which could lead to delay, loss of processing, or erroneous results.

| Identifier | 1019 |
| --- | --- |
| Title | string character 0 |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | For strings, is it possible to address before the first string character? |

Explanation:	Some string handling systems allow free indexing to access particular characters in a string. A typical error is to access the zeroth character. Generally such an error will cause an exception, but in some cases it may corrupt program or data.

| Identifier | 1019A |
| --- | --- |
| Title | string length |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | For strings, is it possible to address beyond the last string character? |

Explanation:	Some string handling systems allow free indexing to access particular characters in a string. A typical error is to access the zeroth character. Generally such an error will cause an exception, but in some cases it may corrupt program or data.

| Identifier | 1020 |
| --- | --- |
| Title | string termination |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | For strings, is there a proper string termination? |

Explanation:	Strings should preferably have a termination character, or a string length counter, which prevents access beyond the end of the string. Otherwise the program may access invalid characters, or corrupt data or program.

| Identifier | 1021 |
| --- | --- |
| Title | array limits |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | For arrays, is it possible to address beyond the array limit? |

Explanation:	Addressing beyond array limits will retrieve erroneous data or corrupt data or programs. Some compilers prevent addressing outside the array limits, in which case an exception will result

| Identifier | 1022 |
| --- | --- |
| Title | array zero element |
| Clue type | path-component component-only |
| Item | For arrays, is the zero the element addressed without being defined? |
| Clue Text | assign+box |

Explanation:	The addressing convention for arrays shall be well defined. Addressing a non-existent zeroth element in an array is a typical error, especially if programmers work in a multi-language environment.

| Identifier | 1023 |
| --- | --- |
| Title | multidimensional arrays |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | For multi dimensional arrays, is the array Identifier sequence clear? |

Explanation:	A common error is to confuse the order of array indexes.

| Identifier | 1025 |
| --- | --- |
| Title | aliasing |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Are any data elements aliassed or overloaded? |

Explanation:	If so, the mapping between data types may be in error, with shifts of byte boundaries or value representation.

| Identifier | 1026 |
| --- | --- |
| Title | pointer data access |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Are any data elements aliassed, or addressed by assembler code or by pointers? |

Explanation:	If so, and all possible bit patterns are allowed some patterns may not be relevant for both aliases?

| Identifier | 1027 |
| --- | --- |
| Title | initialization of values |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Are data values entering into the expression initialized along every flow path leading to the statement? |

Explanation:	Uninitialized data may contribute to an erroneous calculation. The problem may be missed in a test environment where data has a fixed default value, and then show the error during actual operation.

| Identifier | 1028 |
| --- | --- |
| Title | numeric constants |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Are numeric constants formatted properly, avoiding commas, and with the correct number of zeros? |

Explanation:	Numeric constants can be given different internal values, depending on format, even for nominally identical numbers.

| Identifier | 1029 |
| --- | --- |
| Title | decimal point placement |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Are commas and decimal points confused? |

Explanation:	This can be a problem especially if there is an interchange between English speaking and European programming environments.

| Identifier | 1030 |
| --- | --- |
| Title | mismatch in parameter list |
| Clue type | path-component component-only |
| Item | assign+box+call |
| Clue Text | Does the call parameter list match the function definition list? In number, types, and number of array indices? |

Explanation:	Mismatch between the call and declaration of a procedure, in number and type of parameters, is a typical programming error. It is especially prevalent when procedures shall be extended.

| Identifier | 1031 |
| --- | --- |
| Title | overloading |
| Clue type | path-component component-only |
| Item | assign+box+call |
| Clue Text | Is a function name or operator symbol overloaded? Can confusion of type then occur? |

Explanation:	Overloading (multiple typing) of procedures, operators, etc can systematically lead to error, if the code is not modified accurately to reflect the change in operand or parameter types.

| Identifier | 1032 |
| --- | --- |
| Title | assignment to constants |
| Clue type | path-component component-only |
| Item | assign+box+call |
| Clue Text | Can new values be assigned to constants when they are used in the parameter list? |

Explanation:	When constants are used in a parameter list for a procedure or function call, it may be possible to assign a new value to the “constant”. This can cause an error which is very difficult to locate, with erroneous calculation in later unrelated functions.

| Identifier | 1033 |
| --- | --- |
| Title | recursive call |
| Clue type | path-component component-only |
| Item | assign+box+call |
| Clue Text | If a function in the statement is recursive (or indirectly recursive), is data space allocated dynamically? |

Explanation:	Recursive functions may accidentally reuse a fixed data area. Runaway or excessive recursion may also lead to running out of stack or heap space.

| Identifier | 1034 |
| --- | --- |
| Title | stack overflow |
| Clue type | path-component component-only |
| Item | assign+box+call |
| Clue Text | Is there a possibility for stack overflow on assignment or call? |

Explanation:	If so memory exceptions may be raised or misaddressing may occur.

| Identifier | 1034A |
| --- | --- |
| Title | heap space exhaustion |
| Clue type | path-component component-only |
| Item | assign+box+call |
| Clue Text | Is there a possibility for exhaustion of heap space when memory is allocated dynamically? |

Explanation:	If so, a memory allocation exception will occur.

| Identifier | 1035 |
| --- | --- |
| Title | freeing memory |
| Clue type | path-component component-only |
| Item | assign+box+call |
| Clue Text | Is all dynamically allocated memory also freed? |

Explanation:	Memory which is not freed may be needed by other procedures. This can arise because:

-	the free instruction is omitted;

-	the free instruction can be bypassed exception exits from the    procedure.

| Identifier | 1036 |
| --- | --- |
| Title | endless recursion |
| Clue type | path-component component-only |
| Item | assign+box+call |
| Clue Text | For the recursive function, is the termination condition guaranteed? |

Explanation:	The number of recursion levels shall be predictable.

| Identifier | 1037 |
| --- | --- |
| Title | case coverage in nested IF |
| Clue type | path-component component-only |
| Item | if+diamond+case |
| Clue Text | For a nested If or case statement, is the list of conditions exhaustive? |

Explanation:	For safety-related programs, the list of possible cases should be made explicit, rather than relying on a final “other” or “else” clause. Then any non-allowed condition can lead to error handling.

| Identifier | 1038 |
| --- | --- |
| Title | test sequence and priority |
| Clue type | path-component component-only |
| Item | if+diamond+case |
| Clue Text | For nested IF statements, is the sequence or priority of tests correct or consistent? |

Explanation:	The actual sequence of condition nesting may be irrelevant, provided that the logic is completely correct. However, lack of clarity and consistency often lead to error, for example neglecting some special case.

| Identifier | 1039 |
| --- | --- |
| Title | function calls in IF test |
| Clue type | path-component component-only |
| Item | assign+box+call |
| Clue Text | Are there repeated function calls within the IF condition? Does the function allow side effects? |

Explanation:	Function calls with side effects within conditional statements are almost guaranteed to give erroneous values for side effect variables at some stage of processing.

| Identifier | 1040 |
| --- | --- |
| Title | floating point equality |
| Clue type | path-component component-only |
| Item | if+diamond+case |
| Clue Text | Are there tests for equality between floating point numbers? If so, how is equality guaranteed? |

Explanation:	Two real numbers will in general never be equal unless values are assigned identically.

| Identifier | 1040A |
| --- | --- |
| Title | zero representation |
| Clue type | path-component component-only |
| Item | if+diamond+case |
| Clue Text | Are both positive and negative zero values possible? |

Explanation:	On some computers, there is a possibility for a sign difference, even when the numbers represent zero. This can make equality comparison incorrect.

| Identifier | 1041 |
| --- | --- |
| Title | TRUE and FALSE representation |
| Clue type | path-component component-only |
| Item | if+diamond+case+assign+box |
| Clue Text | Is the coding for TRUE and FALSE values consistent, and used consistently? |

Explanation:	Bit coding for representation TRUE and FALSE varies from system to system, and programmers often make errors when transferring between systems.

| Identifier | 1043 |
| --- | --- |
| Title | loop termination |
| Clue type | path-component component-only |
| Item | while+for+loop |
| Clue Text | Is loop termination guaranteed? |

Explanation:	An endless loop can cause a stop of all processing on a processor, or, if time slicing is used, it can cause a single task to fail.

| Identifier | 1043A |
| --- | --- |
| Title | loop termination |
| Clue type | path-component component-only |
| Item | while+for+loop |
| Clue Text | Does the loop deal with precisely all elements of an array? For loops that deal with arrays, does the loop Identifier track the array precisely, and deal with all loop elements, including the zero’th and the last? |

Explanation:	Take special care on loops which relate the i’th and the (i+1)’th array elements, or similar offset access to arrays

| Identifier | 1045 |
| --- | --- |
| Title | variable loop Identifier limit |
| Clue type | path-component component-only |
| Item | while+for+loop |
| Clue Text | For the FOR loop Identifier, is the Identifier limit value fixed, or can it vary, and if so, can the variation result in a loop Identifier error? |

Explanation:	Changing the limit value for a for loop can often result in difficult logic, in which it is not clear whether a final limit will be reached. The usual result is an array Identifier overflow error, but in some cases, the program may request all available memory, before an exception occurs.

| Identifier | 1046 |
| --- | --- |
| Title | control transfer into a loop |
| Clue type | path-component component-only |
| Item | while+for+loop |
| Clue Text | Are there labels within the loop, and can control transfer into the loop? |

Explanation:	Transfer of control into a program loop will in virtually all cases result in errors due to lack of loop initialization. Some programs allow for transfer of control out of a loop and then back again, but this is generally bad practice, since some compilers produce undefined results in such cases.

| Identifier | 1047 |
| --- | --- |
| Title | loop termination |
| Clue type | path-component component-only |
| Item | while+for+loop |
| Clue Text | Is the loop Identifier used outside the loop and if so, is its value also undefined outside the loop? |

Explanation:	Some languages and some compilers allow an Identifier in a FOR loop to be incremented, and the resulting value to be available on leaving the loop. This is not always the case though, and for some compilers, the loop Identifier value is undefined when the loop is excited.

| Identifier | 1048 |
| --- | --- |
| Title | loop termination |
| Clue type | path-component component-only |
| Item | while+for+loop |
| Clue Text | Are there exits from inside the loop? If so, are the loop terminations identical or are they separated on purpose (completion, success, failure) with appropriate processing? |

Explanation:	Flags or labelled exit points are two alternative ways of implementing multiple exits from loops.

| Identifier | 1049 |
| --- | --- |
| Title | event processing |
| Clue type | path-component component-only |
| Item | software |
| Clue Text | For event driven programs, does the processing depend on the sequence of events? If so, are all sequences allowed? |

Explanation:	Event driven programs can be difficult to write, because the programmer has one step by step sequence in mind, for example that described in the program user manual. However, alternative sequences will generally occur, and if the program is sequence dependent, the alternative sequences shall be catered for.

| Identifier | 1051 |
| --- | --- |
| Title | shared data processing |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Are shared variables semaphore protected? |

Explanation:	If not, it may be possible for two tasks to attempt to update shared variables at the same time, so corrupting the values.

| Identifier | 1052 |
| --- | --- |
| Title | shared data processing |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Are groups of variables which shall be updated consistently semaphore protected? |

Explanation:	If not, multiple tasks may attempt to update records at the same time, and the relations between variable values may then be inconsistent.

| Identifier | 1053 |
| --- | --- |
| Title | shared table processing |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Can the current task address a table element which has been deleted by another? |

Explanation:	Deletion is a particularly difficult operation in a multitasking environment, particularly if tasks maintain their own Identifier variables into a table or array.

| Identifier | 1054 |
| --- | --- |
| Title | semaphore/lock matching |
| Clue type | path-component component-only |
| Item | assign+semaphore |
| Clue Text | Can a program reserve or lock data and fail to free it? |

Explanation:	Matching of requesting and releasing commands s essential. Release may be missing on some program paths, or if an exception arises in a program.

| Identifier | 1055 |
| --- | --- |
| Title | event processing |
| Clue type | path-component component-only |
| Item | assign+box+semaphore |
| Clue Text | Can the program fail to free reserved data due to exceptions or errors? |

Explanation:	Matching of requesting and releasing commands s essential. Release may be missing on some program paths, or if an exception arises in a program.

| Identifier | 1056 |
| --- | --- |
| Title | deadlock |
| Clue type | path-component component-only |
| Item | semaphore+task |
| Clue Text | Can deadlock arise? |

Explanation:	Deadlock arises when two or more tasks are competing for two or more resources. If task A has resource X and waits for resource Y, and task B has resource Y and waits for X, neither task can proceed. Solutions to the problem are to request resources in groups, or to use a fixed order for resource requests. Typical resources are memory, data areas, data base records, indexes, buffers, and peripheral devices.

| Identifier | 1058 |
| --- | --- |
| Title | shared data access protection |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Are all program data interactions synchronised by semaphores or rendezvous, or are some dependent on timing? |

Explanation:	If dependent on timing, is the timing insensitive to load, processor type, or priority changes.

| Identifier | 1059 |
| --- | --- |
| Title | buffer overflow |
| Clue type | path-component component-only |
| Item | input+output |
| Clue Text | Can the pipe, buffer, or queue be overfilled if input load increases? |

Explanation:	Heavy input, bus or transaction loads can lead to queue or buffer overfilling. This can in turn lead to addressing errors, to loss of input, or to corruption of entries

| Identifier | 1060 |
| --- | --- |
| Title | overload data loss |
| Clue type | path-component component-only |
| Item | input |
| Clue Text | Can processing overload delay response so that short inputs are missed? |

Explanation:	Input signals which only exist for a short time may be lost completely if the computer is overloaded, and does not scan inputs fast enough. Interrupts maybe missed if interrupt processing is slow.

| Identifier | 1061 |
| --- | --- |
| Title | overload delay of response |
| Clue type | path-component component-only |
| Item | input+task |
| Clue Text | Can overload delay response so that response frequency becomes comparable to a natural frequency of the controlled system? |

Explanation:	If the delay grows to approach the time constant for a controlled system, then oscillation can result. With large delays, this can lead to an equivalent problem in transaction processing, where queries are returned faster than the system can process the original request.

| Identifier | 1062 |
| --- | --- |
| Title | overload delay of espouse |
| Clue type | path-component component-only |
| Item | input+task |
| Clue Text | Can overload delay an output so that a critical deadline is missed? |

Explanation:	Critical deadlines are typical of cyclic or one-shot controlled systems, in which there is a time window for correct input or output to the computer.

| Identifier | 1063 |
| --- | --- |
| Title | error handling interruption |
| Clue type | path-component component-only |
| Item | exception+assign |
| Clue Text | Does error handling stop processing? IF so, is the stop hazardous? |

Explanation:	Exception handling cannot always lead to system restart or recovery.

| Identifier | 1064 |
| --- | --- |
| Title | error handling lockout |
| Clue type | path-component component-only |
| Item | exception+assign+box |
| Clue Text | Can reserved data or memory be frozen due to error handling? |

Explanation:	Semaphores which are not released can lead to lock out, as can bus contention, and memory contention lines.

| Identifier | 1065 |
| --- | --- |
| Title | error recovery reinitialization |
| Clue type | path-component component-only |
| Item | exception+assign+box |
| Clue Text | Are all data values reinitialized on error recovery? |

Explanation:	If a restart occurs, reinitialization is necessary.

| Identifier | 1066 |
| --- | --- |
| Title | data consistency on error recovery |
| Clue type | path-component component-only |
| Item | exception+assign+box |
| Clue Text | Does error handling restore data consistency? |

Explanation:	In some cases, stored or shared data will be corrupted. In this case, restart programs should at least restore consistency, so that further software exceptions do not occur, and the data can be accessed reliably.

| Identifier | 1067 |
| --- | --- |
| Title | deep level errors |
| Clue type | path-component component-only |
| Item | procedure+function+call |
| Clue Text | Can the procedure terminate the task? |

Explanation:	A typical example arises in error processing, especially when a low level subroutine terminates on error.

| Identifier | 1068 |
| --- | --- |
| Title | error codes on procedure exit |
| Clue type | path-component component-only |
| Item | procedure+function+call |
| Clue Text | Are there any error conditions and codes on procedure exit comparable to a natural frequency of the controlled system? |

Explanation:	A consistent philosophy for treating exceptions is necessary. This is particularly difficult to achieve if library or generic routines are used.

| Identifier | 1073 |
| --- | --- |
| Title | data corruption |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Can memory be corrupted, so that wrong data values are used? |

Explanation:	Corruption can arise due to addressing errors, data transmission errors, or memory failure. Transient memory failure may not always be detectable, even when redundancy is used.

| Identifier | 1074 |
| --- | --- |
| Title | erroneous control transfer |
| Clue type | path-component component-only |
| Item | goto+return |
| Clue Text | Can random control transfers be made into critical sections of code? |

Explanation:	Go to statements can lead to unexpected transfers of control, particularly if label or procedure arrays (switches) are used. Erroneous control transfers can also arise due to hardware error in calculating jump addresses.

| Identifier | 1075 |
| --- | --- |
| Title | computed address error |
| Clue type | path-component component-only |
| Item | return+goto |
| Clue Text | Is the address for the control transfer computed or taken from an array? If so can the address be corrupted? |

Explanation:	A corrupted address will lead to a random transfer of control.

| Identifier | 1077 |
| --- | --- |
| Title | use of access types and pointers |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Does the assignment make use of access types? |

Explanation:	Access types and pointers are one of the biggest sources of errors in delivered software, and in safety related software should be used only where strictly necessary, e.g. in list processing packages. The dangers with the use of pointers is that simple arithmetic, initialization, and usage errors can lead to corruption of data and programs in an untraceable and seemingly random fashion.

| Identifier | 1078 |
| --- | --- |
| Title | predefined exceptions |
| Clue type | path-component component-only |
| Item | assign+box |
| Clue Text | Is it possible to activate predefined exceptions? |

Explanation:	The processing of predefined exceptions is often wrong for safety related programs, e.g. the reporting and termination of tasks which often takes place is generally wrong. The program needs to be able either to take safety action, or to continue monitoring.

- (informative)
Clues for hydraulic equipment

| Identifier | 2000 |
| --- | --- |
| Title | evaporation of a liquid |
| Clue type | component-only+path-component |
| Item | tank+tankbladder+tankpres+vessel |
| Clue Text | Is there any device which protects against the decrease of pressure? |

Explanation:	If the pressure decreases below the liquid steam pressure, it would produce a change of phase which could have catastrophic consequences if the necessary measures are not taken into account (for instance, part of the obtained vapour could circulate upstream the tank and mix with other liquids or vapours).

| Identifier | 2001 |
| --- | --- |
| Title | pressure relief |
| Clue type | component-only |
| Item | checkv+cvalve+pregulator+sv |
| Clue Text | Has any device been foreseen in order to quickly reduce overpressure peaks? |

Explanation:	To avoid the failure in a hydraulic system, a safety valve or device should be installed to relieve overpressure.

| Identifier | 2003 |
| --- | --- |
| Title | engine’s efficiency |
| Clue type | component-only |
| Item | pump+turbine |
| Clue Text | Are the characteristic parameters the correct ones? |

Explanation:	An engine has optimum efficiency in some specific conditions of its parameters (for instance in the case of a pump, for a specific height, flow, work pressure, the efficiency, the efficiency will be maximum or not depending on its characteristic graph).

| Identifier | 2004 |
| --- | --- |
| Title | cavitation |
| Clue type | component-only |
| Item | pump+turbine |
| Clue Text | Are there any regions where the vapour pressure is lower than the vapour tension of a liquid? |

Explanation:	If pressure is lower than the vapour tension, vapour bubbles are produced within the liquid. These bubbles are drag down with the liquid to a region where a higher pressure is reached. If the bubbles are close to a wall the pressure that the liquid produces when it enters the cavities might create very high local pressures which could damage the solid surface. Additionally, vibration, noise and a decrease of efficiency of the hydraulic engine occur.

| Identifier | 2005 |
| --- | --- |
| Title | water hammer |
| Clue type | path-component |
| Item | cvalve+valve |
| Clue Text | Has the ’water hammer’ effect caused by the sudden closure of a valve been considered? |

Explanation:	When a flow is quickly decelerated due to the closure of a valve, the liquid compressibility and the elasticity of the pipe walls might produce a phenomenon called a “water hammer”. This produces a wave of high pressure downstream that could damage the piping and the equipment mounted on it.

| Identifier | 2006 |
| --- | --- |
| Title | wear out |
| Clue type | component-only |
| Item | checkv+cvalve+drain+pump+sv+tank+tankbladder+tankpres+ turbine+valve+vessel |
| Clue Text | Has the wear out of mechanical components and piping in hydraulic systems been considered? |

Explanation:	With use piping, valves, tanks, etc. are affected by corrosion, incrustations and deposit of material on their walls. This could also impact the friction coefficients and therefore the characteristics of the fluid flow.

| Identifier | 2007 |
| --- | --- |
| Title | reaction of corrosion products |
| Clue type | once only |
| Item | checkv+drain+pump+sv+tank+tankbladder+tankpres+turbine+ valve+vent+vessel |
| Clue Text | Can corrosion products catalyse a reaction? |

Explanation:	Some corrosion products might catalyse a reaction with the fluid contained in the hydraulic system.

| Identifier | 2008A |
| --- | --- |
| Title | common drain or venting lines |
| Clue type | path-component |
| Item | drain+vent |
| Clue Text | Do common drain, vent, pressurisation, blanketing, connect several sources? |

Explanation:	If several fluid sources are connected through common lines, the unintended mixture or reaction of two (or more) of them could damage the system or affect the required performance of the system. If the common lines are unavoidable, precautions (e.g. installation of valve allowing only one-way flow) should be taken to prevent unintended mass flow trough these “common lines”.

| Identifier | 2008B |
| --- | --- |
| Title | connected flow sources |
| Clue type | path-component |
| Item | tank+tankbladder+tankpres+vessel |
| Clue Text | Have sources of different fluids and/or at different pressures/temperatures been connected together? |

Explanation:	Some flow sources might require to be ’isolated’ from other sources. If this is not done undesired chemical reaction and/or an unintended change of the characteristic of the fluid flow might occur.

| Identifier | 2009 |
| --- | --- |
| Title | loss of drains |
| Clue type | path-component |
| Item | drain |
| Clue Text | Does loss of drains allow backfeeding into the path? |

Explanation:	If the drains are used to control the pressure in a branch of the circuit, the loss of drains (after switching of a valve for instance) will let the junction point in a “floating state” and might induce an unintended current to flow in other branches of the circuit.

| Identifier | 2011A |
| --- | --- |
| Title | protection against reverse flow |
| Clue type | path-component |
| Item | cvalve+tank+tankpres+valve+vessel |
| Clue Text | Is each branch of the circuit protected against potential reverse flow? |

Explanation:	For instance, during the switching from one branch to another, there may be a reverse current between branches. This applies especially if there are multiple fluid sources that are connected or branches originating from the same source, that have different pressure drops, and later reconnect. When no protection against reverse flow is implemented downstream to each flow source there might be current flowing from other sources to the unprotected one(s). Usually, check valves are used to eliminate this problem.

| Identifier | 2012 |
| --- | --- |
| Title | valve used for different functions |
| Clue type | path-component |
| Item | cvalve+valve |
| Clue Text | Is the valve used to implement different functions in different operating modes? |

Explanation:	If a valve implements different functions (e.g. isolation and flow control) in different operating modes of the system there is the possibility that a specific function might be unintentionally activated or deactivated in some operating modes of the system.

| Identifier | 2013 |
| --- | --- |
| Title | pressure level during different operating modes |
| Clue type | path-component |
| Item | cvalve+tank+tankpres+valve+vessel |
| Clue Text | Does the pressure level remain in its expected range(s) during the various operating modes? |

Explanation:	During particular phase (e.g. testing) or changes of operating mode some unforeseen switching combinations might be triggered and this could lead the pressure of the system to exit from the expected range.

| Identifier | 2014A |
| --- | --- |
| Title | temporal gap between switching of valves |
| Clue type | path-component |
| Item | cvalve+valve |
| Clue Text | Is the temporal gap between the activation and deactivation of different valves in a system justified? |

Explanation:	When multiple valves shall be activated or deactivated, care should be taken that the temporal gap between their activation and deactivation does not lead to unintended effects (e.g. reverse flow, flow at the wrong time)

| Identifier | 2014B |
| --- | --- |
| Title | switching sequence of valves |
| Clue type | path-component |
| Item | svelte+valve |
| Clue Text | Is the activation or deactivation sequence of different valves in a system the proper one? |

Explanation:	When multiple valves in system shall be activated or deactivated the proper sequence of activation needs to be ensured either automatically or by manual procedures, otherwise unintended or untimely flow can occur.

| Identifier | 2015 |
| --- | --- |
| Title | transient flow paths |
| Clue type | component-only+path-component |
| Item | cvalve+pump+turbine+valve |
| Clue Text | Has the flow transient time when an item changes state been taken into account? |

Explanation:	When a valve is opened or closed a certain time is needed for the flow to be fully established or stopped as the case may be. This transient time could cause problems of synchronization with other equipment (e.g. other valves) in the circuit that have different response times. Similar considerations apply also to other items (e.g. pumps, turbines, etc). The synchronisation problems could cause unintended flow (or no flow) to occur during the transient.

| Identifier | 2016 |
| --- | --- |
| Title | leakage flow |
| Clue type | component-only |
| Item | checkv+cvalve+drain+pump+sv+tank+tankbladder+tankpres+ turbine+valve+vent+vessel |
| Clue Text | Can the leakage flow of the component lead to an undesired activation or inhibition of other equipment in the circuit? |

Explanation:	For instance when a circuit segment should be nominally isolated, internal leakage from the above segment can cause an unintended flow to other parts of the circuit.

| Identifier | 2017 |
| --- | --- |
| Title | storage |
| Clue type | path-component |
| Item | tank+tankpres+vessel |
| Clue Text | Has the presence of internal storage devices been taken into account when designing the power off sequence of the system? |

Explanation:	As internal storage devices (e.g. vessels, containers, accumulators) might contain significant masses of fluids after external supply sources are removed or isolated a proper isolation sequence of these devices should be envisaged during the power off phase of the circuit.

Index

(un)expected switch/command activation	63, 64

acceptable charge/discharge current	30

activation by leakage current	30

address code control	72

adjustment of input impedance	51

aliassing	78

array limits	77

array zero element	77

assignment to constants	80

balanced push pull transistors	36

bias/polarization signal	68

buffer overflow	86

capacitive feedback within an amplifier	51

capacitor used as status indicator	31

case coverage in nested IF	81

cavitation	92

coherence between label wordings	41

collector-base junction reverse biased	18

command duration adapted to circuit char	45

command input implementation	22

common drain or venting lines	93

commutation time of transistor	52

compensation of amplifier input capacitance	51

component direction	60

computed address error	89

connected flow sources	93

connected power sources	54

connected tristate outputs enabled	36

connection of testing circuits	65

connector in earth return line	47

contacts and inductive load switching	50

continuous current path via an auto transformer	53

control transfer into a loop	83

correct bias for zener	53

correct initialization time	38

correct polarization with joining capacitor	70

correct power settling time	39

correct signal attachment on drawing	55

correct signal for mode	32

correct state during initialization	36, 37, 38

current limiter shared by multiple loads	60

current limiters in series	46

data consistency on error recovery	88

data corruption	89

data read frequency	74

deadlock	86

decimal point placement	79

decoupling capacitor for each board	49

decoupling capacitor values	28

deep level errors	88

delay due to parasitic capacitance	23

delay of a signal	21

delay of a signal, high output resistance	23

digital distributed signal	24

diode prevents energy release	29

discharge circuit for a power-source	44

disconnecting separated connectors	48

disconnection from signal source	24

disconnection of a static energy protect	58

disconnection of electrical interlocks	65

division by zero	76

drawing consistency with part list	59

endless recursion	81

energy received by an indicator	45

energy storage/release	28, 29

engine’s efficiency	91

erroneous control transfer	89

error codes on procedure exit	89

error handling interruption	87

error handling lockout	88

error recovery reinitialisation	88

evaporation of a liquid	91

event processing	84, 86

exception side effects	75

excessive reverse bias	19

exhaustive labelling	41

explicit wording of a label	71

false indication during switching	40

fan out/fan in rating	33

feedback network on separated boards	19

filtering of a power source	20

filtering of cables and long lines	20

fixed data read cycle	74

floating point equality	82

freeing memory	81

frequency pass band for measurement	21

function calls in IF test	82

function side effects	75

heap space exhaustion	80

high switching current for tantalum capacitors	30

high value resistor	67

hysteresis for noise immunity	51

ICs needing external component	57

impedance matching	43

indication stability during transition	24

indicator correct function label	71

indicator for a unique function	71

inductance of a wire wound resistor	54

initialisation of values	78

input register read timing	75

internal unexpected link of ICs pins	35

internally unsuppressed pins	57

irreversible state after switching	48

label ambiguity	40

label coherence between separated boards	42

lack of surge resistor	29

leakage flow	96

load characteristic after use	58

load rating	33

load ratings	34

load ratings for EED	35

load seen by the transformer primary	45

location of monitoring circuit signal so	24

location of the protection device	55, 70

loop termination	83, 84

loss of drains	93

loss of ground	66

loss of power source	66

low impedance between power sources	50

low voltage drift with time	35

material quality of the contact	49

maximum frequency delays	22

maximum frequency signal levels	54

memory mapped register	72, 73

mismatch in parameter list	79

monitoring circuit independent of monitored function	56

multidimensional arrays	78

multiple earth returns for a signal	46, 47

multiple paths of a signal	46

no fire	59

non-linear transfer function	19

numeric constants	79

one earth path for separate power source	56

one path for separate power returns	55

one path for separate signal returns	56

operating range of a limiting device	60

overflow	26, 76

overload data loss	87

overload delay of espouse	87

overload delay of response	87

overloading	79

parallel diodes for redundancy/power	53

parallel MOS/FET transistors	18

permanent electrical link through a cont	17

place of the switching items	16

plug	52

pointer data access	78

power and signal lines in separated conn	48

power transient	64

predefined exceptions	90

pressure level during  different operating modes	94

pressure relief	91

protection against reverse flow	94

protection against reverse voltage	20, 31, 32

protection against transient currents	26

protection against transient voltage	25

protective and polarization device	69

pull-up resistor for multi-user line/bus	27

pull-up resistor impedance	27

RC/LC time constant	22

reaction delay of one-shot items	68

reaction of corrosion products	93

recursive call	80

reference potential for chassis	67

reference potential for power supply	67

register address modification	73

register number modification	72

register overflow	75

register sequence dependency	73

register timing	74

register use coordination	74

response time of a monitoring circuit	61

resynchronization of a sequential logic	70

resynchronization of sequential logic with long logic train	71

reverse current in a relay coil	68

reverse current through collector	17

reverse voltage on a polarized capacitor	50

saturated or clipped signal	37

saturated or clipped signal input	38

saturation detection	44

self-healing of plastic film capacitors	38

semaphore/lock matching	85

shared data access protection	86

shared data processing	85

shared table processing	85

signal noise compatible with noise immunity	57

signal on a power line	47

simultaneous activation of exclusive inputs	43

spike on the initialization signal	33

splitting and recombining of a digital signals	69

stack overflow	80

start up latency	61

static energy protection	58

storage	96

stray capacitance	44

string character 0	76

string length	77

string termination	77

sufficient resources for the loads	39

switch function for target	69

switch/command activation	64

switching item dedicated to a function	37

switching item shared by several function	39

switching sequence of valves	95

synthesis of control codes	73

temperature limit for multi layer tantalum capacitors	50

temperature stability problems	35

temporal gap between switching of valves	95

temporal independence between switching	42

test sequence and priority	81

timing compatibility	28

transient current through a fuse or breaker	44

transient current through a relay contact	33

transient exceeding breakdown voltage	17

transient exceeding max curren	16

transient flow paths	95

transitory current gap while switching	62

transitory current path through switches	61, 62

tristate outputs for multi-user line/bus	26

TRUE and FALSE representation	82

truth table for digital function	43

turn-on delay of a diode	67

type coercion	76

unconnected signals	41

unconnected signals due to switching	41

undesired state inhibition	43

unexpected activation	59

uniform logical indication	42

unstable or irreversible state	52

unused inputs	25

unused inputs tied to other inputs	25

use of access types and pointers	90

valve used for different functions	94

variable loop index limit	83

voltage compatibility between different	18

voltage delay during power transition	31

voltage protection on inputs	23

water hammer	92

wear out	92

zero representation	82